3GPP TSG-RAN WG2 Meeting #131	R2-2505447

Bengaluru, India, 25-29 August 2025

Agenda item:	8.2.3

Source:	Apple

Title:	Discussion on Paging for Ambient IoT

WID/SID:	Ambient_IoT_solutions – Release 19

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#129 meeting [1], the following agreements about paging has been reached:

- Parallel service requests by the same reader is not supported. - The device is expected to only perform one procedure at a time. FFS device behaviour if multiple requests are received in parallel (if needed). - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID. - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size. - RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose. We can rely on transaction ID and implementation to handle it. - The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2. - The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer. FFS if there is really a need for visibility in the MAC layer - the A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field - The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In RAN2#129bis meeting [2], the following agreements about paging and R2D trigger message has been reached:

- FFS which solution if any for device behavior if it gets a new service request while one procedure is still ongoing or leave it to implementation. - RAN2 aims to design Rel-19 AIoT R2D messages extensible to accommodate devices and features of future release. - Introduce an explicit 1 bit indication to indicate whether it is CFRA or CBRA per paging message. - A field indicating Paging ID length information is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message. - The number of bits required for paging ID length field should be as small as possible. This would require the number of different Paging ID lengths to be small. - Send an LS to SA2 to tak this into account for their design. - A new R2D message other than the paging message is introduced for A-IoT device determining MSG1 resources unless RAN1 concludes to use L1 signaling. The R2D message indicates the start of a set of MSG1 resources that were configured in paging message. - Assumption: The R2D message does not include slot number/count down number.

In RAN2#130 meeting [3], the following agreements about paging has been reached:

1. For CFRA, as a baseline the fields related to the transaction ID, indication of paging ID present/absent and number of access occasions are absent. FFS on the need for the transaction ID for command case. 2. For CFRA, the device always responds to paging regardless of transaction ID (if we put a transaction ID) (i.e. as long as it is addressed to the corresponding device). 3. To ensure forward compatibility for paging with multiple identifiers, introduce at least one R field. FFS if more than one R bit is required. 4. Rel-19 devices would ignore the content of future release instead of ignoring the whole paging message. 5. Issue (1-4) For number of access occasions introduce exponential way, 4 bits, value range FFS

In this paper, we discuss some remaining issues for the normative work for Ambient IoT paging:

# 2 Discussion

### 2.1 “Number of AO” in Paging

RAN2#130 meeting has made the following agreement:

For number of access occasions introduce exponential way, 4 bits, value range FFS

In the current MAC CR implementation, this has been represented by a 4-bit field which means the total number of AOs are only to be in values in the power of 2, such as 32, 128, …, up to 32768 (215). In the last RAN1 meeting [4], RAN1 has made the following agreement on the D2R resources for Msg1 transmission:

For indication of frequency domain resources for Msg 1 transmissions when Y≥1, the reader indicates  a single bit duration Tb which is same for all frequency domain resources  a set of R values, where the possible R values correspond to the agreed table of values of Tb, Tchip, and R § note: the set of R values could be signalled using a bitmap The detailed signalling design is left to RAN2.

Note that the bitmap method indicated in RAN1 agreement above suggest that multiple different frequence offset value (R values) can be represented simultaneously to allow multiple FDM resources to be indicated for RACH. For an 8-bit bitmap, this means the number of resource in frequency domain can be 1 to 8. It is worth noting that values 3, 5, 6 and 7 are not exponential of 2. So there is a discrepancy between the number of AOs indicated in Paging message and the actual number of AOs provided in the paging procedure. For example, if there are 7 frequency shifts indicated in the bitmap, and the total number of AO is shown as 128 (“0111”) in the paging message, the reader cannot actually provide 128 resources.

Instead, one of the following erroneous situation will occur:

- The reader uses 17 Access Triggers message (together with the paging message) to indicate 126 (18x7) total number of access occasions. So any device which chooses AO=126 or AO=127 will not get the chance to access.

- The reader use 18 Access Triggers message (together with the paging message) to indicate 133 (19x7) total number of access occasions. As a result, AO =129, 130, 131, 132 and 133 will not be chosen by any device and get wasted.

To fix this error, two options can be considered

Option 1: Use linear codepoints to indicate number of Access occasions. For example, 15-bit field can represent any number between 1 and 32768. So, there will be no issue to indicate the exact number of AO no matter how PHY layer resource mapping is formulated.

Option 2: Another method is to change the IE “Number of AO” to only represent the time domain slots, which are not affected by the choice of frequency bitmap. Such a number can be still discretely coded as exponentials.

RAN2 need to discuss which option is used to address this problem:

Proposal 1 	RAN2 chooses one of the following for “Number of AO” field in the Paging message: 1) use linear codepoints instead of exponentials; 2) Keep exponential coding, but rename “Number of AO” field to “Number Access Slot” field to only represent the total number of RACH slots in the time domain.

### 2.2 Device Tracking of “Access Trigger” messages

RAN2 129bis [2] has agreed to introduce a new R2D trigger message to indicate the start of the set of Msg1 resource. Those Msg1 resource may be TDM/FDMed. It has been assumed the R2D message does not include slot number/count down number. In the current MAC running CR, this Access Trigger message contains a 3-bit R2D message type filed, but there is an FFS issue whether the Access Trigger message should be byte-aligned.

There is a view that only 3-bit is needed as the content of Access Trigger and any more bits will add overhead.  However, besides the message type indicating this is a R2D access trigger, Access Trigger message need contain something to more reliably indicate the AOs associated with this particular trigger. Note that based on the low-cost nature of Type1 A-IoT device and channel conditions in PRDCH, it is very likely that the device may fail to decode every Access Trigger. Then, we think it is problematic for a device to first determine an AO and the corresponding Access Slot # for slotted Aloha access, but then lost track of the slot number due to Rx failure. In this problematic case, the device may miscount the slot numbers.

This will cause two problems:

- Device may lose the access opportunities. An A-IoT device which lost track of AOs may end up not access at all. For example, each slot provide a single AO, and there are total number of 32 slots. However, the device choose AO=31 has missed the two trigger messages in the earlier reception. Consequently, before the device’s internal counting of AO reaches 31, the reader will initiate a subsequent paging message, and the device will be forced to re-access.

- Unfairness in the random AO selection. Literally, a device choose a smaller AO will have better access success probability than the device choose a later AO. This is because even some mishap happens for such a device and the device miscount the AOs, it will still access in a subsequent slot one way or the other. Literally, the later slots will be more congested than the earlier slots. This may motivate the device implementation to hack the 3GPP-speficied RACH algorithm to bias towards an early AO selection.

For this error case, we think adding the five LSBs of slot number in R2D trigger message is a very good measure to mitigate this issue. For example, the first Access Trigger message will indicate “00001” after the 3-bit Message Type, and the next trigger will indicate “00010”, etc. With this extra information in Access Trigger message, an A-IOT device, even after missing a few R2D triggers, can still be correctly triggered at its selected slot occasion, based on the slot number information matching the last 5 LSB bits indicated in the Access Trigger message.

Proposal 2 	The 5 LSB of “Access Slot number” is included in Access Trigger message.

# 3 Conclusion

In this contribution, we discuss the paging issue for Ambient IoT, and have the following proposals:

Proposal 1 	RAN2 chooses one of the following for “Number of AO” field in the Paging message: 1) use linear codepoints instead of exponentials; 2) Keep exponential coding, but rename “Number of AO” field to “Number Access Slot” field to only represent the total number of RACH slots in the time domain.

Proposal 2 	The 5 LSB of “Access Slot number” is included in Access Trigger message.

# 4 References

[1] Chairman’s note R2-129

[2] Chairman’s note R2-129-bis

[3] Chairman’s note R2-130

[4] Chairman’s note R1-121