3GPP TSG-RAN WG2 #131	R2-2505461

India, Bangalore, August 25th – 29th, 2025

Agenda item:	   8.3.2.2

Source:		Lenovo

Title:                             Functionality Management for measurement event prediction

Document for:		Discussion and Decision

# Introduction

As instructed by the SID, the AI/ML for mobility SI will study the RRM measurement prediction and measurement event prediction. In this paper, we discuss essential functionality management aspects for L3 event prediction, considering these use cases and the respective scenarios of temporal, frequency and spatial domain predictions. .

# Discussion

For AI mobility use cases, initial agreements have been made in RAN #130 on functionality management based on the progress of AI/ML PHY BM use case. To apply AI/ML functionalities for inference, the respective configuration and thus the functionalities must be applicable from both the network and UE perspectives. Therefore, determining applicability is a crucial step for the smooth functioning of all AI/ML features. Both Option A and Option B for inference configuration or set of inference parameters, are considered. Upon receiving the set of parameters for inference or the full inference configuration from the network, the UE is expected to report the applicability of the configured functionalities. It has been agreed in the last RAN2 meeting that, for AI/ML Mobility, UE reports the applicability via RRCReconfigurationComplete and/or UAI message.

Proposal 1: A common functionality management framework for inference configuration and applicability reporting can be applied for measurement prediction and event prediction.

Agreements

- Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case)

•	Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress)

•	Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

•	Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly.

•	Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes.

•	Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress).

•	No prohibit timer is introduced.

- Associated ID should be optionally configurable for training and  inference (i.e. it may not mandatorily required for all training/inference configurations).  FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

## 2.1. Measurement Event Prediction

The events A1-A5, which are utilized to evaluate the signal quality of serving and neighbour cells located at intra/inter frequency, are designed for mobility purposes. When one of the events is met, UE is triggered to report the measurement results. Once the network receives the measurement results, the network will decide whether to prepare a handover and which neighbour cell can be selected as the target cell. Event A3 was agreed as a baseline since it is most relevant for mobility.

In current specifications, the entering condition and leaving condition are specified for each event. When the entry condition of an event is met during TTT, the event is determined as satisfied. This implies UE enters the cell edge and consequently, the measurement report is triggered. When the leaving condition of an event is met during TTT, it means UE returns to the cell centre from the cell edge. UE is triggered to report the measurement result if reportOnLeave is configured. Similarly, the network can configure UE to predict whether the entering condition or the leaving condition is fulfilled. The prediction of the leaving condition could be helpful for the network to decide a suitable time instance for handover execution.

Proposal 2: Network can configure UE to predict whether the entering condition and/or the leaving condition of a concerned RRM event will be met.

UE receives a configuration for event prediction. Meanwhile, a prediction window may be provided together with the configuration. If a prediction window is provided, UE is expected to predict whether an event occurs within the prediction window.

Proposal 3: The prediction window is configured for UE. If a prediction window is provided, UE is expected to predict whether an event occurs within the prediction window.

UE can be configured to report the prediction result for a configured event, which may be based on the configured prediction window. It is up to UE’s implementation to decide on the choice between indirect and direct prediction methodology. More specifically, once the prediction result is available, UE will report whether the event occurs within the prediction window or not. If the prediction window is not configured, the time instance of the event occurrence should be reported as well. If the direct prediction is applied, the probability may be reported as well. If the indirect prediction is applied, the predicted/actual measurement results used in the event inequality may be reported as well.

Proposal 4: Once the prediction result is available, UE reports whether the event occurs within the prediction window or not. If the prediction window is not configured, the time instance should be reported as well.

Proposal 4a: If the direct prediction is applied, the probability may be reported.

Proposal 4b: If the indirect prediction is applied, the predicted/actual measurement results used in the event inequality may be reported.

## Performance Monitoring

For UE-sided model, NW-sided monitoring, the performance monitoring procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. For UE-sided model, UE-sided monitoring, the performance monitoring procedure as baseline contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases.

For indirect RRM event prediction (based on RRM measurement prediction), the monitoring will be essentially about comparing the predicted measurement result and the actual measurement result, for which both NW-sided monitoring and UE-sided monitoring can be applied.

Proposal 5: For indirect RRM event prediction with UE-sided model, RAN2 considers both NW-sided monitoring and UE-sided monitoring.

For direct RRM event prediction on the other hand, we are not sure if gNB-centric performance monitoring is practical, considering:

- The RRM event will not occur or be predicted to occur very frequently when UE is under a cell coverage. There may not be enough samples for gNB to calculate the monitoring metric.

- The UE may be handed over to another cell after reporting a predicted RRM event. There may not be enough time for gNB to determine and make use of the monitoring metric before handover.

Observation 1: For direct RRM event prediction, NW-sided performance monitoring may be not practical considering:

- The RRM event will not occur or be predicted to occur very frequently when UE is under a cell coverage. There may not be enough samples for gNB to calculate the monitoring metric.

- The UE may be handed over to another cell after reporting a predicted RRM event. There may not be enough time for gNB to determine and make use of the monitoring metric before handover.

Comparatively, since the same UE sided model for direct RRM event prediction will be maintained at UE, and may be used across cells within an area, it will certainly be helpful for UE to keep monitoring based on statistics when UE stays in different cells.

Proposal 6: For direct RRM event prediction with UE-sided model, UE-sided performance monitoring is supported.

Proposal 7: For direct RRM event prediction with UE-sided model, RAN2 considers NW-sided performance monitoring is not practical.

One challenge for both direct and indirect RRM event prediction is that it could happen that a handover is triggered before a predicted event (e.g., A3/A5) really occurs. In this case, the UE cannot effectively monitor if a previously made RRM event prediction is accurate or not, since UE will probably not continue measuring the old cell after the handover. RAN2 is suggested to study if/how UE can continue monitoring RRM event prediction upon handover.

Proposal 8: To monitor RRM event prediction with UE-sided model, RAN2 studies the scenario wherein a handover is triggered before the time point a RRM event is predicted to occur.

# Conclusion

Based on the discussion above, we have the following observations:

Observation 1: For direct RRM event prediction, NW-sided performance monitoring may be not practical considering:

- The RRM event will not occur or be predicted to occur very frequently when UE is under a cell coverage. There may not be enough samples for gNB to calculate the monitoring metric.

- The UE may be handed over to another cell after reporting a predicted RRM event. There may not be enough time for gNB to determine and make use of the monitoring metric before handover.

Based on the discussion above, we request RAN2 to discuss and agree on the following proposals:

Proposal 1: A common functionality management framework for inference configuration and applicability reporting can be applied for measurement prediction and event prediction.

Measurement event prediction

Proposal 2: Network can configure UE to predict whether the entering condition and/or the leaving condition of a concerned RRM event will be met.

Proposal 3: The prediction window is configured for UE. If a prediction window is provided, UE is expected to predict whether an event occurs within the prediction window.

Proposal 4: Once the prediction result is available, UE reports whether the event occurs within the prediction window or not. If the prediction window is not configured, the time instance should be reported as well.

Proposal 4a: If the direct prediction is applied, the probability may be reported.

Proposal 4b: If the indirect prediction is applied, the predicted/actual measurement results used in the event inequality may be reported.

Performance Monitoring

Proposal 5: For indirect RRM event prediction with UE-sided model, RAN2 considers both NW-sided monitoring and UE-sided monitoring.

Proposal 6: For direct RRM event prediction with UE-sided model, UE-sided performance monitoring is supported.

Proposal 7: For direct RRM event prediction with UE-sided model, RAN2 considers NW-sided performance monitoring is not practical.

Proposal 8: To monitor RRM event prediction with UE-sided model, RAN2 studies the scenario wherein a handover is triggered before the time point a RRM event is predicted to occur.

# References

[1] RP-250971, TR 38.744 Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR.

[2] TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18)

[3] TS 38.300 running CR for AI for air interface