# 3GPP TSG RAN WG2 Meeting #131      	                           R2-2505498
Bengaluru, Indian, August 25 – 29, 2025

Agenda item:	8.5.2

Source:	Apple

Title:	Remaining issues on on-demand SSB for SCell

WID/SID:	Netw_Energy_NR_enh-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]

  - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling)

  - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

The Rel-19 WID of Network Energy Savings (WID RP- RP-250343) was updated in RAN#107 [1]. The WI objective on on-demand SSB (OD-SSB) SCell operation is copied below:

By RAN2#130 [2], OD-SSB made good progress. In this contribution, we continue to discuss remaining issues on OD-SSB identified by MAC running CR [3] and RRC running CR [4].

# 2 Discussion

## 2.1 Remaining issues on OD-SSB MAC-CE

Existing MAC running CR has specified the OD-SSB MAC-CE format as illustrated below [3]:

Figure 6.1.3.x-1: On-demand SSB Activation/Deactivation MAC CE of one Activation/Deactivation octet

Editor’s note: the size of OD-SSB ConfigIndexi is FFS

As highlighted above, one remaining issue is the size of OD-SSB ConfigIndex. This issue is also captured as open issue Q9 in RRC CR discussion [4]:

Q9: Suggest value for this parameter or indicate if another WG needs to be liaised?

Only one company responded maxNrofOD-SSB can be 16 (4bit) or 32 (5bit).

Conclusion: value for maxNrofOD-SSB is an open issue

We think it is sufficient to have 4bit OD-SSB ConfigIndex (i.e. up to 16 OD-SSB configurations).

Proposal 1 (MAC and RRC open issue 1): Size of OD-SSB ConfigIndex is 4bit.

## 2.2 Remaining RRC open issues:

According to RRC running CR discussion [4], 3 open issues were identified:

- Issue 1: Q9

- Issue 2: CATT003

- Issue 3: Fujitsu006 / HW002

As issue 1 is covered by Proposal 1, we discuss issue 2 and 3 in this section.

### 2.2.1 RRC open issue 2: CATT003

The issue is:

| CATT003 | od-ssb-nrofBurst Indicates the number of OD-SSB bursts to be transmitted after OD-SSB is indicated.. Agreement (RAN1 #121) For a cell supporting on-demand SSB SCell operation, the following combinations are supported. - For OD-SSB transmission activation (OD-Tact) and OD-SSB transmission adaptation (OD-TA), - Case A1: RRC-based OD-Tact without N (i.e., od-ssb-nrofBurst) configured + MAC CE-based OD-TA; - Subject to UE capability - Case B1: MAC CE-based OD-Tact without N configured + MAC CE-based OD-TA; - Case B2: MAC CE-based OD-Tact with N configured + MAC CE-based OD-TA. - For OD-SSB transmission deactivation (OD-TD), - Case X1: RRC-based OD-Tact without N configured + MAC CE-based OD-TD; - Subject to UE capability - Case Y1: MAC CE-based OD-Tact or OD-TA without N configured + MAC CE-based OD-TD; - Case Y2: MAC CE-based OD-Tact or OD-TA with N configured + implicit OD-TD; - Case Y3: MAC CE-based OD-Tact or OD-TA with N configured + MAC CE-based OD-TD. - Conclusion: There is no RAN1 consensus to support RRC activation of OD-SSB transmission configuring od-ssb-nrofBurst. - Note: “Implicit OD-TD” above implies that the on-demand SSB is deactivated based on the value for od-ssb-nrofBurst according to NW indication. According to the conclusion of RAN1, od-ssb-nrofBurst can only be configured by RRC and activated by MAC CE, and RAN2 has not discussed this issue. The od-ssb-nrofBurst is configured in parallel with od-ssb-ActivationStatus in OD-SSB-Config, so od-ssb-nrofBurst should be able to be configured and activated through RRC? We think we need to make a clarification. | Open issue |
| --- | --- | --- |

We think CATT005 discuss the following new case:

- RRC-based OD-Tact w N (i.e., od-ssb-nrofBurst) configured + RRC based OD-TA.

As this case has no RAN1 consensus, we tend to follow RAN1 conclusion. Thus, we propose:

Proposal 2 (RRC open issue 2): Following RAN1#121 conclusion, RAN2 clarify that “only MAC-CE can activate OD-SSB when this field is present” in field description of od-ssb-nrofBurst (i.e. RRC-based OD-Tact with od-ssb-nrofBurst configured + RRC based OD-TA is not allowed).

### 2.2.2 RRC open issue 3: Fujitsu006

The issue is:

| Fujitsu 006 | – OD-SSB-Config RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.; Field servingCellMO is introduced in IE OD-SSB-Config. In this way it is possible to keep the same field name an hence all the procedural text intact.. [Fujitsu] RAN2 agreed to introduce a new servingCellMO in ServingCellConfig in RAN2#130. Although the current CR may work, we think the original agreement is simpler for implementation, because it can keep the same message structure. We also think even if it is introcuded in ServingCellConfig, the current procedural text is not changed (similar to servingCellMO-r17). We would like to add it into open issue list and ask other company’s views. [Apple] We have same view as Fujisu. | Open issue. |
| --- | --- | --- |

Current RRC Rapporteur implementation is to include new servingCellMO under OD-SSB-Config-r19:

od-SSB-ConfigToAddModList-r19            SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-Config-r19    OPTIONAL,   -- Need N

od-SSB-ConfigToReleaseList-r19          SEQUENCE (SIZE (1.. maxNrofOD-SSB-r19)) OF OD-SSB-ConfigId-r19    OPTIONAL   -- Need N

OD-SSB-Config-r19 ::= SEQUENCE {

od-ssb-ConfigId-r19                    OD-SSB-ConfigId-r19,

od-ssb-ActivationStatus-r19            ENUMERATED {activated},

od-ssb-Periodicity-r19                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 }       OPTIONAL, -- Cond ODssbAOssb

od-ssb-sfn-Offset-r19                   INTEGER (0..15)                                                OPTIONAL, -- Cond ODssbAOssb

od-ssb-halfFrameIndex-r19              ENUMERATED {zero, one}                                        OPTIONAL, -- Cond ODssbAOssb

od-ssb-absoluteFrequency-r19           ARFCN-ValueNR                                                  OPTIONAL, -- Cond ODssbAOssb

od-ssb-PositionsInBurst-r19            CHOICE {

shortBitmap                            BIT STRING (SIZE (4)),

mediumBitmap                           BIT STRING (SIZE (8)),

longBitmap                             BIT STRING (SIZE (64))

}                                                                                                          OPTIONAL, -- Cond ODssbAOssb

od-ssbSubcarrierSpacing-r19            SubcarrierSpacing                                           OPTIONAL, -- Cond ODssbOnly

od-ss-PBCH-BlockPower-r19              INTEGER (-60..50)                                            OPTIONAL, -- Cond ODssbOnly

od-ssb-nrofBurst-r19                   INTEGER (1..8)                                                 OPTIONAL, -- Need R

servingCellMO-r19                      MeasObjectId                                                    OPTIONAL, -- Cond InterFreq

smtc1-r19                            SSB-MTC                                                              OPTIONAL, -- Cond IntraFreq

...

}

OD-SSB-ConfigId-r19   ::= INTEGER (0.. maxNrofOD-SSB-1-r19)

We support Fujitsu’s suggestion due to the following 2 justifications:

- One serving cell can include a list of OD-SSB-Config-r19, which means existing RRC implementation allows multiple servingCellMOs for one serving cell. However, we only agreed to introduce up to one servingCellMO for one SCell, it should be per SCell configuration, not per OD-SSB configuration.

- In existing RRC spec [5], legacy servingCellMO is included in ServingCellConfig. We think it is more readable to introudcce new servingCellMO for OD-SSB under the same place.

Observation 1: Current RRC implementation is to include new servingCellMO under OD-SSB-Config-r19. As one serving cell can include a list of OD-SSB-Config-r19, it means existing RRC implementation allows multiple servingCellMOs for one serving cell. However, we only agreed to introduce up to one servingCellMO for 1 SCell.

Thus, we propose:

Proposal 3 (RRC open issue 3): Following RAN2#130 agreement, move new servingCellMO for OD-SSB under ServingCellConfig (i.e. servingCellMO is per SCell configuration rather than per OD-SSB configuration).

Meanwhile, on highlighted IE SMTC, we also don’t prefer it to be included in OD-SSB-Config-r19. As legacy, it should be included in MO and follow existing measurement framework.

Proposal 4 (RRC open issue 3): Move SMTC for OD-SSB under MeasObjectNR and add procedure text.

Finally, on highlighted IEs, we think both RAN1 and RAN2 have not agreed to use MAC-CE to dynamically change them. In our understanding, only od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst were agreed to be dynamically adapted by MAC-CE.

RAN1 related agreements Agreement (RAN1#118) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s), multiple candidate values can be configured by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - Periodicity of the on-demand SSB - FFS: Any other relevant parameters Agreement (RAN1#120b) For a cell supporting on-demand SSB SCell operation, at least for the following parameter(s) (in addition to agreed ones), multiple candidate values can be configured (includes the case where no candidate values are configured) by RRC and the applicable value can be indicated by MAC CE for on-demand SSB transmission indication for the cell. - SSB positions within an on-demand SSB burst by using signaling similar to ssb-PositionsInBurst (i.e., od-ssb-PositionsInBurst) for the following cases - The case where center frequency of AO-SSB and OD-SSB are different - Case 1 - Number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated (i.e., od-ssb- nrofBurst) FFS: Additional restrictions RAN2 related agreement Agreements on OD-SSB (RAN2#130) - (RRC-1) The configuration index is associated with a set of parameters, i.e., od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst.

Observation 2: RAN1 and RAN2 only agreed od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst can be dynamically adapted by OD-SSB MAC-CE.

On highlighted IEs, we don’t think it is reasonable to use MAC-CE to dynamically adapt its value. For example, dynamic change of od-ssbSubcarrierSpacing doesn’t make sense. These IEs should be configured by RRC only.

Observation 3: It is not reasonable to use MAC-CE to dynamically adapt values of some OD-SSB parameters (e.g. subcarrier spacing and AFRCN).

To resolve this issue, we think the simplest way is to move these IEs to SCellConfig.

Proposal 5 (RRC open issue 3): Move the following OD-SSB IEs which can’t be dynamically adapted via MAC-CE under SCellConfig (i.e. they can only be reconfigured by RRC rather than by MAC-CE)

- od-ssb-sfn-Offset-r19

- od-ssb-halfFrameIndex-r19

- od-ssb-absoluteFrequency-r19

- od-ssbSubcarrierSpacing-r19

- od-ss-PBCH-BlockPower-r19

## 2.3 Remaining issues on L3 measurement on OD-SSB

In RAN2#130 [2], it was agreed that different servingCellMO are switched depending on whether OD-SSB is activated or deactivated when OD-SSB and AO-SSB are in different frequencies.

When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

We think there are two remaining issues when OD-SSB and AO-SSB are in different frequencies:

- How to define intra-frequency and inter-frequency measurement

- How to handle measurement gap

Please note that Section 9.2.4 of TS 38.300 [6] has captured above aspects:

From Section 9.2.4 of TS 38.300: How to define intra-frequency and inter-frequency measurement Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows: - SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the SSB frequency configured in the measurement object associated with the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same. - SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different. NOTE 2: For SSB based measurements, one measurement object corresponds to one SSB and the UE considers different SSBs as different cells. NOTE 2a: If a UE is configured to perform serving cell measurements based on an NCD-SSB configured in its active BWP, this NCD-SSB is considered as the SSB of the serving cell in the definition of intra-frequency and inter-frequency measurements as above. - CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that: - The subcarrier spacing of CSI-RS resources on the neighbour cell configured for measurement is the same as the SCS of CSI-RS resources on the serving cell indicated for measurement; and - For 60kHz subcarrier spacing, the CP type of CSI-RS resources on the neighbour cell configured for measurement is the same as the CP type of CSI-RS resources on the serving cell indicated for measurement; and - The centre frequency of CSI-RS resources on the neighbour cell configured for measurement is the same as the centre frequency of CSI-RS resource on the serving cell indicated for measurement. - CSI-RS based inter-frequency measurement: a measurement is defined as a CSI-RS based inter-frequency measurement if it is not a CSI-RS based intra-frequency measurement. NOTE 3: Extended CP for CSI-RS based measurement is not supported in this release. How to handle measurement gap Whether a measurement is non-gap-assisted or gap-assisted depends on the capability of the UE, the active BWP of the UE and the current operating frequency: - For SSB based inter-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following cases: - If the UE only supports per-UE measurement gaps; - If the UE supports per-FR measurement gaps and any of the serving cells are in the same frequency range of the measurement object. - For SSB based intra-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following case: - If the serving cell is associated with SSB, other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP, and are not configured with NCD-SSB for serving cell measurement; - If the serving cell is not associated with SSB (i.e. SSB-less SCell), if the initial BWP or any of the UE configured BWPs do not contain the SSB frequency configured in the measurement object associated with the serving cell, and are not configured with NCD-SSB for serving cell measurement. - For CSI-RS based intra-frequency measurement, no measurement gap is needed; - For CSI-RS based inter-frequency measurement, a measurement gap configuration is always provided in the following cases: - If the UE only supports per-UE measurement gaps; - If the UE supports per-FR measurement gaps and any of the serving cells are in the same frequency range of the measurement object. In non-gap-assisted scenarios, the UE shall be able to carry out such measurements without measurement gaps. In gap-assisted scenarios, the UE cannot be assumed to be able to carry out such measurements without measurement gaps. Network may request the UE to measure NR and/or E-UTRA carriers in RRC_IDLE or RRC_INACTIVE via system information or via dedicated measurement configuration in RRCRelease. If the UE was configured to perform measurements of NR and/or E-UTRA carriers while in RRC_IDLE or in RRC_INACTIVE, it may provide an indication of the availability of corresponding measurement results to the gNB in the RRCSetupComplete message. The network may request the UE to report those measurements after security activation. The request for the measurements can be sent by the network immediately after transmitting the Security Mode Command (i.e. before the reception of the Security Mode Complete from the UE). If the UE was configured to perform measurements of NR and/or E-UTRA carriers while in RRC_INACTIVE, the gNB can request the UE to provide corresponding measurement results in the RRCResume message and then the UE can include the available measurement results in the RRCResumeComplete message. Alternatively, the UE may provide an indication of the availability of the measurement results to the gNB in the RRCResumeComplete message and the gNB can then request the UE to provide these measurement results.

### 2.3.1 How to define intra-frequency and inter-frequency measurement

We think there are basically two options:

- Option 1: When the center frequency of either AO-SSB or OD-SSB of one SCell is same as that of neighbor SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

- Option 2: When the center frequency of the applied servingCellMO of one SCell is same as that of neighbor SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

  - Note: which servingCellMO is applied follows RAN2#130 agreement.

    - We slightly prefer Option 1 because it avoids dynamically changing between intra-frequency measurement and inter-frequency measurement.

    - Observation 4: When defining intra-frequency and inter-frequency measurement if OD-SSB and AO-SSB are in different frequencies, it is preferred to avoid dynamically change between intra-frequency measurement and inter-frequency measurement.

    - But we are open for discussion.

Proposal 6: RAN2 discuss and down-select between the following two options on how to define intra-frequency and inter-frequency measurement when OD-SSB and AO-SSB are in different frequencies:

- Option 1: When the center frequency of either AO-SSB or OD-SSB of one SCell is same as that of neighbor cell SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

- Option 2: When the center frequency of the applied servingCellMO of one SCell is same as that of neighbor cell SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

  - Note: which servingCellMO is applied follows RAN2#130 agreement.

### 2.3.2 How to handle measurement gap

Following the same logic on intra/inter-frequency measurement, we think there are basically two options:

- Option 1: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of both AO-SSB and OD-SSB, a measurement gap configuration is always provided.

- Option 2: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of SSB in the applied servingCellMO, a measurement gap configuration is always provided.

  - Note: which servingCellMO is applied follows RAN2#130 agreement.

Similarly, we also slightly prefer Option 1 because it avoids dynamically change between non-gap-assisted and gap-assisted requirement.

    - Observation 5: When defining measurement gap if OD-SSB and AO-SSB are in different frequencies, it is preferred to avoid dynamically change between non-gap-assisted and gap-assisted requirement.

    - But we are open for discussion.

Proposal 7: RAN2 discuss and down-select between the following two options on how to handle measurement gap when OD-SSB and AO-SSB are in different frequencies:

- Option 1: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of both AO-SSB and OD-SSB, a measurement gap configuration is always provided.

- Option 2: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of SSB in the applied servingCellMO, a measurement gap configuration is always provided.

  - Note: which servingCellMO is applied follows RAN2#130 agreement.

In Appendix 1, we provide TP on implementation option 1 of Proposal 6 and Proposal 7 in TS 38.300. And in Appendix 2, we provide TP on implementation option 2 of Proposal 6 and Proposal 7 in TS 38.300.

# 3 Conclusion

In this contribution, we discuss remaining issues on-demand SSB for SCell. Our observations are:

Observation 1: Current RRC implementation is to include new servingCellMO under OD-SSB-Config-r19. As one serving cell can include a list of OD-SSB-Config-r19, it means existing RRC implementation allows multiple servingCellMOs for one serving cell. However, we only agreed to introduce up to one servingCellMO for 1 SCell.

Observation 2: RAN1 and RAN2 only agreed od-ssb-Periodicity, od-ssb-PositionsInBurst and od-ssb-nrofBurst can be dynamically adapted by OD-SSB MAC-CE.

Observation 3: It is not reasonable to use MAC-CE to dynamically adapt values of some OD-SSB parameters (e.g. subcarrier spacing and AFRCN).

    - Observation 4: When defining intra-frequency and inter-frequency measurement if OD-SSB and AO-SSB are in different frequencies, it is preferred to avoid dynamically change between intra-frequency measurement and inter-frequency measurement.

    - Observation 5: When defining measurement gap if OD-SSB and AO-SSB are in different frequencies, it is preferred to avoid dynamically change between non-gap-assisted and gap-assisted requirement.

Based on above observations, our proposals are:

Remaining issues on MAC-CE format

Proposal 1 (MAC and RRC open issue 1): Size of OD-SSB ConfigIndex is 4bit.

Remaining issues on RRC open issues

Proposal 2 (RRC open issue 2): Following RAN1#121 conclusion, RAN2 clarify that “only MAC-CE can activate OD-SSB when this field is present” in field description of od-ssb-nrofBurst (i.e. RRC-based OD-Tact with od-ssb-nrofBurst configured + RRC based OD-TA is not allowed).

Proposal 3 (RRC open issue 3): Following RAN2#130 agreement, move new servingCellMO for OD-SSB under ServingCellConfig (i.e. servingCellMO is per SCell configuration rather than per OD-SSB configuration).

Proposal 4 (RRC open issue 3): Move SMTC for OD-SSB under MeasObjectNR and add procedure text.

Proposal 5 (RRC open issue 3): Move the following OD-SSB IEs which can’t be dynamically adapted via MAC-CE under SCellConfig (i.e. they can only be reconfigured by RRC rather than by MAC-CE)

- od-ssb-sfn-Offset-r19

- od-ssb-halfFrameIndex-r19

- od-ssb-absoluteFrequency-r19

- od-ssbSubcarrierSpacing-r19

- od-ss-PBCH-BlockPower-r19

Remaining issues on L3 measurement

Proposal 6: RAN2 discuss and down-select between the following two options on how to define intra-frequency and inter-frequency measurement when OD-SSB and AO-SSB are in different frequencies:

- Option 1: When the center frequency of either AO-SSB or OD-SSB of one SCell is same as that of neighbor cell SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

- Option 2: When the center frequency of the applied servingCellMO of one SCell is same as that of neighbor cell SSB, it is intra-frequency measurement. Otherwise, it is inter-frequency measurement.

  - Note: which servingCellMO is applied follows RAN2#130 agreement.

Proposal 7: RAN2 discuss and down-select between the following two options on how to handle measurement gap when OD-SSB and AO-SSB are in different frequencies:

- Option 1: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of both AO-SSB and OD-SSB, a measurement gap configuration is always provided.

- Option 2: if any of the UE configured BWPs (other than initial BWP) do not contain the frequency domain resources of SSB in the applied servingCellMO, a measurement gap configuration is always provided.

  - Note: which servingCellMO is applied follows RAN2#130 agreement.

In Appendix 1, we provide TP on implementation option 1 of Proposal 6 and Proposal 7 in TS 38.300. And in Appendix 2, we provide TP on implementation option 2 of Proposal 6 and Proposal 7 in TS 38.300.

# 4 References

[1] RP-250343, Revised WID for Enhancements of network energy savings for NR, Ericsson.

[2] RAN2#130, Chair Notes.

[3] R2-250xxxx, Running MAC CR for network energy saving, InterDigital.

[4] R2-250xxxx, Running RRC CR for enhancements for network energy efficiency, Ericsson.

[5] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.

[6] TS 38.300-v18.6.0, NR and NG-RAN Overall Description; Stage 2, 2025-06.

# Appendix 1 (38.300 TP of option 1 of P6 and P7)

### 9.2.4	Measurements

In RRC_CONNECTED, the UE measures multiple beams (at least one) of a cell and the measurements results (power values) are averaged to derive the cell quality. In doing so, the UE is configured to consider a subset of the detected beams. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). Measurement reports may contain the measurement results of the X best beams if the UE is configured to do so by the gNB.

The corresponding high-level measurement model is described below:

Figure 9.2.4-1: Measurement Model

NOTE 1:	K beams correspond to the measurements on SSB or CSI-RS resources configured for L3 mobility by gNB and detected by UE at L1.

-	A: measurements (beam specific samples) internal to the physical layer.

-	Layer 1 filtering: internal layer 1 filtering of the inputs measured at point A. Exact filtering is implementation dependent. How the measurements are actually executed in the physical layer by an implementation (inputs A and Layer 1 filtering) is not constrained by the standard.

-	A1: measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering.

-	Beam Consolidation/Selection: beam specific measurements are consolidated to derive cell quality. The behaviour of the Beam consolidation/selection is standardised and the configuration of this module is provided by RRC signalling. Reporting period at B equals one measurement period at A1.

-	B: a measurement (i.e. cell quality) derived from beam-specific measurements reported to layer 3 after beam consolidation/selection.

-	Layer 3 filtering for cell quality: filtering performed on the measurements provided at point B. The behaviour of the Layer 3 filters is standardised and the configuration of the layer 3 filters is provided by RRC signalling. Filtering reporting period at C equals one measurement period at B.

-	C: a measurement after processing in the layer 3 filter. The reporting rate is identical to the reporting rate at point B. This measurement is used as input for one or more evaluation of reporting criteria.

-	Evaluation of reporting criteria: checks whether actual measurement reporting is necessary at point D. The evaluation can be based on more than one flow of measurements at reference point C e.g. to compare between different measurements. This is illustrated by input C and C1. The UE shall evaluate the reporting criteria at least every time a new measurement result is reported at point C, C1. The reporting criteria are standardised and the configuration is provided by RRC signalling (UE measurements).

-	D: measurement report information (message) sent on the radio interface.

-	L3 Beam filtering: filtering performed on the measurements (i.e. beam specific measurements) provided at point A1. The behaviour of the beam filters is standardised and the configuration of the beam filters is provided by RRC signalling. Filtering reporting period at E equals one measurement period at A1.

-	E: a measurement (i.e. beam-specific measurement) after processing in the beam filter. The reporting rate is identical to the reporting rate at point A1. This measurement is used as input for selecting the X measurements to be reported.

-	Beam Selection for beam reporting: selects the X measurements from the measurements provided at point E. The behaviour of the beam selection is standardised and the configuration of this module is provided by RRC signalling.

-	F: beam measurement information included in measurement report (sent) on the radio interface.

Layer 1 filtering introduces a certain level of measurement averaging. How and when the UE exactly performs the required measurements is implementation specific to the point that the output at B fulfils the performance requirements set in TS 38.133 [13]. Layer 3 filtering for cell quality and related parameters used are specified in TS 38.331 [12] and do not introduce any delay in the sample availability between B and C. Measurement at point C, C1 is the input used in the event evaluation. L3 Beam filtering and related parameters used are specified in TS 38.331 [12] and do not introduce any delay in the sample availability between A1 and E.

Measurement reports are characterized by the following:

-	Measurement reports include the measurement identity of the associated measurement configuration that triggered the reporting;

-	Cell and beam measurement quantities to be included in measurement reports are configured by the network;

-	The number of non-serving cells to be reported can be limited through configuration by the network;

-	Cells belonging to an exclude-list configured by the network are not used in event evaluation and reporting, and conversely when an allow-list is configured by the network, only the cells belonging to the allow-list are used in event evaluation and reporting;

-	Beam measurements to be included in measurement reports are configured by the network (beam identifier only, measurement result and beam identifier, or no beam reporting).

Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows:

-	SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the SSB frequency configured in the measurement object associated with the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

-	SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different.

NOTE 2:	For SSB based measurements, one measurement object corresponds to one SSB and the UE considers different SSBs as different cells.

NOTE 2a:	If a UE is configured to perform serving cell measurements based on an NCD-SSB configured in its active BWP, this NCD-SSB is considered as the SSB of the serving cell in the definition of intra-frequency and inter-frequency measurements as above.

-	CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

-	The subcarrier spacing of CSI-RS resources on the neighbour cell configured for measurement is the same as the SCS of CSI-RS resources on the serving cell indicated for measurement; and

-	For 60kHz subcarrier spacing, the CP type of CSI-RS resources on the neighbour cell configured for measurement is the same as the CP type of CSI-RS resources on the serving cell indicated for measurement; and

-	The centre frequency of CSI-RS resources on the neighbour cell configured for measurement is the same as the centre frequency of CSI-RS resource on the serving cell indicated for measurement.

-	CSI-RS based inter-frequency measurement: a measurement is defined as a CSI-RS based inter-frequency measurement if it is not a CSI-RS based intra-frequency measurement.

NOTE 3:	Extended CP for CSI-RS based measurement is not supported in this release.

When both AO-SSB and OD-SSB are configured for one serving cell with different center frequency, intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows:

-	SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the center frequency of either AO-SSB or OD-SSB of the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

-	SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different.

Whether a measurement is non-gap-assisted or gap-assisted depends on the capability of the UE, the active BWP of the UE and the current operating frequency:

-	For SSB based inter-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following cases:

-	If the UE only supports per-UE measurement gaps;

-	If the UE supports per-FR measurement gaps and any of the serving cells are in the same frequency range of the measurement object.

-	For SSB based intra-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following case:

-	If the serving cell is associated with SSB in same frequency, other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP, and are not configured with NCD-SSB for serving cell measurement;

-	If the serving cell is associated with both AO-SSB and OD-SSB in different frequency, other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of both AO-SSB and OD-SSB associated to the initial DL BWP, and are not configured with NCD-SSB for serving cell measurement;

-	If the serving cell is not associated with SSB (i.e. SSB-less SCell), if the initial BWP or any of the UE configured BWPs do not contain the SSB frequency configured in the measurement object associated with the serving cell, and are not configured with NCD-SSB for serving cell measurement.

-	For CSI-RS based intra-frequency measurement, no measurement gap is needed;

-	For CSI-RS based inter-frequency measurement, a measurement gap configuration is always provided in the following cases:

-	If the UE only supports per-UE measurement gaps;

-	If the UE supports per-FR measurement gaps and any of the serving cells are in the same frequency range of the measurement object.

In non-gap-assisted scenarios, the UE shall be able to carry out such measurements without measurement gaps. In gap-assisted scenarios, the UE cannot be assumed to be able to carry out such measurements without measurement gaps.

Network may request the UE to measure NR and/or E-UTRA carriers in RRC_IDLE or RRC_INACTIVE via system information or via dedicated measurement configuration in RRCRelease. If the UE was configured to perform measurements of NR and/or E-UTRA carriers while in RRC_IDLE or in RRC_INACTIVE, it may provide an indication of the availability of corresponding measurement results to the gNB in the RRCSetupComplete message. The network may request the UE to report those measurements after security activation. The request for the measurements can be sent by the network immediately after transmitting the Security Mode Command (i.e. before the reception of the Security Mode Complete from the UE).

If the UE was configured to perform measurements of NR and/or E-UTRA carriers while in RRC_INACTIVE, the gNB can request the UE to provide corresponding measurement results in the RRCResume message and then the UE can include the available measurement results in the RRCResumeComplete message. Alternatively, the UE may provide an indication of the availability of the measurement results to the gNB in the RRCResumeComplete message and the gNB can then request the UE to provide these measurement results.

# Appendix 2 (38.300 TP of option 2 of P6 and P7)

### 9.2.4	Measurements

In RRC_CONNECTED, the UE measures multiple beams (at least one) of a cell and the measurements results (power values) are averaged to derive the cell quality. In doing so, the UE is configured to consider a subset of the detected beams. Filtering takes place at two different levels: at the physical layer to derive beam quality and then at RRC level to derive cell quality from multiple beams. Cell quality from beam measurements is derived in the same way for the serving cell(s) and for the non-serving cell(s). Measurement reports may contain the measurement results of the X best beams if the UE is configured to do so by the gNB.

The corresponding high-level measurement model is described below:

Figure 9.2.4-1: Measurement Model

NOTE 1:	K beams correspond to the measurements on SSB or CSI-RS resources configured for L3 mobility by gNB and detected by UE at L1.

-	A: measurements (beam specific samples) internal to the physical layer.

-	Layer 1 filtering: internal layer 1 filtering of the inputs measured at point A. Exact filtering is implementation dependent. How the measurements are actually executed in the physical layer by an implementation (inputs A and Layer 1 filtering) is not constrained by the standard.

-	A1: measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering.

-	Beam Consolidation/Selection: beam specific measurements are consolidated to derive cell quality. The behaviour of the Beam consolidation/selection is standardised and the configuration of this module is provided by RRC signalling. Reporting period at B equals one measurement period at A1.

-	B: a measurement (i.e. cell quality) derived from beam-specific measurements reported to layer 3 after beam consolidation/selection.

-	Layer 3 filtering for cell quality: filtering performed on the measurements provided at point B. The behaviour of the Layer 3 filters is standardised and the configuration of the layer 3 filters is provided by RRC signalling. Filtering reporting period at C equals one measurement period at B.

-	C: a measurement after processing in the layer 3 filter. The reporting rate is identical to the reporting rate at point B. This measurement is used as input for one or more evaluation of reporting criteria.

-	Evaluation of reporting criteria: checks whether actual measurement reporting is necessary at point D. The evaluation can be based on more than one flow of measurements at reference point C e.g. to compare between different measurements. This is illustrated by input C and C1. The UE shall evaluate the reporting criteria at least every time a new measurement result is reported at point C, C1. The reporting criteria are standardised and the configuration is provided by RRC signalling (UE measurements).

-	D: measurement report information (message) sent on the radio interface.

-	L3 Beam filtering: filtering performed on the measurements (i.e. beam specific measurements) provided at point A1. The behaviour of the beam filters is standardised and the configuration of the beam filters is provided by RRC signalling. Filtering reporting period at E equals one measurement period at A1.

-	E: a measurement (i.e. beam-specific measurement) after processing in the beam filter. The reporting rate is identical to the reporting rate at point A1. This measurement is used as input for selecting the X measurements to be reported.

-	Beam Selection for beam reporting: selects the X measurements from the measurements provided at point E. The behaviour of the beam selection is standardised and the configuration of this module is provided by RRC signalling.

-	F: beam measurement information included in measurement report (sent) on the radio interface.

Layer 1 filtering introduces a certain level of measurement averaging. How and when the UE exactly performs the required measurements is implementation specific to the point that the output at B fulfils the performance requirements set in TS 38.133 [13]. Layer 3 filtering for cell quality and related parameters used are specified in TS 38.331 [12] and do not introduce any delay in the sample availability between B and C. Measurement at point C, C1 is the input used in the event evaluation. L3 Beam filtering and related parameters used are specified in TS 38.331 [12] and do not introduce any delay in the sample availability between A1 and E.

Measurement reports are characterized by the following:

-	Measurement reports include the measurement identity of the associated measurement configuration that triggered the reporting;

-	Cell and beam measurement quantities to be included in measurement reports are configured by the network;

-	The number of non-serving cells to be reported can be limited through configuration by the network;

-	Cells belonging to an exclude-list configured by the network are not used in event evaluation and reporting, and conversely when an allow-list is configured by the network, only the cells belonging to the allow-list are used in event evaluation and reporting;

-	Beam measurements to be included in measurement reports are configured by the network (beam identifier only, measurement result and beam identifier, or no beam reporting).

Intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows:

-	SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the SSB frequency configured in the measurement object associated with the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

-	SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the center frequency of the SSB of the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different.

NOTE 2:	For SSB based measurements, one measurement object corresponds to one SSB and the UE considers different SSBs as different cells.

NOTE 2a:	If a UE is configured to perform serving cell measurements based on an NCD-SSB configured in its active BWP, this NCD-SSB is considered as the SSB of the serving cell in the definition of intra-frequency and inter-frequency measurements as above.

-	CSI-RS based intra-frequency measurement: a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

-	The subcarrier spacing of CSI-RS resources on the neighbour cell configured for measurement is the same as the SCS of CSI-RS resources on the serving cell indicated for measurement; and

-	For 60kHz subcarrier spacing, the CP type of CSI-RS resources on the neighbour cell configured for measurement is the same as the CP type of CSI-RS resources on the serving cell indicated for measurement; and

-	The centre frequency of CSI-RS resources on the neighbour cell configured for measurement is the same as the centre frequency of CSI-RS resource on the serving cell indicated for measurement.

-	CSI-RS based inter-frequency measurement: a measurement is defined as a CSI-RS based inter-frequency measurement if it is not a CSI-RS based intra-frequency measurement.

NOTE 3:	Extended CP for CSI-RS based measurement is not supported in this release.

When both AO-SSB and OD-SSB are configured for one serving cell with different center frequency, intra-frequency neighbour (cell) measurements and inter-frequency neighbour (cell) measurements are defined as follows:

-	SSB based intra-frequency measurement: a measurement is defined as an SSB based intra-frequency measurement provided the SSB frequency configured in the applied measurement object associated with the serving cell and the center frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs is also the same.

-	SSB based inter-frequency measurement: a measurement is defined as an SSB based inter-frequency measurement provided the SSB frequency configured in the applied measurement object associated with the serving cell and the center frequency of the SSB of the neighbour cell are different, or the subcarrier spacing of the two SSBs is different.

NOTE 4: When both AO-SSB and OD-SSB with different center frequencies are configured for the serving cell, the UE applies the serving cell measurement object configured with SSB frequency of OD-SSB when OD-SSB is activated; Otherwise (i.e. when OD-SSB is deactivated), the UE applies the serving cell measurement object configured with SSB frequency of AO-SSB.

Whether a measurement is non-gap-assisted or gap-assisted depends on the capability of the UE, the active BWP of the UE and the current operating frequency:

-	For SSB based inter-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following cases:

-	If the UE only supports per-UE measurement gaps;

-	If the UE supports per-FR measurement gaps and any of the serving cells are in the same frequency range of the measurement object.

-	For SSB based intra-frequency measurement, if the measurement gap requirement information is reported by the UE, a measurement gap configuration may be provided according to the information. Otherwise, a measurement gap configuration is always provided in the following case:

-	If the serving cell is associated with SSB in same frequency, other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of the SSB associated to the initial DL BWP, and are not configured with NCD-SSB for serving cell measurement;

-	If the serving cell is associated with both AO-SSB and OD-SSB in different frequency, other than the initial BWP, if any of the UE configured BWPs do not contain the frequency domain resources of SSB in the applied measurement object associated with the serving cell, and are not configured with NCD-SSB for serving cell measurement;

-	If the serving cell is not associated with SSB (i.e. SSB-less SCell), if the initial BWP or any of the UE configured BWPs do not contain the SSB frequency configured in the measurement object associated with the serving cell, and are not configured with NCD-SSB for serving cell measurement.

-	For CSI-RS based intra-frequency measurement, no measurement gap is needed;

-	For CSI-RS based inter-frequency measurement, a measurement gap configuration is always provided in the following cases:

-	If the UE only supports per-UE measurement gaps;

-	If the UE supports per-FR measurement gaps and any of the serving cells are in the same frequency range of the measurement object.

In non-gap-assisted scenarios, the UE shall be able to carry out such measurements without measurement gaps. In gap-assisted scenarios, the UE cannot be assumed to be able to carry out such measurements without measurement gaps.

Network may request the UE to measure NR and/or E-UTRA carriers in RRC_IDLE or RRC_INACTIVE via system information or via dedicated measurement configuration in RRCRelease. If the UE was configured to perform measurements of NR and/or E-UTRA carriers while in RRC_IDLE or in RRC_INACTIVE, it may provide an indication of the availability of corresponding measurement results to the gNB in the RRCSetupComplete message. The network may request the UE to report those measurements after security activation. The request for the measurements can be sent by the network immediately after transmitting the Security Mode Command (i.e. before the reception of the Security Mode Complete from the UE).

If the UE was configured to perform measurements of NR and/or E-UTRA carriers while in RRC_INACTIVE, the gNB can request the UE to provide corresponding measurement results in the RRCResume message and then the UE can include the available measurement results in the RRCResumeComplete message. Alternatively, the UE may provide an indication of the availability of the measurement results to the gNB in the RRCResumeComplete message and the gNB can then request the UE to provide these measurement results.