# 3GPP TSG RAN WG2 Meeting #130      	                           R2-2505503
Bengaluru, Indian, August 25 – 29, 2025

Agenda item:	8.1.2.3

Source:	Apple

Title:	Remaining issues on LCM procedure of AI/ML based positioning

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. The WI objective on general LCM framework, including applicable functionalities, is led by RAN2 which is copied below:

By RAN2#130 [2], most of essential issues on LCM of AI/ML based positioning have been closed. However, there are still many open issues to resolve. For example, CR Rapporteur of TS 37.355 provide a list of open issues in summary report [3].

In this contribution, we discuss the following open issues:

- Open issue LPP-6a: Number of PRUs in NR-PRU-DL-Info

- Open issue LPP-18: Consistency between training and inference

- Open issue LPP-19: Applicability of BM related agreements

- Open issue LPP-20: LPP Impacts related to Case 3a/3b

- Open issue LPP-21: "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19)

# 2 Discussion

## 2.1 Open issue LPP-6a: Number of PRUs in NR-PRU-DL-Info

The issue is:

Issue description:

Currently, the IE NR-PRU-DL-Info provides PRU measurements for a single PRU. However, the issue was raised at RAN2#130 whether more PRUs are needed to be included for training purposes.

If more PRUs are needed, a LMF can always provide multiple Provide Assistance Data messages. However, the Request Assistance Data message may need to include an indication of e.g., number of PRUs requested (if needed).

Proposed resolution:

Companies to provide contributions to the following meeting on whether more PRUs are needed to be included for training purposes, and if so, the corresponding signalling details.

We think the existing IE NR-PRU-DL-Info with single PRU can work because LMF can always provide multiple Provide Assistance Data messages for different PRUs (if needed). Supporting multiple PRUs in one single LPP message is an optmization and needs further spec impact. It should be avoided at late stage of Rel-19. Thus, we propose:

Proposal 1 (Open issue LPP-6a): Keep the existing IE NR-PRU-DL-Info with PRU measurement of single PRU, i.e. don’t support multiple PRUs in NR-PRU-DL-Info.

## 2.2 Open issue LPP-18: Consistency between training and inference

The issue is:

Issue description: At RAN2#130, contribution R2-2503403 "Discussion on consistency between training and inference for AI POS", vivo et al. was discussed which resulted in the following agreement: To ensure the consistency between training and inference, the UE should be able to request assistance data associated with a specific group of TRPs. FFS the request associated information in on demand prs request.. Proposed resolution: Companies to provide contributions to the following meeting on details for "the request associated information in on demand prs request".

In legacy LPP, the UE can on-demand request a PRS by providing either required PRS’s properties (e.g. periodicity and bandwidth) or QCL information (e.g. DL PRS ID) in IE NR-On-Demand-DL-PRS-PerFreqLayer-r17.

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-On-Demand-DL-PRS-PerFreqLayer-r17

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...,

p128-v1760, p256-v1760, p512-v1760}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17		ENUMERATED {n2, n4, n6, n12, ..., n1-v1800}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReqTRPlist-r17	DL-PRS-QCL-InformationReqTRPlist-r17	OPTIONAL,

...

}

DL-PRS-QCL-InformationReqTRPlist-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

DL-PRS-QCL-InformationReqPerTRP-r17

DL-PRS-QCL-InformationReqPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17						INTEGER (0..255),

nr-PhysCellID-r17					NR-PhysCellID-r16							OPTIONAL,

nr-CellGlobalID-r17					NCGI-r15									OPTIONAL,

nr-ARFCN-r17						ARFCN-ValueNR-r15							OPTIONAL,

dl-prs-QCL-InformationReqSet-r17	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-QCL-InfoReq-r17,

...

}

Observation 1: In legacy, the UE can on-demand request a PRS by providing either required PRS’s properties (e.g. periodicity and bandwidth) or QCL information (e.g. DL PRS ID) in IE NR-On-Demand-DL-PRS-PerFreqLayer-r17.

In our understanding, the motivation to request DL PRS from a group TRP(s) is for the following scenario 1:

- Scenario 1: after the model training, the UE may stay in the same area, but the NW turns off some TRPs used in training stage. In this case, the UE may request the LMF to turn on the same group of TRPs used in training.

- Scenario 2: the UE may move to a new area. In this case, the UE may not request or LMF should not turn on the TRPs used in training.

We think it is a valid scenario. On detailed signaling, please note that RAN1#121 has agreed to introduce associated ID to implicitly indicate geographical coordinates of a group TRP(s) in the same cell [3].

Agreement For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

Observation 2: RAN1#121 has agreed to introduce associated ID to implicitly indicate geographical coordinates of a group TRP(s) in the same cell.

Thus, we think the simplest way is to introduce both associated ID and cell information (NCGI or PCI+ARFCN) in IE NR-On-Demand-DL-PRS-PerFreqLayer-r17. Below is an example of ASN.1:

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...,

p128-v1760, p256-v1760, p512-v1760}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17		ENUMERATED {n2, n4, n6, n12, ..., n1-v1800}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReqTRPlist-r17	DL-PRS-QCL-InformationReqTRPlist-r17	OPTIONAL,

...

[[

AssociatedID      INTEGER (0..63) 										OPTIONAL,

nr-PhysCellID-r16 NR-PhysCellID-r16 									OPTIONAL,

nr-CellGlobalID-r16 NCGI-r15 											OPTIONAL,

nr-ARFCN-r16 ARFCN-ValueNR-r15 										OPTIONAL

]]

}

Based on above analysis, we propose:

Proposal 2 (Open issue LPP-18): To support on demand request for DL PRS of a group of TRPs, introduce both associated ID and cell information (NCGI or PCI+ARFCN) in IE NR-On-Demand-DL-PRS-PerFreqLayer-r17.

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...,

p128-v1760, p256-v1760, p512-v1760}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17		ENUMERATED {n2, n4, n6, n12, ..., n1-v1800}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReqTRPlist-r17	DL-PRS-QCL-InformationReqTRPlist-r17	OPTIONAL,

...

[[

AssociatedID      INTEGER (0..63) 										OPTIONAL,

nr-PhysCellID-r16 NR-PhysCellID-r16 									OPTIONAL,

nr-CellGlobalID-r16 NCGI-r15 											OPTIONAL,

nr-ARFCN-r16 ARFCN-ValueNR-r15 										OPTIONAL

]]

}

## 2.3 Open issue LPP-19: Applicability of BM related agreements

The issue is:

Issue description: As mentioned in [POST130][023][AI PHY] 38.305 CR (CATT), since the RAN2 intention is to have the same design for BM case and positioning case as much as possible, whether the agreements made for BM applicability reporting can also be applicable to positioning case 1. Whether the following agreements made for BM applicability reporting are also applicable to positioning case 1 and whether there are any associated LPP impacts: - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. Proposed resolution: Companies to provide contributions to the following meeting.

As summarized by CR Rapporteur, the possible LPP impacts are the following two aspects:

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

- Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed.

On 1), we think that AI positioning can reuse how the UE decides applicability in AI BM. In detail, the UE decides the applicable functionalities based on NW-side additional conditions, UE-side additional conditions and model availability in device. UE-side additional condition is not explicitly reported to the NW.

On 2), we don’t think any LPP spec impact is needed. In AI BM, it supports initial reporting and subsequent reporting with different RRC messages (i.e. RRCReconfigurationComplete and UAI) because legacy RRC UE capability messages can’t indicate dynamic change of UE capability. However, UE capability message in LPP (i.e. LPP ProvideCapabilties) already supports reporting dynamic change of UE LPP capability. Thus, no LPP spec change is needed.

Observation 3: In AI/ML based BM, it supports initial reporting and subsequent reporting with different RRC messages (i.e. RRCReconfigurationComplete and UAI) because legacy RRC UE capability messages can’t indicate dynamic change of UE capability. However, UE capability message in LPP (i.e. LPP ProvideCapabilties) already supports reporting dynamic change of UE LPP capability.

Thus, we propose:

Proposal 3 (Open issue LPP-19): Same as AI/ML based BM, the UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

## 2.4 Open issue LPP-20: LPP Impacts related to Case 3a/3b

The issue is:

Issue description:

Determine whether there are any LPP impacts related to Case 3a/3b.

Proposed resolution:

Companies to provide contributions to the following meeting.

We don’t see immediate RAN2 spec impacts on Case 3a/3b for now. Further work can be triggered by LS from SA2/RAN3.

Proposal 4 (Open issue LPP-20): On LPP spec impact on Case 3a/3b, RAN2 assume no further work is needed unless triggered by LS from SA2/RAN3 (if any).

## 2.5 Open issue LPP-21: "Associated ID" for TRP Location Coordinates

The issue is:

Issue description: The following issues have been discovered during LPP running CR review: - Must the "Associated ID" for the TRP coordinates be associated with a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated with a cell) being handled? And how to implement RAN1 agreement that UE does not expect to receive different values of associated ID for TRPs belonging to the same cell? - Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs? - Is the value range of 0..255 for the "Associated ID" sufficient? In particular if the "Associated ID" should be "configured per cell"? - Are the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both? - If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16), must a UE support also implicit location info (nr-TRP-ImplicitLocationInfo-r19) (and vice versa)? Proposed resolution: Wait for more RAN1 input and/or companies to provide contributions to the following meeting.

We think CR Rapporteur identified a good list of open issues related to associated ID in AI/ML based positioning. So, we would like to discuss them one by one.

Issue 1: Must the "Associated ID" for the TRP coordinates be associated with a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated with a cell) being handled? And how to implement RAN1 agreement that UE does not expect to receive different values of associated ID for TRPs belonging to the same cell?

In RAN1#121 [3], it was agreed that associated ID is cell specific:

- an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15

We understand that RAN1 intended to reuse conclusion of cell specific associated ID in AI/ML BM. However, we think this RAN1 agreement needs further discussion due to the following 3 considerations:

- Cell specific associated ID is an artificial restriction in AI positioning: Different from AI/ML BM, associated ID is generated by LMF (rather than by cell in AI/ML BM) because it is provided in LPP Assistance Data. Therefore, the associated ID is naturally applied to multiple cells. Only supporting cell specific associated ID for AI positioning is an artificial restriction.

- Cell specific associated ID can’t support PRS-only TRPs without cell information: Please note that cell information (including NCGI and PCI+ARFCN) is optional in existing LPP TRP information, i.e. PRS-only TRPs don’t have cell information and thereby they can’t be supported in cell specific associated ID. This will introduce another artificial restriction for AI positioning.

- Extra requirement for NW to configure same associated ID for TRPs in same cell: This is RAN1 agreement to ensure the UE don’t receive different associated ID for TPRs in same cell.

Figure.1: Issue illustration if one TRP (e.g. TRP 5) is configured with different associated IDs in same cell.

Observation 3: Only supporting cell specific associated ID in AI positioning will introduce the following artificial restrictions:

- Different from AI BM, associated ID is generated by LMF (rather than by cell in AI BM) because it is provided in LPP Assistance Data. Therefore, the associated ID is naturally applied to multiple cells.

- Cell specific associated ID can’t support PRS-only TRPs without cell information.

- Extra requirement for NW to configure same associated ID for TRPs in same cell.

Thus, we think RAN2 need to re-consider associated ID in AI positioning.

Proposal 5 (Open issue LPP-21): RAN2 discuss the following two way-forward on associated ID:

- Alt-1: Remove the artificial restriction of per cell configured associated ID in AI positioning (i.e. associated ID is PLMN specific). Send LS to RAN1 to inform RAN2 decision.

- Alt-2: Keep RAN1 conclusion of cell specific associated ID with the following restrictions:

  - Only apply to TRPs with cell information (i.e. either NCGI or PCI+ARFCN is mandatory present in TRP-ImplicitLocationInfoElement-r19).

  - Capture RAN1 agreed NW requirement in LPP spec: “One TRP can’t be configured with different values of nr-AIML-AssociatedID in the same cell.”

Issue 2: Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs?

In our understanding, RAN1 intention on associated ID is to cover same information as existing info#7, which already includes DL PRS resource ID and ARPs associated with the TRP.

|  | Information | UE-based DL-TdoA | UE-based DL-AoD |
| --- | --- | --- | --- |
| 1 | Physical cell IDs (PCIs), global cell IDs (GCIs), ARFCN, and PRS IDs of candidate NR TRPs for measurement |  |  |
| 7 | Geographical coordinates of the TRPs served by the gNB (include a transmission reference location for each DL-PRS Resource ID, reference location for the transmitting antenna of the reference TRP, relative locations for transmitting antennas of other TRPs) |  |  |

So, we understand it is just RAN1 agreement wording issue to only use TRP. Thus, we think associated ID is also applied to associated DL PRS resource ID and ARPs.

Proposal 6 (Open issue LPP-21): RAN2 confirm that associated ID is also applied to geographical coordinates of TRP associated DL PRS resource set and ARPs.

Issue 3: Is the value range of 0..255 for the "Associated ID" sufficient? In particular if the "Associated ID" should be "configured per cell"?

We think this issue depends on outcome of Proposal 5. If RAN2 decide to go Alt-2 (i.e. only supporting cell specific associated ID), we think 0…255 is sufficient.

Issue 4: Are the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both?

As both explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) carry the same information (i.e. geographical coordinates of TRP), we think it doesn’t make sense to provide both of them at the same time. Otherwise, it will confuse the UE which one to use.

Observation 4: As both explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) carry the same information (i.e. geographical coordinates of TRP), we think it doesn’t make sense to provide both of them at the same time. Otherwise, it will confuse the UE which one to use.

Thus, we think it is necessary to clarify that nr-TRP-ImplicitLocationInfo-r19 can be present only when nr-TRP-LocationInfo-r16 is absent.

Proposal 7 (Open issue LPP-21): nr-TRP-ImplicitLocationInfo-r19 can be present only when nr-TRP-LocationInfo-r16 is absent. It is up NW implementation which one to provide in LPP DL Assistance data.

Issue 5: If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16), must a UE support also implicit location info (nr-TRP-ImplicitLocationInfo-r19) (and vice versa)?

Explicit TRP location info (NR-TRP-LocationInfo-r16) was introduced in Rel-16 while implicit TRP location information (nr-TRP-ImplicitLocationInfo-r19) is introduced in later release. We don’t think the UE supporting a legacy feature is mandated to support a feature of later release. On the other hand, we think it is reasonable to mandate the UE supporting implicit TRP location information to support explicit TRP location info. Thus, we propose:

Proposal 8 (Open issue LPP-21): Introduce a separate UE capability on whether supporting implicit TRP location info (nr-TRP-ImplicitLocationInfo-r19) and the UE supports this capability shall also support explicit TRP location info (NR-TRP-LocationInfo-r16). But the UE supports e explicit TRP location info is NOT mandated to support implicit TRP location info.

# 3 Conclusion

In this contribution, we discuss remaining issues on LCM procedure of AI/ML based positioning. Our observations are:

Observation 1: In legacy, the UE can on-demand request a PRS by providing either required PRS’s properties (e.g. periodicity and bandwidth) or QCL information (e.g. DL PRS ID) in IE NR-On-Demand-DL-PRS-PerFreqLayer-r17.

Observation 2: RAN1#121 has agreed to introduce associated ID to implicitly indicate geographical coordinates of a group TRP(s) in the same cell.

Observation 3: In AI/ML based BM, it supports initial reporting and subsequent reporting with different RRC messages (i.e. RRCReconfigurationComplete and UAI) because legacy RRC UE capability messages can’t indicate dynamic change of UE capability. However, UE capability message in LPP (i.e. LPP ProvideCapabilties) already supports reporting dynamic change of UE LPP capability.

Observation 4: Only supporting cell specific associated ID in AI positioning will introduce the following artificial restrictions:

- Different from AI BM, associated ID is generated by LMF (rather than by cell in AI BM) because it is provided in LPP Assistance Data. Therefore, the associated ID is naturally applied to multiple cells.

- Cell specific associated ID can’t support PRS-only TRPs without cell information.

- Extra requirement for NW to configure same associated ID for TRPs in same cell.

Observation 5: As both explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) carry the same information (i.e. geographical coordinates of TRP), we think it doesn’t make sense to provide both of them at the same time. Otherwise, it will confuse the UE which one to use.

Based on observations, our proposals can be found below:

Proposal 1 (Open issue LPP-6a): Keep the existing IE NR-PRU-DL-Info with PRU measurement of single PRU, i.e. don’t support multiple PRUs in NR-PRU-DL-Info.

Proposal 2 (Open issue LPP-18): To support on demand request for DL PRS of a group of TRPs, introduce both associated ID and cell information (NCGI or PCI+ARFCN) in IE NR-On-Demand-DL-PRS-PerFreqLayer-r17.

NR-On-Demand-DL-PRS-PerFreqLayer-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...},

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...,

p128-v1760, p256-v1760, p512-v1760}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17		ENUMERATED {n2, n4, n6, n12, ..., n1-v1800}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReqTRPlist-r17	DL-PRS-QCL-InformationReqTRPlist-r17	OPTIONAL,

...

[[

AssociatedID      INTEGER (0..63) 										OPTIONAL,

nr-PhysCellID-r16 NR-PhysCellID-r16 									OPTIONAL,

nr-CellGlobalID-r16 NCGI-r15 											OPTIONAL,

nr-ARFCN-r16 ARFCN-ValueNR-r15 										OPTIONAL

]]

}

Proposal 3 (Open issue LPP-19): Same as AI/ML based BM, the UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device.

Proposal 4 (Open issue LPP-20): On LPP spec impact on Case 3a/3b, RAN2 assume no further work is needed unless triggered by LS from SA2/RAN3 (if any).

Proposal 5 (Open issue LPP-21): RAN2 discuss the following two way-forward on associated ID:

- Alt-1: Remove the artificial restriction of per cell configured associated ID in AI positioning (i.e. associated ID is PLMN specific). Send LS to RAN1 to inform RAN2 decision.

- Alt-2: Keep RAN1 conclusion of cell specific associated ID with the following restrictions:

  - Only apply to TRPs with cell information (i.e. either NCGI or PCI+ARFCN is mandatory present in TRP-ImplicitLocationInfoElement-r19).

  - Capture RAN1 agreed NW requirement in LPP spec: “One TRP can’t be configured with different values of nr-AIML-AssociatedID in the same cell.”

Proposal 6 (Open issue LPP-21): RAN2 confirm that associated ID is also applied to geographical coordinates of TRP associated DL PRS resource set and ARPs.

Proposal 7 (Open issue LPP-21): nr-TRP-ImplicitLocationInfo-r19 can be present only when nr-TRP-LocationInfo-r16 is absent. It is up NW implementation which one to provide in LPP DL Assistance data.

Proposal 8 (Open issue LPP-21): Introduce a separate UE capability on whether supporting implicit TRP location info (nr-TRP-ImplicitLocationInfo-r19) and the UE supports this capability shall also support explicit TRP location info (NR-TRP-LocationInfo-r16). But the UE supports e explicit TRP location info is NOT mandated to support implicit TRP location info.

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#130, Chair Notes.

[3] R2-2505702, LPP open issues for feature "AI/ML for NR air interface”, Qualcomm Incorporated (Rapporteur).