3GPP TSG-RAN WG2 Meeting #131       	                      R2-2505510

Bangalore, India, 25th-29th Aug 2025

Agenda item:	8.1.1

Source:	CMCC, OPPO

Title:	Discussion and draft reply LS on functionality activation

WID/SID:	NR_AIML_air-Core

Document for:	Discussion

- Introduction

In last meeting, RAN4 sent a LS [1] on functionality activation to RAN2 to clarify which interpretation is aligned with RAN2 agreements.

RAN4 is discussing activation delay for AI/ML functionality activation based on the following RAN2 agreement: RAN2#129 agreements: - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).. Companies in RAN4 have different interpretations about the following RAN2 agreement: “the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4”. - Interpretation 1: the applicable functionalities are already activated (i.e., the applicable functionalities activation is completed) before reporting applicable functionalities via RRCReconfigurationComplete in step 4 - Interpretation 2: UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

This contribution will discuss the clarification of RAN2 agreements.

- Discussion

- In legacy periodic CSI reporting, if the UE sends RRCReconfigurationComplete message to the network, it means that the UE is ready to perform CSI measurements and report, and no additional latency for activation is required. In our understanding, the similar logical can be applied in AI/ML based beam management.

- From the perspective of operator, the network needs to know whether the functionality is ready for inference. We understand that the following RAN2 agreement means the applicable functionalities are ready for inference upon reporting applicable functionalities via RRCReconfigurationComplete in step 4, i.e. the applicable functionalities activation is completed.

RAN2#129 agreements: - If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- On the other hand, RAN1 has the following agreements on functionality activation, which also means that the applicable periodic CSI report is activated upon reporting RRCReconfigurationComplete message.

RAN1 agreement in LS [2]: • In Step 4, UE reports applicability for all the above A) one or more CSI-ReportConfig and/or B) set(s) of inference related parameters - FFS on whether/what other information along with the applicability is needed - If A) is configured in Step 3, - Applicable aperiodic CSI Report and semi-persistent CSI report can be activated/triggered by NW after the applicability reported. - Applicable periodic CSI Report is considered as activated only if the applicability of the corresponding CSI-ReportConfig is reported in RRCReconfigurationComplete.

- Based on the above analysis, we think Interpretation 1 in RAN4 LS is aligned with RAN1 and RAN2 agreements, and we suggest to update the wording to follow RAN1/RAN2 agreements to avoid further misalignment across WGs.

- Interpretation 1: the applicable functionalities are already considered activated (i.e., the applicable functionalities activation is completed) before upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

- For Interpretation 2, the UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 means that the additional latency for activation is required and there is delay for model inference, which is not aligned with RAN1 and RAN2 agreements.

- Proposal 1: RAN2 confirm that Interpretation 1 is common understanding for AI/ML functionality activation for option A, and update the wording as follows:

- Interpretation 1: the applicable functionalities are already considered activated (i.e., the applicable functionalities activation is completed) before upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

- Furthermore, for option B, the UE receives the full inference configuration via RRCReconfiguration in step 5, and subsequently sends RRCReconfigurationComplete to the network. We think the same mechanism used for option A can be also applied, thereby achieving a unified design for option A and B. Otherwise, the additional activation latency would be introduced for option B. That is, RRCReconfigurationComplete indicates that the applicable functionalities are activated (i.e., the applicable functionalities activation is completed).

- Figure 1. Basic flow for applicability reporting

- Proposal 2: For option B, RRCReconfigurationComplete after step 5 (RRCReconfiguration with full inference configuration) means that the applicable functionalities are activated (i.e., the applicable functionalities activation is completed).

- Based on the above discussion, we prepare a draft LS on functionality activation to RAN4 in Annex.

- Proposal 3: RAN2 to adopt the draft LS in Annex on functionality activation to RAN4.

# 3	Conclusion

Here are the proposals for reply LS on functionality activation.

- Proposal 1: RAN2 confirm that Interpretation 1 is common understanding for AI/ML functionality activation for option A, and update the wording as follows:.

- Interpretation 1: the applicable functionalities are already considered activated (i.e., the applicable functionalities activation is completed) before upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

- Proposal 2: For option B, RRCReconfigurationComplete after step 5 (RRCReconfiguration with full inference configuration) means that the applicable functionalities are activated (i.e., the applicable functionalities activation is completed).

- Proposal 3: RAN2 to adopt the draft LS in Annex on functionality activation to RAN4.

# 4	Reference

- R4-2508085 LS on AI/ML functionality activation

- R1-2410898 Reply LS on applicable functionality reporting for beam management UE-sided model

# 5	Annex-Draft LS

3GPP TSG-RAN WG2 Meeting #131	                  R2-250XXXX

Bangalore, India, 25th-29th Aug, 2025

Title:	[Draft] Reply LS on AI/ML functionality activation

Response to:	R2-2505045 (R4-2508085)

Release:	Release 19

Work Item:	NR_AIML_air-Core

Source:	CMCC [to be RAN2]

To:	RAN4

Cc:	RAN1

Contact person:	Ningyu Chen

chenningyu@chinamobile.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: None

# 1	Overall description

RAN2 thanks RAN4 for the LS on AI/ML functionality activation. RAN2 discussed the following interpretations for option A, and agreed that Interpretation 1 is common understanding with updating:

- Interpretation 1: the applicable functionalities are already considered activated (i.e., the applicable functionalities activation is completed) before upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

- Interpretation 2: UE starts to activate the applicable functionalities (i.e., the applicable functionalities activation is not completed) upon reporting applicable functionalities via RRCReconfigurationComplete in step 4

Furthermore, RAN2 discussed the similar issue for option B, after the RRCReconfiguration including full inference configuration in step 5, RRCReconfigutationComplete also means that the applicable functionalities are activated (i.e., the applicable functionalities activation is completed).

RAN2 kindly requests RAN4 to take above replies into account.

# 2	Actions

To RAN4

ACTION: 	RAN2 respectfully requests RAN4 to take above replies into account.

# 3	Dates of next TSG RAN WG2 meetings

TSG-RAN WG2 Meeting #131-bis			October 13 to 17, 2025      Prague, CZ

TSG-RAN WG2 Meeting #132			    November 17 to 21, 2025	   Dallas, US