3GPP TSG-RAN WG2 Meeting #131			           		R2-2505524

Bangalore, India, August 25 - 29, 2025

Agenda item:	8.1.2.2

Source:	Samsung, Apple, Xiaomi, Qualcomm, Google, Interdigital, NTT DOCOMO, Nokia, LG Electronics, vivo

Title:	Support of associated ID

Document for:	Discussion and Decision

# Introduction

Based on RAN1 higher layer parameter list, RAN1 concluded that the value range of associatedIDforSetA-r19 is up to RAN2.

| RAN2 Parent IE | Parameter name in the spec | New or existing? | Description | Value range |
| --- | --- | --- | --- | --- |
| CSI-ReportConfig | associatedIDforSetA-r19 | new | UE may assume the similar properties of a DL Tx beam or beam set/list associated with the same associatedIDforSetA or with the same associatedIDforSetB. When Set B (configured by resourcesForChannelMeasurement) is equal or a subset of set A (configured by resourcesForSetA (i.e., NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set B is within the NZP-CSI-RS-ResourceId/SSB-Index in the resource set for Set A), only AssociatedIDforSetA is configured, otherwise, associatedIDforSetA is configured for Set A and associatedIDforSetB is configured for Set B. | Up to RAN 2 |

Furthermore, in addition to the range of associated ID, RAN1 couldn’t conclude on whether/how UE assumption can be applicable for multiple cells.

In order to define associated ID, there could be some aspects we need to look at. Therefore, we would like to discuss overall the usage of associated ID.

# Cell specific ID vs. Multi-cell specific ID

An associated ID is introduced to ensure consistence between training and inference. More specifically, it is used to distinguish different NW side additional conditions. If a model is trained under a specific NW condition, it may not achieve the same accuracy/performance if the same model is used for inference in NW having different NW condition. It is understood that although RAN1 doesn’t officially agree what constitutes for NW side additional condition, it would include different antenna pattern, codebook sets, etc.

The main issue with the associate ID is whether it is cell specific or multiple-cell specific. RAN1 has not concluded this issue because there was a diverged view and it was generally expected that it can be discussed in RAN2 from system operation point view.

Regarding this issue, we should consider about the granularity of categorizing dataset and consequence in model training. If associated ID is defined as cell specific, it would mean that UE should categorize dataset per cell and per associated ID, which requires UE to train model per cell and per associated ID and consequently more complexity/burden in model training at UE side.

Observation 1: cell specific associated ID will require UE to train a huge number of models and consequently more complexity/burden in model training at UE side.

In addition, NW side additional conditions may not be different in each cell because it is more related to network deployment and antenna configuration. One model trained for a certain network deployment can be reused to multiple cells.

Observation 2: NW side addition condition is likely to be the same for multiple cells with the same NW configuration.

On the other hand, the concern of multiple cell specific ID from NW side is potential risk that that the limited number of associated IDs might reveal the network deployment configuration. Therefore, a cell specific associated ID may be preferred if it is concerned that the limited number of associated IDs might reveal the network deployment configuration.

Observation 3: cell specific associated ID may be preferred if it is concerned that the limited number of associated IDs might reveal the network deployment configuration.

One may think that define different RRC parameters for cell-specific ID and multiple cell-specific ID would be required if it is assumed to have different size. However, given that there is no requirement on the range of associated ID, the same size could be assumed for cell-specific ID and multiple cell-specific ID.

Observation 4: same RRC parameter for cell specific ID and multiple cell-specific ID could be defined.

Considering pros and cons for each option, it would be desirable to provide flexibility to UE/NW implementation. That is, the signaling of associated ID should be defined to support both approaches i.e. cell specific ID and multiple cell-specific IDs.

One flexible solution is to support both single cell and multi-cell associated ID based on NW implementation. That is, the network may allocate an Associated ID to a single cell and/or to multiple cells. For example, NW may allocate different associated IDs to the different cell if it wants to operate as cell specific ID. In order for UE to handle associated IDs for training and inference, at least, it should be ensured that associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.

Proposal 1: Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

Proposal 2: Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.

# Conclusion

In this contribution, we discussed the pros and cons depending on coverage of associated ID i.e. cell specific and multi-cell specific. Based on the discussion, we propose the following observations and proposals.

Observation 1: cell specific associated ID will require UE to train a huge number of models and consequently more complexity/burden in model training at UE side.

Observation 2: NW side addition condition is likely to be the same for multiple cells with the same NW configuration.

Observation 3: cell specific associated ID may be preferred if it is concerned that the limited number of associated IDs might reveal the network deployment configuration.

Observation 4: same RRC parameter for cell specific ID and multiple cell-specific ID could be defined.

Proposal 1: Both single cell and multi-cell associated ID can be supported based on NW implementation (i.e., the network may allocate an Associated ID to a single cell and/or to multiple cells).

Proposal 2: Associated IDs shall be unique within a PLMN in that they can only be associated with one same/similar beam deployment.