3GPP TSG-RAN WG2 Meeting #131	R2-2505530

Bangalore, India, 25th – 29th Aug. 2025

Agenda item:	8.4.3

Source:	Samsung

Title:	RRM measurement relaxation and offloading in RRC Idle /
Inactive Mode

Document for:	Discussion & Decision

1 Introduction

Energy conservation has emerged as an important issue in recent mobile communication network environments. Accordingly, RAN2 starts with the new Work Item (WI), Low-power wake-up signal and receiver for NR (LP-WUS/WUR) [1].

In this contribution, we would like to discuss the detailed RRM measurement relaxation and offloading methods to support LP-WUS in RRC IDLE/INACTIVE mode.

- Discussion

  - Exit condition on RRM measurement relaxation

As previously agreed upon by RAN2, NR supports MR serving cell RRM measurement relaxation and/or measurement offloading under conditions where the UE resides near the cell center, assuming that RRM measurement on LR is feasible and supported. Subsequently, RAN2 defined the ‘Serving cell measurement offloading’ and ‘Serving cell RRM measurement relaxation’ as follows:

Serving cell measurement offloading

: The serving cell measurement of an LP-WUS capable UE is fully offloaded to LR and no further MR serving cell measurements are mandated

Serving cell RRM measurement relaxation

: serving cell RRM measurement of an LP-WUS capable UE is relaxed with larger periodicity

If a UE supports the serving cell measurement offloading or RRM measurement relaxation, the serving cell quality could be measured using LP-WUR via LR-SS.

Considering the use cases of serving cell measurement offloading and relaxation, the triggering criteria for the entry/exit condition itself does not need to differ from the current criteria of NR RRM measurement relaxation.

Currently the ongoing topic of discussion is about exit condition of MR serving cell RRM measurement relaxation when UE is in RRC connected mode. Specifically, debate around whether the ‘not fulfilling the entry condition’ suffices, or whether exit condition include MR and/or LR-based measurements.

Given that explicit exit criteria for MR neighbour RRM measurement relaxation, analogous criteria could conceivably apply here too, by ‘not fulfilling the entry condition’. Nevertheless, considering regular LP-SS measurements conducted on the serving cell, LR-derived exit thresholds seems more reasonable.

Proposal 1: (RRC-7, 38304-3) Similar to serving cell offloading, the exit conditions of MR serving cell RRM measurement relaxation, in addition to "not fulfilling the entry condition," should also consider:

    - If the serving cell quality from LR is below a threshold

  - Low Mobility

Building upon the discussions from the previous RAN2 127bis meeting, RAN2 has established the working assumption of having FFS on low mobility. In the RAN2 130 meeting, RAN2 discussed whether to include the low mobility criteria, but it was not concluded.

#127bis - Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Here we propose to reaffirm and refine the existing working assumption regarding neighbor cell measurement relaxation for UEs capable of LP-WUS, especially with FFS on UE with low mobility.

While the original WA effectively addresses key aspects, it does not account for scenarios where rapid movement within a cell could lead to suboptimal behavior. Specifically, relying solely on measurement-based thresholds may result in inefficient operation for high-speed UEs, such as those on vehicles or trains. These UEs often toggle between serving cell measurement offloading/relaxation and regular MR paging, causing ping-pong effects and increased energy consumption.

To mitigate this issue and enhance efficiency, we propose integrating a "low mobility" criterion as an optional feature that networks can configure based on specific deployment requirements. This refinement aims to optimize LP-WUS operations by reducing unnecessary power consumption and improving overall system performance.

Proposal 2: (RRC-10, 38304-6) For Rel19 MR serving/neighbour cell measurement relaxation for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

Furthermore, we may extend the discussion of including ‘low-mobility’ to ‘serving cell measurement offloading’.

Proposal 3: (RRC-10, 38304-6) For Rel19 serving cell measurement (full) offloading for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

3 Conclusion

RAN2 is requested to discuss and agree to the following proposals:

Proposal 1: (RRC-7, 38304-3) Similar to serving cell offloading, the exit conditions of MR serving cell RRM measurement relaxation, in addition to "not fulfilling the entry condition," should also consider:

    - If the serving cell quality from LR is below a threshold

Proposal 2: (RRC-10, 38304-6) For Rel19 MR serving/neighbour cell measurement relaxation for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

Proposal 3: (RRC-10, 38304-6) For Rel19 serving cell measurement (full) offloading for UEs capable of LP-WUS, UE not at cell edge and UE with low mobility are reused.

References

[1] RP-234056 Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC