3GPP TSG-RAN WG2 Meeting #131                                    	R2-2505536

Bengaluru, India, 25th – 29th August 2025

Agenda item:	8.9.3

Source:	Qualcomm Incorporated, European Space Agency, German Aerospace Center

Title:	Support of OCC

Document for:	Discussion and Decision

- Introduction

In RAN2#129bis, following agreements were made.

If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

In RAN2#129bis, following agreements were made.

We continue the discussion on OCC for CB-msg3-EDT in the next meeting.

In this document, we provide details on OCC.

- Discussion

OCC is important to improve the system capacity further by multiplexing two users in the same carrier resource. Since network can configure separate resources for non-OCC and OCC based transmission, there will be no issue of UE capability differentiation. For DSA, successful response message includes contention resolution ID regardless of resource, i.e., time, frequency and OCC. Therefore, the transmission group specific RNTI agreed for DSA will work. The procedure for Msg3 transmission and Msg4 reception defined for non-OCC based CB-Msg3 EDT can be reused for OCC-based CB-Msg3 EDT.

The procedure for Msg3 transmission and Msg4 reception defined for non-OCC based CB-Msg3 EDT is applicable for OCC-based CB-Msg3 EDT. Only additional work is to define signalling support for OCC configuration.

For non-OCC based CB-Msg3 EDT, the total number of PUSCH resources available is given as (Number of occasions)*(Number of frequency domain resources per occasion). In the capacity analysis, we assume 2 UEs choosing a different occasion or a different frequency resource do not interfere with each other. We consider a collision or contention to occur only when both the occasion and frequency resource chosen by both UEs are identical.

For OCC based CB-Msg3 EDT, the total number of PUSCH resources available is given as (Number of occasions)*(Number of frequency domain resources per occasion)*(Number of OCC sequences). The OCC sequences can be considered as orthogonal resources similar to frequency resources. UE1 transmitting in (Occasion1,Frequency_Resource1, OCC_index1) does not interfere with UE2’s transmission in (Occasion2,Frequency_Resource2, OCC_index2) provided at least the occasion, frequency resource or occ index is different.

There will also be no power imbalance issue even if 2 UEs are using different OCC index in the same occasion and frequency resource since the UEs will still be orthogonal. For example, let’s say UE 1 uses codeword [1 1] and let’s say UE 2 has a power imbalance of 10 dB, which can be considered as that UE 2 is using codeword [√10 -√10] with respect to UE 1. However, the codewords [1 1] and codeword [10  -√10] are still orthogonal. To verify this assumption, we perform some link level simulations with power imbalance.

We assume a power imbalance of 10 dB between 2 UEs i.e., UE 2 has a power boost of 10 dB more power than UE1. We assume TDM DMRS for both UEs, since it has been agreed as a working assumption already. Also note that TDM DMRS won’t be impacted by power imbalance. We concentrate on the performance of UE1.

We use the following parameters for the link level simulations - TBS 328 bits, 4Rus and 8 repetitions, 3.75kHz SCS, NTN-TDL-C Rural 10 degree elevation with 5Hz doppler, CFO b/w 200Hz.

We show the performance of UE1 with OCC2 in Figure 1 for 3 cases -

- UE2 with no power imbalance

- UE2 with 10dB power boost

- UE2 no transmission

Figure 1 Comparison between PER of UE1 with OCC2 with UE2 (a)no power imbalance, (b)10dB power imbalance and (c)no transmission and (d)UE transmitting without OCC

It can be observed from Fig. 1 that the loss due to power imbalance is minimal. If we observe the SNRs required to achieve 1% PER, we see that there is only a ~0.25 dB loss between the no power imbalance case and the 10 dB power imbalance case. At –6dB SNR we see that the case when UE2 does not transmit or transmits with no power imbalance, the PER is around 1.2% while it is similar(2.2%) when UE2 transmits with 10dB power boost.

Noting that the PER are close to 1% for both scenarios at –6dB, it is reasonable to assume that the packet error probability of a transmission in an (occasion, frequency resource, OCC index) resource is independent of transmissions by other UEs in resources where atleast one of the occasion, frequency resource or OCC index is different.

Even a power imbalance of 10 dB results in only a performance degradation that is minimal i.e., 0.25 dB for NPUSCH with OCC.

It can be observed from Fig. 1, that for OCC2 with TDM DMRS we can achieve ~1% PER when using 4 RUs and 8 repetitions(Transmission duration of 1024ms ignoring UL gaps).

For non OCC case, at -6db SNR, we can achieve ~1% PER  when using 6 RUs and 4 repetitions(Transmission duration of 768ms ignoring UL gaps).

Considering a 180kHz UL carrier reserved only for CB-Msg3, when not using OCC, we have on average, 62.5 NPUSCH resources per second while for the case of OCC2, we have 93.75 NPUSCH resource per second(50% more resources). Considering only the same collision probability, a 50% increase in NPUSCH resources would allow us to support 50% more UEs/sec.

Figure 2 Collision probability vs UE arrival rate for non OCC and OCC2 CB-Msg3 EDT for different values of N(Number of NPUSCH occasions per CB-Msg3 window) and k(Number of replicas)

In short, OCC-based CB-Msg3 EDT improves total system performance and procedural texts defined for non-OCC based CB-Msg3 EDT will be applicable to OCC-based CB-Msg3 EDT. Therefore, OCC should be supported as an important feature for UL capacity enhancement.

Confirm that OCC-based CB-Msg3 EDT is supported.

- Conclusion

Following proposals are made:

Observation 1.	The procedure for Msg3 transmission and Msg4 reception defined for non-OCC based CB-Msg3 EDT is applicable for OCC-based CB-Msg3 EDT. Only additional work is to define signalling support for OCC configuration.

Observation 2.	Even a power imbalance of 10 dB results in only a performance degradation that is minimal i.e., 0.25 dB for NPUSCH with OCC.

Proposal 1	Confirm that OCC-based CB-Msg3 EDT is supported.