3GPP TSG RAN WG2 #131       	                            R2-2505548

Bangalore, India Aug 25th – 29th, 2025

Agenda Item:	8.6.3

Source:	     KDDI Corporation

Title:	Discussion on L1 event triggered measurement reporting for LTM

WID:       	NR_Mob_Ph4-Core

Tdoc Type:	Discussion

Document for: 	Discussion and decision

- Introduction

In RAN #106, the Rel-19 WID on NR mobility enhancements Phase 4 was updated [1]:

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management [RAN1] - Specify CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch [RAN1] NOTE: RAN1 WG to decide on whether to support CSI report before or during the LTM cell switch, as part of the CSI acquisition procedure.

In RAN2#130, the following agreement was made:[2]

Agreements on L1 event-triggered MR

- UE deactivates SP CSI-RS resource of candidate cells (other than the target cell) after cell switch. FFS on the target cell.

- Instead of candidate cell id and SP CSI-RS resource set id, LTM-CSI-ResourceConfigId is included into SP CSI-RS activation/deactivation MAC CE.

- Confirm working assumption, i.e. four codepoints to represent one of {type1, type2, type3, type4}, as an agreement. For detailed description of type 1, it can be handled as part of running CR preparation.

- (MAC-6) For the truncated MR MAC CE, the UE determines the beam to be included in the MAC CE, based on the priority: Type 1 beam > Type 2 beam > Type 3 beam > Type 4 beam. If the UE cannot include all of the beams with the same priority, the beams are selected in the (truncated) MR MAC CE based on measured RS quality. Among all selected beams, the UE puts the best quality beam as the reference beam as the first one included in the (truncated) MR MAC CE.

- Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

- Same priority between regular MR MAC CE and truncated MR MAC CE. It is up to UE implementation how to handle/include if more than one triggering events are pending and UL grant is not sufficient for all regular L1 MR MAC CEs.

- No prohibit timer is applied to L1 event-triggered MR.

In RAN2#130, the following agreement was made:[2]

POST130][117][MOB] (Nokia) Scope: Discuss issues of coexistence between L1 event triggered MR and mTRP (source cell). - Whether to allow coexistence between event triggered MR and mTRP (source cell)? - If companies agree to allow it, how to define the current beam? Intended outcome: Summary discussion. Deadline: Long email discussion. Simple solution should be preferable and considering next meeting is the last meeting for WI completion, everything (including companies’ consensus, one solution, and the corresponding TP) should be ready for next meeting.

In this contribution, we will discuss remaing issues from RAN2#129 and share our views on L1 event triggered MR.

- Discussion

2.1 Coexistence between event triggered report and mTRP (source cell)

When mTRP is configured in the serving cell, the UE uses the best beam (in terms of RSRP) of the two “current beams” for LTM event evaluation. Implementation is also relatively simple, requiring only the addition of normal measurement value comparison and selection logic on the UE side, ensuring high compatibility with current specifications. Adopting the best beam makes the most of the characteristics of multiple TRPs and is expected to improve service reliability and handover response.

When multiple TRPs are operating, simply reporting the RSRP value makes it difficult for the network to determine which TRP/beam the value is associated with, making it difficult to make optimal control and handover decisions. If the adopted TCI state (beam/TRP) can be made clear by utilizing reserved bits in the MR MAC CE, the network's cell switching decisions and scheduling will also be more accurate. Specification changes can also be kept to a minimum, and this can be fully implemented within the R19 specification.

mTRP contributes to improved coverage and reliability, improving system performance. Coexistence between L1 event triggered report and mTRP (source cell) is technically straightforward and should be supported within the Rel-19 specifications.

Proposal 1: When mTRP is configured in the serving cell the UE uses the best beam (in terms of RSRP) of the two “current beams” for LTM event evaluation.

Proposal 2: MR MAC CE shall indicate the TCI state associated with the reported RSRP value for the serving/current beam.

Proposal 3: The coexistence between event triggered L1 measurement reporting and mTRP for the source cell is supported in Release 19 LTM.

2.2 Truncated L1 MR MAC CE: Handling of the current beam

Maintaining Consistency and Identification of Measurement Results: The current serving beam is particularly important information for network control and mobility control, serving as the basis for event triggers and state transitions. Even when transmitting via truncated MR MAC CE, consistently reporting the current serving beam as a set facilitates measurement interpretation and consistent evaluation on the network side, preventing malfunctions and quality degradation.

Simplifying Specifications and Implementation: Clear rules governing the order and priority of beams that UEs should report reduce the risk of implementation differences and operational inconsistencies between different vendors.

Ensuring Minimum Measurement Quality: Information on the trigger beam alone cannot consistently capture serving beam transitions and current status. Reporting the "currently most important beam" information (=current serving beam), which is important for mobility evaluation and handover decisions, as a set without omitting any information, ensures minimum measurement quality. When sufficient UL resources are unavailable, prioritizing measurement reliability and refraining from transmission altogether is more rational from a system quality perspective than making unreasonable reports.

Proposal 4: Confirm the working assumption which is following.

- Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

2.3 SP CSI-RS: Handling of the activated SP CSI-RS after LTM cell switch

Efficient resource utilization: SP CSI-RS resources other than those of the target cell are no longer needed after an LTM switch, reducing UE power consumption and measurement load.

Importance of continued CSI acquisition in the target cell: CSI reports are still required in the target cell to accurately grasp the radio conditions immediately after the switch and to optimize mobility, maintain the serving beam, and maintain throughput.

Consistency with LTM procedures: Continuous CSI feedback in the target cell allows the network to quickly optimize parameters and control resources, minimizing quality degradation and the risk of communication outages after a cell switch.

Simplified implementation and improved manageability: Setting clear rules (after switch: target cell → continue, candidate cell → stop) ensures UE behavior when implemented and makes operational troubleshooting easier.

Therefore, at least for the target cell, it should be continued for CSI acquisition purpose.

Proposal 5: UE continues to measure the activated semi-persistent CSI-RS resources of target cell after LTM cell switch.

- Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1: When mTRP is configured in the serving cell the UE uses the best beam (in terms of RSRP) of the two “current beams” for LTM event evaluation.

Proposal 2: MR MAC CE shall indicate the TCI state associated with the reported RSRP value for the serving/current beam.

Proposal 3: The coexistence between event triggered L1 measurement reporting and mTRP for the source cell is supported in Release 19 LTM.

Proposal 4: Confirm the working assumption which is following.

- Working assumption: If network configures to include the current serving beam in MR, the UE always includes the current serving beam in the truncated MR MAC CE. In this case, minimum size of truncated MR MAC CE is {at least one triggered beam + the current serving beam}. If grant is not enough for that, the UE does not assemble a truncated MR MAC CE.

Proposal 5: UE continues to measure the activated semi-persistent CSI-RS resources of target cell after LTM cell switch.

- References

[1] RP-250339 Revised Work Item: NR mobility enhancements Phase 4 (Apple Inc, China Telecom)

[2] RAN2 #130 Report from session on LTE V2X and NR SL (Vice Chairman: Samsung)