3GPP TSG-RAN WG2 Meeting #131	R2-2505550

Bengaluru, India, 25th - 29th August, 2025

Agenda Item:	8.9.2

Source: 	OPPO

Title:  	Discussion on Store & Forward satellite operation

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the following agreements for Store & Forward satellite operation were made.

Agreements:

1.	When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start.

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

3.	Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

This contribution aims to provide our considerations on the remaining open issues based on the outcome of [Post130][306], [Post130][308], and [Post130][309].

- Discussion

  - Working Assumption on S&F mode operation indication for neighbour cells (36304 Open issue 2)

In RAN2#130 meeting, the following working assumption was made regarding SF mode operation indication for neighbour cells.

Working Assumption:

1.	In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

As mentioned in the WA, it can only be confirmed if we converge on the corresponding UE behaviour, i.e., the main controversial part for this WA. However, during the post-meeting email discussion, it seems companies still have different views on the corresponding UE behaviour if introducing SF mode operation indication for neighbour cells, e.g., up to UE implementation to use these indications or specify UE behaviour on cell reselection enhancement based on these indications.

Besides, we cannot see significant issue to block the S&F satellite operation feature even without this enhancement. Considering the last meeting before Rel-19 feature freeze in RAN2, it is suggested not to do this kind of over-optimization in Rel-19.

- Considering no common understanding on the corresponding UE behaviour, RAN2 does not consider introducing SF mode operation indication for neighbour cells.

If RAN2 would like to progress more, one straightforward direction is to allow Rel-19 UEs not supporting S&F operation to start neighbour cell measurement for reselection before transiting to “S&F operation” mode, which is much similar to UE behaviour related to t-Service introduced in Rel-17 NTN.

- Rel-19 UEs not supporting S&F operation allow to start neighbour cell measurement for reselection before transiting to S&F operation mode, like the behaviours adopted for t-Service.

  - NAS configured S&F monitoring list (36304 Open issue 3)

In RAN2#130 meeting, the following agreement was made. It is still FFS on the usage of NAS configured S&F monitoring list in AS layer, e.g., for discontinuous coverage procedure.

Agreements:

2.	In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list.

Note that in SA2 CR S2-2502513 [1] for TS 23.401, based on NOTE B in clause 4.13.9 as shown below, it is fully up to UE implementation to use the S&F monitoring list in NAS layer, even though in this case there is increased probability that it will not be able to complete the NAS procedure.

NOTE B: How UE behaves when receiving the S&F Monitoring List is up to UE implementation. When a UE receives a S&F Monitoring List and the UE access a satellite that supports Store and Forward Satellite operation that is not on the S&F Monitoring List there is increased probability that it will not be able to complete the NAS procedure. The UE can continue to use the previously provided S&F Monitoring List, if the MME did not send one and the UE has previously been provided with one.

Considering that even in NAS layer the S&F monitoring list is informative, i.e., the NAS behavior related to S&F monitoring list is up to UE implementation, there is also no motivation for AS layer to specify the related AS operation. We understand that it is proper to also leave to UE implementation to use the NAS configured S&F monitoring list in AS layer for AS operations, e.g., discontinuous coverage procedure, or whether to prioritize frequency(ies)/cell(s) corresponding to the satellite(s) indicated in the NAS configured S&F monitoring list during cell reselection procedure. No any AS impact is needed.

- Up to UE implementation on the usage of the NAS configured S&F monitoring list in AS layer, i.e., no any AS impact needed.

  - Paging enhancement for S&F Mode operation (36304 Open issue 4)

Based on the outcome of the post-meeting email discussion [Post130][308], paging enhancement for S&F Mode operation is still an open issue, which is expected to provide company contribution for discussion. The main intention for this enhancement is to relax paging monitoring for power saving in case that cell operating in S&F mode doesn’t have any MT data buffered for UE.

We understand although the power saving gain for this enhancement is clearer, when we look into the detailed solution, additional discussion is needed, e.g., for a cell operating in S&F mode has MT data buffered for UE, if the stored MT data has already been transmitted, whether the indication needs to be changed or not, thus whether SI modification notification is needed or not. Considering the last meeting before Rel-19 feature freeze in RAN2, it is suggested not to consider further enhancement to avoid additional debate of detailed solution.

- RAN2 does not consider paging monitoring relaxation for cell operating in S&F mode without any stored MT data in Rel-19.

  - AS Capability for support of S&F (36306 Open issue)

Based on the outcome of the post-meeting email discussion [Post130][309], one open issue for capability is whether to introduce a UE AS capability with signalling for support of S&F mode or not, given S&F mode capability is already exchanged between UE and MME in NAS layer. The decision on this issue depends on whether eNB need to know this information, e.g., to decide whether to keep the UEs or release the UEs when feeder link connection is lost or will be lost soon.

For UE in RRC_CONNECTED mode, we understand to rely on NW implementation for UE handling if eNB switches between S&F operation mode and normal mode, and the different ways of NW handling in AS layer includes: 1) release UE to RRC_IDLE mode, 2) trigger handover to switch the UE to a correct cell, or 3) update the radio bearer configuration to adapt the operation mode of the current cell, for Rel-19 UEs supporting S&F operation. Therefore, to assist eNB to make the proper implementation, eNB also needs to know the information of whether the UE supports S&F operation or not.

Considering that NAS layer capability for UE supporting S&F operation has already been introduced, we understand eNB could know this information of UE based on the network-side interaction between eNB and MME by network implementation. For example, when MME detaches the UE by MME-initiated Detach Procedure in NAS layer, at the same time MME also sends S1 Release Command message to eNB to release the S1-MME signalling connection for the UE. Therefore, eNB can simply follow the indication of MME to perform AS handling accordingly. It is redundant for UE to indicate the capability in both AS layer and NAS layer. Considering that the issue is more about network side, it is also suggested to send LS to RAN3 to check and confirm if any issue/impact on RAN3.

- RAN2 assumes no need to introduce AS capability signalling for UE supporting of S&F mode for now, given the introduced NSA capability for this. Send LS to RAN3 for further check/confirmation.

  - AS-NAS interaction on S&F mode transition information (RRC Open issue ERI04/vivo03/ZTE02)

Based on the outcome of the post-meeting email discussion [Post130][306], one open issue for RRC is AS-NAS interaction on the information of transition time between “S&F operation mode” and “normal mode”.

In RAN2#129bis meeting, RAN2 made the following agreements on this.

Agreements:

1.	For UEs supporting S&F, the UE AS indicates the information on transition time (if any) from current “S&F operation mode” to “normal mode” (if received) to the upper layers. Whether/how it is used by the upper layers is up to CT1.

2.	We introduce an indication in system information for the normal mode to S&F mode transition, at least for NAS use. FFS on the details (e.g. whether we can link this to other existing information). The information on transition time for the normal mode to S&F mode transition is sent from AS to NAS

Before sending LS to CT1 for further confirmation, RAN2 firstly assumed that NAS layer needs the information on transition time between “S&F operation mode” and “normal mode”, thus made decision that AS-NAS interaction on S&F mode transition information is needed.

In the current RRC running CR, the related AS-NAS interaction has already been implemented as shown below, i.e., upon reception of SIB31-NB, UE will forward the t-ModeSwitching to upper layers, if present.

5.2.2.39 Actions upon reception of SystemInformationBlockType31 Upon receiving SystemInformationBlockType31 (SystemInformationBlockType31-NB), the UE shall: 1> start or restart timer T317 with the duration ul-SyncValidityDuration from the subframe indicated by epochTime. 1> forward the t-ModeSwitching to upper layers, if present.

However, based on the Reply LS from CT1 [2], as shown below, CT1 clarifies that there is no impact to the NAS layer for the moment according to the current requirements for S&F mode provided by SA2.

1. Overall Description: CT1 would like to thank RAN2 for their LS C1-253033 / R2-2503070 on providing S&F mode indications in system information for the transition between "normal mode" and "S&F operation mode" to NAS. CT1 discussed the topic and concludes that there is no impact to the NAS layer for the moment according to the current requirements for S&F mode provided by SA2. 2. Actions: To RAN2 group: ACTION: CT1 kindly requests RAN2 to take the above information into account.

After further check with our SA2/CT1 colleagues internally, we understand that NAS layer does not need to rely on S&F mode transition information forwarded by AS layer. From SA2/CT1 perspective, the current solution for S&F satellite operation in NAS layer designed by SA2 is self-consistent, which mainly focuses on how to finish the end-to-end exchange of NAS signalling / data traffic for cells in “S&F operation mode”, rather than delay the triggering of some NAS procedures from UE side until cell is working in “normal mode”. In details, as mentioned in TS 23.401, the NAS signalling / data traffic is handled in a sequence of steps reflecting the intermittent availability of the service link when the satellite can exchange data with the UE and of the intermittent availability of the feeder link when the satellite can exchange data with the core network. Therefore, UE AS layer does not need to forward the S&F mode transition information to NAS layer. We propose to remove the related UE behaviour to avoid unnecessary UE effort for cross-layer interaction.

- RAN2 agrees that UE AS layer does not forward the information on transition time between “S&F operation mode” and “normal mode” to upper layers, and to remove the related change in the current RRC running CR, which reverts the previous agreements made on AS-NAS interaction on S&F mode transition information.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

WA: S&F mode operation indication for neighbour cells (36304 Open issue 2)

- Considering no common understanding on the corresponding UE behaviour, RAN2 does not consider introducing SF mode operation indication for neighbour cells.

- Rel-19 UEs not supporting S&F operation allow to start neighbour cell measurement for reselection before transiting to S&F operation mode, like the behaviours adopted for t-Service.

NAS configured S&F monitoring list (36304 Open issue 3)

- Up to UE implementation on the usage of the NAS configured S&F monitoring list in AS layer, i.e., no any AS impact needed.

Paging enhancement for S&F Mode operation (36304 Open issue 4)

- RAN2 does not consider paging monitoring relaxation for cell operating in S&F mode without any stored MT data in Rel-19.

AS Capability for support of S&F (36306 Open issue)

- RAN2 assumes no need to introduce AS capability signalling for UE supporting of S&F mode for now, given the introduced NSA capability for this. Send LS to RAN3 for further check/confirmation.

AS-NAS interaction on S&F mode transition information (RRC Open issue ERI04/vivo03/ZTE02)

- RAN2 agrees that UE AS layer does not forward the information on transition time between “S&F operation mode” and “normal mode” to upper layers, and to remove the related change in the current RRC running CR, which reverts the previous agreements made on AS-NAS interaction on S&F mode transition information.

4. Reference

[1] S2-2502513, Support of S&F Monitoring List in S&F Mode, Huawei, HiSilicon, Samsung

[2] R2-2505004, Reply LS on S&F mode indications to NAS (C1-254119; contact: CICT Mobile)