3GPP TSG-RAN WG2 Meeting #131	R2-2505551

Bengaluru, India, August. 25th – 29th, 2025

Agenda Item:	8.9.3

Source: 	OPPO

Title:  	Discussion on CB-msg3 EDT and msg4 enhancement

Document for:	Discussion and Decision

Introduction

In last RAN2 meeting, the agreements for uplink capacity enhancement were made as follows.

Agreements:

1.	The maximum TBS could be different for different CE levels.

2.	Due to only CE mode A is supported for eMTC NTN, only 1 separate RSRP thresholds and 2 CE levels are supported (revised agreement)

3.	Multiple contention resolution IDs could be included in CB-MSG4, the information related to multiple UEs can be multiplexed in the MAC PDU.

4.	The number of Msg3 replies in one Msg4 can be left to eNB implementation. Expect no SPEC impact.

5.	The HARQ feedback resource information can be included in the CB-Msg4 together with contention resolution ID which identity the specific UE. RAN2 could revisit this proposal if RAN1 has some concern.

6.	Whether to send the HARQ feedback for CB-Msg4 can be controlled by NW. UE does not send HARQ NACK.

7.	For NB-IoT, the SubCarrierSpacing of the HARQ feedback for CB-Msg4 is same as the CB-Msg3.

8.	Reuse the existing format of HARQ ACK allocation signalling in the DCI. There is 2-bit HARQ ACK resource for eMTC and 4-bit HARQ ACK resource for NB-IoT. Reuse the meaning of DCI field in R1 SPEC. Send LS to RAN1 for information on all RAN2 decisions related to HARQ feedback

9.	Introduce a new MAC PDU for CB-Msg4 including new types of MAC sub-header and a new type of MAC payload

10.	The MAC PDU for CB-Msg4 consists of sub-header(s) followed by MAC payload and optional padding if needed.

11.	Introduce a new CB BI MAC sub-header in CB-MSg4 for backoff parameter. There is 4 bits BI for backoff indication.

12.	Introduce a new CB-Msg3 Response (CBR) MAC sub-header in CB-Msg4. It has 1bit E for sub-header/payload indication, 2 bits T for sub-header type, 1bit T2 for HARQ ACK resource present, 1 bit T3 for TAC present, 1 bit T4 for C-RNTI present and 2bit R for reservation (the field names and sub-header names in above agreements could be further changed during MAC running CR review)

Agreements – part 2:

1.	The CB-Msg3-EDT configuration is configured per carrier (including anchor and non-anchor carrier). Within each carrier, the CB-Msg3-EDT could be configured per CE level

2.	Regarding the mapping of NPUSCH resource to enhanced coverage levels, enhanced coverage levels are numbered from 0 and the mapping of NPUSCH resources to enhanced coverage levels are done in increasing [number of repetition] order (as legacy RACH)

3.	For NB-IoT, when multiple carriers provide CB-Msg3-EDT resources for the same enhanced coverage level, the NB-IoT UE selects the carrier based on the probabilities of each carrier. A new probability parameter for anchor carrier is introduced in SIB22-NB. The remaining probability is evenly split among the non-anchor carriers.

4.	When max re-attempt number for current CE level has been reached, the UE does not move to the next CE level (FFS on the details of the failure behaviour)

5.	The TAC is optionally used in the CB-Msg3 response.

6.	RAN2 assumes that NTA=0 for initial CB-msg3 transmission. Include this in the LS to RAN1 and RAN4

7.	RAN2 assumes the length of the TAC field is 6 bits (we can revisit this if there is major R1 impact on TA calculation)

Agreements – part 3:

1.	Introduce a new CB Data MAC sub-header in CB-MSg4 for MAC SDU for logical channel data. It has 1 bit E for subhead/payload indication, 2 bits T for subhead type, 5 bits LCID, 7 bits or 15 bits L for MAC SDU length, 1 bit F for 15 bits L indication. There is one L field per CB Data sub-header except for the last sub-header.

2.	Introduce a new CB-Msg3 Response (CBR) with variable length. It has 48-bit contention resolution ID, optional HARQ ACK, optional TAC, optional 16-bit C-RNTI.

Agreements:

1.	HARQ process 0 is used to transmit all the CB-Msg3 replicas in the transmission window (RV0 is used to transmit the first repetition of each CB-Msg3 replica in the transmission window)

2.	We do not specify another way of starting Msg4 monitoring window, i.e. it is confirmed that the Msg4 monitoring window always starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not)

3.	A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution.

Working Assumption:

The formula for RNTI for mMsg4 monitoring is:

RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id.

•	X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT,

•	Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19,

•	Y is ceil (Msg4_WS/Msg3_WP),

•	CE_level is the CE level, 0 <= CE_level < 3

•	carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0,

0 <= carrier_id < 16

Can come back to check if the NW can also simply configure RNTI = X

Agreements – part 2:

1.	The value of X is 4097 for NB-IoT and 2401 for eMTC

2.	The value of Msg4_WS is the maximum Msg4 window size

3.	The value of Msg3_WP is the minimum Msg3 window periodicity

In this contribution, we would like to discuss the remaining open issues based on the outcome of [Post130][306] and [Post130][307].

Discussion

CB-RNTI calculation (Open issue MAC-2)

In last RAN2 meeting, a working assumption was made for CB-RNTI calculation

Working Assumption: The formula for RNTI for mMsg4 monitoring is: RNTI=X + Msg3_W_index modulo (Y) + Y*CE_level + 3*Y*carrier_id. • X is the starting RNTI for Msg4 reception, which can be defined by RAN2 e.g. X=2401 for eMTC or 4097 for NB-IoT, • Msg3_W_index is the index of Msg3 transmission window within a periodicity of 1024 SFNs and index 0 corresponds to the Msg3 transmission window starts at the SFN defined by IE startSFN-r19, • Y is ceil (Msg4_WS/Msg3_WP), • CE_level is the CE level, 0 <= CE_level < 3 • carrier_id is the index of the UL carrier of the CB-Msg3 resources, anchor carrier has index 0, 0 <= carrier_id < 16 Can come back to check if the NW can also simply configure RNTI = X Agreements – part 2: 1. The value of X is 4097 for NB-IoT and 2401 for eMTC 2. The value of Msg4_WS is the maximum Msg4 window size 3. The value of Msg3_WP is the minimum Msg3 window periodicity

For the formula, further discussion is needed for the following issues based on the summary of the post email discussion.

Proposal 1: RAN2 to confirm the working assumption on CB-RNTI formula [10/11]. Proposal 2: For the agreed CB-RNTI formula from P1, RAN2 to discuss - Whether to define Msg3_W_index to floor(start SFN_id of Tx window / minimum Tx window length or periodicity]. - Whether to define Y = 16. - Whether starting value X is optionally configurable.

Firstly, we think the WA on CB-RNTI formula can be confirmed. Our considerations for other open issues are provided as follows.

For definition of Msg3_W_index, since there isn’t an explicit index associated with each Msg3 transmission window, the expression in proposal2 can be a more normative way to describe the position of a Msg3 transmission window within a periodicity of 1024 SFNs. The Tx window length or periodicity configured for the selected CE level and carrier can be used as the divisor, i.e., there is no need to use the minimum value to unify the Msg3_W_index range among all CE levels and carriers since it is the Y to restrict the RNTI values that can be used for each CE level and carrier by the modulo operation. Using the Tx window periodicity is slightly preferred based on which the ratio can better reflect the actual order of the window.

Regarding the Y in the formula, we prefer to have a fixed value for it since in a reasonable configuration the Msg4 window size and Msg3 transmission size would be in a certain proportion, i.e., it is not practical to configure a large Msg4 window for a small-sized Msg3 transmission window/periodicity. The values of Y can be different under different configurations. To make the use of RNTI more efficient, it is proposed that the value of Y is configurable by the NW, in which way a suitable value can be provided based on the actual resource configuration.

For the starting value of X, it is fine to skip the whole value range of RA-RNTI as in current MAC running CR, i.e., no need to make it configurable.

The WA on CB-RNTI calculation formula can be confirmed.

Define Msg3_W_index to floor (start SFN_id of Tx window / Tx window periodicity) where the Tx window periodicity configured for the selected CE level and carrier is used.

Define the Y as a fixed value and the value is configured by the NW.

The starting value X is defined to skip the whole value range of RA-RNTI which is not configurable.

Power ramping (Open issue MAC-6)

RAN2 made an initial progress regarding whether to support power ramping for CB-Msg3 transmission in RAN2#130 meeting.

9. RAN2 assumes power ramping should be supported for CB-msg3-EDT (for both eMTC and NB-IoT) should be supported and will ask RAN1 for confirmation and in case which parameters should apply

In RAN1 reply LS, it is pointed out that they did not and will not evaluate the potential performance of power ramping for CB-Msg3-EDT.

Reply to Q1: RAN1 has not evaluated the potential performance of power ramping for CB-msg3-EDT, and it is likely that there will not be sufficient time to evaluate this topic within the R19 timeframe • For open loop power control the following UL power control parameters can be reused for CB-msg3-EDT • p0-UE-NPUSCH-r16 and alpha-r16 for NB-IoT NTN

Based on RAN1 response, it is preferred that power ramping is not captured for CB-Msg3-EDT at this late stage.

Power ramping is not supported for CB-Msg3 transmission.

Need of NW/UE processing time (Open issue MAC-9)

In last RAN2 meeting, how to start the Msg4 monitoring window was confirmed as follows.

2. We do not specify another way of starting Msg4 monitoring window, i.e. it is confirmed that the Msg4 monitoring window always starts at the end of CB-Msg3-EDT transmission window plus UE-eNB RTT (FFS NW/UE processing time is needed or not)

It is still open whether NW/UE processing time needs to be considered when starting the Msg4 monitoring window. During the post email discussion, some companies expressed a preference for not introducing any processing time for simplicity. It is possible that the processing time has been covered by CB-Msg3 transmission window in case that the UE has finished all replica transmissions at a relative early point within the window. But for other cases such as SA or the replica transmissions finish at the last PUSCH occasion in the window, NW/UE processing time is still required. Since we aim to have a unified solution, it is proposed that the processing delay should be considered when starting the CB-Msg3 response window for saving UE power consumption, i.e., 3ms processing time can be chosen according to the proposal from MAC rapporteur.

Proposal 4: RAN2 to choose between 3ms or 0ms processing time when determine the Msg4 monitoring starts.

3ms processing time is considered for the start of CB-Msg3 response window.

Fallback (Open issue MAC-15)

In last RAN2 meeting, it was agreed that when the maximum re-attempt number for current CE level has been reached, the UE can not fallback to the next CE level.

4. When max re-attempt number for current CE level has been reached, the UE does not move to the next CE level (FFS on the details of the failure behaviour)

As an MAC open issue, further discussion is needed on the details of the failure behaviour. In RAN2#130 meeting, whether the UE can initiate the legacy 4-step RACH in case of CB-Msg3-EDT procedure fails was discussed but no consensus was made.

Proposal 13: UE can initiate the legacy 4-step RA when the CB-Msg3 procedure fails. FFS on the condition for the UE to initiate the fallback to legacy 4-step RA. - Ericsson thinks this should be formulated in a way that failure of the procedure is indicated to upper layers. MTK agrees. ZTE wonders which UL Continue in the next meeting

In PUR, when fallback indication is received or the PUR procedure is considered as failure in MAC, RRC will be notified and the follow-up actions is up to UE implementation, e.g., trigger EDT or legacy RACH.

#### 5.3.3.3d UE actions upon receiving PUR indications from lower layers The UE shall: 1> if repetition adjustment is indicated by lower layers: 2> update numRepetitions (npusch-NumRepetitionsIndex in NB-IoT) in previously stored pur-Config in accordance with the received indication; 1> if pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is configured and timing advance adjustment is indicated by lower layers: 2> replace the serving cell reference (N)RSRP value with the current serving cell (N)RSRP value (see 5.3.3.19); For CP transmission using PUR, upon indication from lower layers that transmission using PUR is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. Upon reception of PUR fallback or PUR failure indication from lower layers, the procedure ends. NOTE: For transmission using PUR, further UE actions upon reception of PUR fallback or PUR failure indication from lower layers (see TS 36.321 [6]) is left up to implementation.

For CB-Msg3-EDT, we understand the same behaviour can be followed, i.e., when the maximum attempt time of CB-Msg3 is reached, MAC indicates CB-Msg3-EDT failure to upper layer. In RRC, the follow-up UE action is left up to implementation when CB-Msg3-EDT failure indication is received from lower layers.

MAC indicates CB-Msg3-EDT failure to upper layer when the maximum attempt time of CB-Msg3 is reached.

In RRC, further UE actions upon reception of CB-Msg3-EDT failure indication from lower layers is left up to implementation.

Multiple TBS (Open issue MAC-17)

As an MAC open issue, further discussion is needed on whether to allow multiple TBS as in EDT. In legacy EDT, the small TBS subset is supported to reduce the redundant padding on Msg3, thereby saving some power and time. RAN1 input is required if we want to support the same for CB-Msg3-EDT, while the WID has been declared as completion in RAN1 in last meeting. For progress, it is proposed that multiple TBS is not supported for CB-Msg3-EDT.

Multiple TBS is not supported for CB-Msg3-EDT.

CQI reporting (Open issue MAC-20)

In current MAC spec, an NB-IoT UE needs to include the channel quality reporting in Msg3 if configured by upper layer. For anchor carrier, CQI can be derived before NPRACH transmission, for non-anchor carrier, CQI is shall be derived from the channel quality measured during RAR reception. Since there is no Msg1/2 transmission during CB-Msg3-EDT, CQI derivation defined in current RAN4 specification is not enough to cover the case. For simplicity, it is proposed that CQI reporting is disabled in CB-Msg3-EDT for NB-IoT UE.

CQI reporting is not supported in CB-Msg3-EDT for NB-IoT UE.

CB-Msg3 EDT completion om CP solution (RRC-1.2)

In last RAN2 meeting, it was agreed that the CB-Msg3 EDT in CP solution can be considered as completed when a CB-Msg4 without RRC message but with contention resolution ID is received.

3. A CB-Msg4 without RRC message (but with contention resolution identity) is allowed as the complete response to the CB-Msg3 in CP solution.

While in current RRC running CR, this agreement is not captured. In PUR, the same enhancement was introduced, i.e., eNB may terminate the PUR procedure by sending a L1/L2 ACK, and the procedure is specified as follow in TS36.331.

#### 5.3.3.3d UE actions upon receiving PUR indications from lower layers The UE shall: 1> if repetition adjustment is indicated by lower layers: 2> update numRepetitions (npusch-NumRepetitionsIndex in NB-IoT) in previously stored pur-Config in accordance with the received indication; 1> if pur-RSRP-ChangeThreshold (pur-NRSRP-ChangeThreshold in NB-IoT) is configured and timing advance adjustment is indicated by lower layers: 2> replace the serving cell reference (N)RSRP value with the current serving cell (N)RSRP value (see 5.3.3.19); For CP transmission using PUR, upon indication from lower layers that transmission using PUR is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received. Upon reception of PUR fallback or PUR failure indication from lower layers, the procedure ends. NOTE: For transmission using PUR, further UE actions upon reception of PUR fallback or PUR failure indication from lower layers (see TS 36.321 [6]) is left up to implementation.

As yellow-highlighted, upon reception of L1/L2 ACK from lower layer, the PUR procedure is considered as terminated as if an empty RRCEarlyDataComplete message is received.

For CB-Msg3-EDT, if nothing is captured in RRC as in current running CR, the CB-Msg3-EDT will be considered as ongoing until T300 expires. However, T300 expiry declares CB-Msg3-EDT failure instead of successfully completion, which is not aligned with the agreement. Based on this consideration, it is proposed to capture the same UE behaviour as PUR upon the reception of CB-Msg3-EDT successfully completion indication from the lower layers.

For CB-Msg3-EDT in CP solution, the following UE behaviour should be captured in RRC specification: For CP transmission using CB-Msg3-EDT, upon indication from lower layers that transmission using CB-Msg3-EDT is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received.

OCC

In last RAN2 meeting, the discussion regarding whether or not to support OCC for CB-Msg3-EDT was postponed.

Proposal 7 RAN2 do not work on OCC for CB-Msg3-EDT in release 19. - QC does not support the proposal and thinks that OCC could be supported without additional work for RAN2 (procedure wise, only the corresponding configuration is needed) - Ericsson doubts that OCC would work in this case. Nokia agrees - Toyota thinks that from RAN2 perspective there is no problem to support OCC => We continue the discussion on OCC for CB-msg3-EDT in the next meeting.

If OCC is supported, further discussions are needed at least for the following issues:

- Power imbalanced issue: As indicated in RAN1 LS, OCC solution may be applied to CB-Msg3-EDT when the power imbalance is small. RAN2 assumed that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-Msg3 using OCC. While RAN1 effort is required to evaluate the feasibility of the solution and the values to define the RSRP ranges.

- RNTI calculation/CB-Msg4 impact: As agreed, if OCC supported separate resources will be used for non-OCC and OCC based transmission. For UEs selecting different resources, CB-Msg4 reception should also be distinguished, e.g., OCC is considered in RNTI calculation or including OCC index in CB-Msg4 payload.

Since this is the last meeting for WID completion and we may not have enough time to discuss these details, it is preferred OCC is not supported for CB-Msg3-EDT is R19.

OCC is not supported for CB-Msg3-EDT.

Conclusion

Based on the discussion in section2, we have following observation proposals:

Proposal 1	The WA on CB-RNTI calculation formula can be confirmed.

Proposal 2	Define Msg3_W_index to floor (start SFN_id of Tx window / Tx window periodicity) where the Tx window periodicity configured for the selected CE level and carrier is used.

Proposal 3	Define the Y as a fixed value and the value is configured by the NW.

Proposal 4	The starting value X is defined to skip the whole value range of RA-RNTI which is not configurable.

Proposal 5	Power ramping is not supported for CB-Msg3 transmission.

Proposal 6	3ms processing time is considered for the start of CB-Msg3 response window.

Proposal 7	MAC indicates CB-Msg3-EDT failure to upper layer when the maximum attempt time of CB-Msg3 is reached.

Proposal 8	In RRC, further UE actions upon reception of CB-Msg3-EDT failure indication from lower layers is left up to implementation.

Proposal 9	Multiple TBS is not supported for CB-Msg3-EDT.

Proposal 10	CQI reporting is not supported in CB-Msg3-EDT for NB-IoT UE.

Proposal 11	For CB-Msg3-EDT in CP solution, the following UE behaviour should be captured in RRC specification: For CP transmission using CB-Msg3-EDT, upon indication from lower layers that transmission using CB-Msg3-EDT is successfully completed, the UE shall perform the actions as specified in 5.3.3.4b as if an empty RRCEarlyDataComplete message was received.

Proposal 12	OCC is not supported for CB-Msg3-EDT.