3GPP TSG-RAN WG2 Meeting #131        											                          R2-2505571

Bengaluru, India, Aug 25th – 29th, 2025                                                     Revision of R2-2504528

Agenda item:	8.9.3

Source:	MediaTek Inc.

Title:	Discussion on CB-Msg3-EDT procedure

Document for:	Discussion and Decision

# Introduction

Rel-19 IoT NTN work item was revised in RAN#105 [1] and one of the objectives is to enhance EDT procedure:

- Support of Capacity enhancements for uplink

  - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]:

    - Msg3 transmission without msg1/ Random Access Response (RAR)

    - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete

In this contribution, we continue discussing the remaining issues on CB-Msg3-EDT.

# Discussion

Regarding the support OCC for CB-Msg3-EDT, RAN2 had made the following agreements:

- RAN2 will continue their work on CB-msg3 assuming that OCC2 might also apply to CB-msg3 transmission (“CB-NPUSCH”) (final decision whether this is feasible is up to RAN1). FFS whether an LS to indicate this to RAN1 is needed. - There will be a RSRP threshold that determines whether CB-Msg3 can be used (if the RSRP is below such threshold, PRACH will have to be used) (FFS if we need a separate set of thresholds, including a different minimum threshold, in case CB-msg3 EDT is combined with OCC) - RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC - It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used - If we will decide to support OCC for CB-msg3-EDT, separate resources will be used for non-OCC and OCC based transmission.

There is still a significant amount of unfinished normative work, such as the OCC resource design, details on the RSRP threshold for transmitting CB-Msg3 using OCC, how the UE chooses OCC-based and OCC-free CB-Msg3 resource, how the UE selects the OCC code, how the UE transmits replicas using OCC, whether an extension of the CB-Msg4 monitoring window is needed, UE capability, etc. It is unlikely that all of these can be completed in a single RAN2 meeting. Therefore, we suggest that the OCC for CB-Msg3 is not supported in Release 19.

Proposal 1: OCC for CB-Msg3 is NOT supported in Release 19.

Some companies are interested in introducing the legacy multiple TBS in EDT for CB-Msg3-EDT.However, there will be RAN1 impact as multiple TBS is mainly specified in RAN1. Considering that RAN1 has already completed its work on Rel-19 and the upcoming RAN2 meeting will be the last for Rel-19, the remaining time is limited. And there are other key outstanding issues that need to be addressed, we suggest that the multiple TBS of CB-Msg3-EDT is not supported in Rel-19.

Proposal 2: Multiple TBS of CB-Msg3-EDT is NOT supported in Rel-19.

In the legacy NPRACH Format 2 on non-anchors, if the field is absent in an entry of nprach-ParametersListFmt2 in SIB23-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SIB2-NB applies. If the corresponding entry in SIB2-NB is also absent, the value of the same field, if present, in the corresponding entry of the first occurrence of nprach-ParametersListFmt2 in the non-anchor carrier list applies.

#### 36.331 #### – NPRACH-ConfigSIB-NB The IE NPRACH-ConfigSIB-NB is used to specify the NPRACH configuration for the anchor and non-anchor carriers. NPRACH-ConfigSIB-NB information elements -- ASN1START NPRACH-ConfigSIB-NB-r13 ::= SEQUENCE { nprach-CP-Length-r13 ENUMERATED {us66dot7, us266dot7}, rsrp-ThresholdsPrachInfoList-r13 RSRP-ThresholdsNPRACH-InfoList-NB-r13 OPTIONAL, -- Need OR nprach-ParametersList-r13 NPRACH-ParametersList-NB-r13 } <skip> NOTE: - If the field is absent in an entry of nprach-ParametersList in SystemInformationBlockType22-NB, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies. - If the field is absent in the entry in nprach-ParametersListEDT, the value of the same field in the corresponding entry of nprach-ParametersList on the same UL carrier applies, if present. Otherwise, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies. - If the field is absent in an entry of nprach-ParametersListTDD in SystemInformationBlockType22-NB, the value of the same field in the corresponding entry of nprach-ParametersListTDD in SystemInformationBlockType2-NB applies. The field is mandatory present in nprach-ParametersListTDD in SystemInformationBlockType2-NB. - If the field is absent in an entry of nprach-ParametersListFmt2 in SystemInformationBlockType23-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB applies. Otherwise the value of the same field, if present, in the corresponding entry of the first occurence of nprach-ParametersListFmt2 in the non anchor carrier list applies. Otherwise, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies. - If the field is absent in an entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies. - If the field is absent in an entry of nprach-ParametersListFmt2EDT in SystemInformationBlockType23-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 on the same UL carrier applies. Otherwise, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB applies. Otherwise the value of the same field, if present, in the corresponding entry of the first occurence of nprach-ParametersListFmt2 in the non anchor carrier list applies. Otherwise, the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies. - If the field is absent in an entry of nprach-ParametersListFmt2EDT in SystemInformationBlockType2-NB, the value of the same field, if present, in the corresponding entry of nprach-ParametersListFmt2 in SystemInformationBlockType2-NB applies. Otherwise the value of the same field in the corresponding entry of nprach-ParametersList in SystemInformationBlockType2-NB applies.

This approach helps reduce air interface overhead. Now, with CB-Msg3-EDT configurations present in SIB2-NB and SIB22-NB for anchor and non-anchor carriers respectively, the same legacy approach can be applied to save air interface overhead.

To archive this goal, a new container IE within the CB-Msg3-ConfigSIB-NB is needed. If the container IE is absent, the value in SIB2-NB or the first entry of cb-Msg3-ConfigSIB-NB in SIB22-NB can be used.

Proposal 3: Add a container IE within CB-Msg3-ConfigSIB-NB. If this field is absent in an entry of cb-Msg3-ConfigSIB-NB in SIB22-NB, the value, if present, in the corresponding entry of cb-Msg3-ConfigSIB-NB in SIB2-NB applies. Otherwise, the value in the corresponding entry of first occurrence of cb-Msg3-ConfigSIB-NB in the SIB22-NB list applies.

The corresponding TP is put in Annex.

# Conclusion

Proposal 1: OCC for CB-Msg3 is NOT supported in Release 19.

Proposal 2: Multiple TBS of CB-Msg3-EDT is NOT supported in Rel-19.

Proposal 3: Add a container IE within CB-Msg3-ConfigSIB-NB. If this field is absent in an entry of cb-Msg3-ConfigSIB-NB in SIB22-NB, the value, if present, in the corresponding entry of cb-Msg3-ConfigSIB-NB in SIB2-NB applies. Otherwise, the value in the corresponding entry of first occurrence of cb-Msg3-ConfigSIB-NB in the SIB22-NB list applies.

# References

[1] RP-242397, “Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3”, Rapporteur (MediaTek)

# Annex: 36.331 TP for P2

#### –	CB-Msg3-ConfigSIB-NB

The IE CB-Msg3-ConfigSIB-NB is used to specify the CB-Msg3 EDT configuration.

CB-Msg3-ConfigSIB-NB information element

-- ASN1START

CB-Msg3-ConfigSIB-NB-r19 ::=		SEQUENCE {

cb-Msg3-ConfigSIB-NB-Container-r19		CB-Msg3-ConfigSIB-NB-Container-r19	OPTIONAL,	--Need OP

cb-Msg3-MinRSRP-Threshold-NB-r19		NRSRP-Range-NB-r14				OPTIONAL,	--Need OR

cb-Msg3-RSRP-CE-Level-NB-r19			CB-Msg3-RSRP-CE-Level-NB-r19	OPTIONAL,	--Need OR

cb-Msg3-ConfigList-NB-r19				CB-Msg3-ConfigList-NB-r19

}

cb-Msg3-ConfigSIB-NB-Container-r19 ::=	SEQUENCE {

cb-Msg3-MinRSRP-Threshold-NB-r19		NRSRP-Range-NB-r14				OPTIONAL,	--Need OR

cb-Msg3-RSRP-CE-Level-NB-r19			CB-Msg3-RSRP-CE-Level-NB-r19	OPTIONAL,	--Need OR

cb-Msg3-ConfigList-NB-r19				CB-Msg3-ConfigList-NB-r19

}

CB-Msg3-ConfigSIB-NB field descriptions

cb-Msg3-ConfigSIB-NB-Container If this field is absent in an entry of cb-Msg3-ConfigSIB-NB in SIB22-NB, the value, if present, in the corresponding entry of cb-Msg3-ConfigSIB-NB in SIB2-NB applies. Otherwise, the value in the corresponding entry of first occurrence of cb-Msg3-ConfigSIB-NB in the SIB22-NB list applies.