3GPP TSG-RAN WG2 Meeting #131	R2- 2505581

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.4.4

Source:	Nokia, Nokia Shanghai Bell

Title:	LP-WUS in RRC_CONNECTED

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In [213] post email discussions, the secondary DRX aspects are discussed with the following proposals:

Proposal 1: Confirm the following working assumption to support LP-WUS with dual DRX group.

Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

Proposal 2: If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time. (NOTE: One company has concern)

Proposal 2a: If secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time. Regarding the RAN1 agreement on not supporting simultaneous LR and MR operation, further check with RAN1 whether the agreement is applicable to DC and CA, and whether it has any impact on per CG DRX operation.

Proposal 3: If secondary DRX group is configured, the lpwus-PDCCHMonitoringTimer configuration for secondary DRX group is different from that for the default DRX group.

Proposal 3a: The lpwus-PDCCHMonitoringTimer configuration for secondary DRX group is smaller than that for the default DRX group.

This contribution discusses the concern on Proposal 2/2a raised during the email discussion.

# 2	Discussions

## 2.1	LP-WUS and Dual DRX

RAN2 has discussed two options:

-	Option 1: UE monitors LP-WUS when both DRX groups are not in DRX active time;

-	Option 2: UE monitors LP-WUS when the default DRX group is not in DRX active time and secondary DRX group is in DRX active time.

While most companies agreed option 1- to be aligned with RAN1 agreement in general -, there was a concern on option 1 regarding power saving gain:

For option 1, UE has to wait for non-active time on both DRX groups, UE power saving will be lost if one DRX group has always data transmission and always in active time.

First, the option 1 doesn’t mean that both DRX groups must remain in DRX active time until LP-WUS monitoring starts. If the DRX active time of one DRX group ends earlier, that group stops PDCCH monitoring and waits, while the other DRX group continues PDCCH monitoring until it ends, and LP-WUS monitoring begins. Therefore, no additional power consumption occurs before LP-WUS monitoring starts.

Observation 1: No additional power consumption is expected with option 1.

In addition, the general design principle would be configuring shorter drx-inactivityTimer for the secondary DRX group since the main goal is to handle bursty traffic on high frequency bands and quickly turn off the RF chain when not needed. Therefore, Therefore, even if the active time of the primary DRX group ends earlier than the secondary DRX group, the active time of the secondary DRX group would not be maintained for long either. As a result, the delay in starting LP-WUS monitoring would not be significant.

Observation 2: LP-WUS monitoring would start without significant delay even if the primary DRX group’s active time ends earlier since the secondary DRX group’s active time is expected to be short.

Therefore, we think RAN2 can stick to rapporteur’s proposal 2.

Proposal 1: As proposed in the email discussion – proposal 2 –, if secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time.

## 2.2	Uplink aspect – Scheduling Request

For both option 1-1 and 1-2, it is assumed that the UE does not monitor LP-WUS during C-DRX active time. This raises the question of whether the UE can automatically transition to active time by uplink traffic, such as when SR is pending, or the UE should continue waiting for LP-WUS from the network.

When SR is pending, the network may not yet be aware that the UE has entered C-DRX active time. As a result, the network might send LP-WUS for downlink scheduling, which would be missed if the UE has stopped monitoring of LP-WUS. Only after the gNB receives the SR, will it stop sending LP-WUS and schedule the UE via PDCCH.

Although downlink latency is generally considered non-critical, the potential failure of the initial SR transmission may motivate to maintain LP-WUS monitoring until the gNB successfully receives the SR and responds – the UE starts monitoring of PDCCH based on the new timer started by LP-WUS received SR is pending. It would imply also that the SR is responded by LP-WUS instead of PDCCH, which would need to be clarified in the specification.

Observation 3: If the UE monitors LP-WUS instead of PDCCH while SR is pending, reception of LP-WUS would need to be considered as a response to SR.

Alternatively, the UE could monitor both LP-WUS and PDCCH while the SR is pending; however, the feasibility of this may depend on a decision by RAN1.

Observation 4: Monitoring both LP-WUS and PDCCH simultaneously while SR is pending would need to be confirmed by RAN1.

In our view, the simplest solution is to begin PDCCH monitoring when SR is pending, as is done in legacy, unless there is good reason to keep the UE’s C-DRX active time synchronized with the gNB. Our understanding is that the current MAC running CR is already based on this principle. However, given that the LP-WUS monitoring will be mainly specified in RAN1 specification, there could be an impact to RAN1, e.g., stop LP-WUS monitoring when SR is triggered, based on some interaction.

Proposal 2: RAN2 confirms that the UE stops LP-WUS monitoring when SR is pending, and discuss how/where to capture it, e.g., in RAN1 and/or in RAN2. If needed, send an LS to RAN1.

# 3	Conclusion

We discussed the secondary DRX group aspect and untouched issues for uplink, and proposed the followings:

Dual DRX:

Observation 1: No additional power consumption is expected with option 1.

Observation 2: LP-WUS monitoring would start without significant delay even if the primary DRX group’s active time ends earlier since the secondary DRX group’s active time is expected to be short.

Proposal 1: As proposed in the email discussion – proposal 2 –, if secondary DRX group is configured, UE monitors LP-WUS only when both DRX groups are not in DRX active time.

Scheduling Request:

Observation 3: If the UE monitors LP-WUS instead of PDCCH while SR is pending, reception of LP-WUS would need to be considered as a response to SR.

Observation 4: Monitoring both LP-WUS and PDCCH simultaneously while SR is pending would need to be confirmed by RAN1.

Proposal 2: RAN2 confirms that the UE stops LP-WUS monitoring when SR is pending, and discuss how/where to capture it, e.g., in RAN1 and/or in RAN2. If needed, send an LS to RAN1.