3GPP TSG-RAN WG2 Meeting #131                                       R2-2505593

Bangalore, India, 25 - 29 Aug 2025

Source:        ZTE Corporation

Title:           Discussion on LCM for positioning use case

Agenda item:   8.1.2.3

Document for:  Discussion and Decision

- Introduction

The open issues are given in R2-2505702 [1] . As this is the last Rel-19 RAN2 meeting, this contribution provides our view on the remaining open issues.

- Discussion

LPP-6a: PRU info transfer

| Issue | RAN2 Agreement | Status | Action |
| --- | --- | --- | --- |

| LPP#6a (new) | Number of PRUs in NR-PRU-DL-Info | The IE NR-PRU-DL-Info is also applicable to NR AI/ML positioning Case 1. The corresponding Editor's Notes in clause 6.4.3 can be removed. FFS if more PRUs are needed to be included for training purposes | Open (only the FFS part) | Company contributions (if any) |
| --- | --- | --- | --- | --- |

RAN1 agreed to let LMF transfer/forward PRU’s label/channel measurements to the target UE for training or monitoring, and NR-PRU-DL-Info can be reused for such delivery. Since current NR-PRU-DL-Info is introduced for calibration in CPP positioning, the signaling only contains one PRU’s location and measurement information:

NR-PRU-DL-Info-r18 ::= SEQUENCE { nr-PRU-LocationInfo-r18 LocationCoordinates OPTIONAL, -- Need ON nr-PRU-DL-TDOA-MeasInfo-r18 NR-DL-TDOA-SignalMeasurementInformation-r16 OPTIONAL, -- Need ON nr-PRU-DL-AoD-MeasInfo-r18 NR-DL-AoD-SignalMeasurementInformation-r16 OPTIONAL, -- Need ON nr-PRU-RSCP-MeasInfo-r18 NR-PRU-RSCP-MeasurementInformation-r18 OPTIONAL, -- Need ON ... }

When the IE is reused for AI/ML training or monitoring purpose, only one PRU’s part A/part B information at a time is not enough nor efficient to build a AI data set. If only relying on LMF sends each PRU’s info in separate/different message, it will be hundreds of LPP messages to deliver this information, leads to high system burden.

Also the NR-PRU-DL-Info itself can only be sent in a periodic manner. For AI/ML positioning, this information should be able to send on a one-shot manner, so a new IE to deliver the PRU info in AI/ML is needed.

Therefore, we suggest to create a list in NR-PositionCalculationAssistance containing a list of PRU’s location and/or measurement for AI/ML. Furthermore, UE should be able to request that how many PRU’s information the UE wants to receive in a single provideAssistanceData message. Since the number depends on UE’s model capability, training status, etc, it should be a UE’s decision and NW should know this number to better match the UE’s requirement.

The example TP is given below:

- – NR-PositionCalculationAssistance The IE NR-PositionCalculationAssistance is used by the location server to provide assistance data including integrity information to enable UE-based downlink positioning. -- ASN1START NR-PositionCalculationAssistance-r16 ::= SEQUENCE { nr-TRP-LocationInfo-r16 NR-TRP-LocationInfo-r16 OPTIONAL, -- Need ON nr-DL-PRS-BeamInfo-r16 NR-DL-PRS-BeamInfo-r16 OPTIONAL, -- Need ON nr-RTD-Info-r16 NR-RTD-Info-r16 OPTIONAL, -- Need ON ..., [[ nr-TRP-BeamAntennaInfo-r17 NR-TRP-BeamAntennaInfo-r17 OPTIONAL, -- Need ON nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17 NR-DL-PRS-ExpectedLOS-NLOS-Assistance-r17 OPTIONAL, -- Need ON nr-DL-PRS-TRP-TEG-Info-r17 NR-DL-PRS-TRP-TEG-Info-r17 OPTIONAL -- Need ON ]], [[ nr-IntegrityServiceParameters-r18 NR-IntegrityServiceParameters-r18 OPTIONAL, -- Need OR nr-IntegrityServiceAlert-r18 NR-IntegrityServiceAlert-r18 OPTIONAL, -- Need OR nr-IntegrityRiskParameters-r18 NR-IntegrityRiskParameters-r18 OPTIONAL, -- Need OR nr-IntegrityParametersTRP-LocationInfo-r18 NR-IntegrityParametersTRP-LocationInfo-r18 OPTIONAL, -- Cond Integrity1 nr-IntegrityParametersDL-PRS-BeamInfo-r18 NR-IntegrityParametersDL-PRS-BeamInfo-r18 OPTIONAL, -- Cond Integrity2 nr-IntegrityParametersRTD-Info-r18 NR-IntegrityParametersRTD-Info-r18 OPTIONAL, -- Cond Integrity3 nr-IntegrityParametersTRP-BeamAntennaInfo-r18 NR-IntegrityParametersTRP-BeamAntennaInfo-r18 OPTIONAL, -- Cond Integrity4 nr-PRU-DL-Info-r18 NR-PRU-DL-Info-r18 OPTIONAL -- Need ON ]], [[ nr-TRP-LocationInfo-Implicit-r19 NR-TRP-LocationInfo-Implicit-r19 OPTIONAL -- Need ON nr-PRU-DL-InfoList-r19 ::= SEQUENCE (SIZE(1..nrMaxPRUs-r19)) OF NR-PRU-DL-Info-r18 OPTIONAL -- Need ON ]] } - – NR-DL-AIML-RequestAssistanceData The IE NR-DL-AIML-RequestAssistanceData is used by the target device to request assistance data from a location server for NR DL AI/ML positioning. -- ASN1START NR-DL-AIML-RequestAssistanceData-r19 ::= SEQUENCE { nr-PhysCellID-r19 NR-PhysCellID-r16 OPTIONAL, nr-DL-PRS-AssistanceDataReq-r19 ENUMERATED { requested } OPTIONAL, nr-DL-PRS-ExpectedAoD-or-AoA-Req-r19 ENUMERATED { eAoD, eAoA } OPTIONAL, nr-on-demand-DL-PRS-Req-r19 NR-On-Demand-DL-PRS-Request-r17 OPTIONAL, pre-configured-AssistanceDataReq-r19 ENUMERATED { true } OPTIONAL, nr-PositionCalculationAssistanceReq-r19 BIT STRING { trpLoc (0), beamInfo (1), rtdInfo (2), losNlosInfo (3), trpTEG-Info (4), nr-IntegrityBounds (5), pruInfo (6), trpLoc-Implicit (7) } (SIZE (1..8)) OPTIONAL, nr-IntegrityAssistanceReq-r19 BIT STRING { serviceParameters (0), serviceAlert (1), riskParameters (2), integrityParaTRP-Loc (3), integrityParaBeamInfo (4), integrityParaRTD-Info (5) } (SIZE (1..8)) OPTIONAL, ..., [[ nr-PRU-InfoListReq-r19 INTEGER (1..nrMaxPRUs-r19) OPTIONAL ]] } -- ASN1STOP - 6.6 Multiplicity and type constraint values - – Multiplicity and type constraint definitions -- ASN1START … nrMaxPRUs-r19 INTEGER ::= XX -- maximum number of PRUs which provides location and/or measurements. Value TBD

Proposal 1: Support LMF to send UE a list of multiple PRU info (i.e., PRU’s location and/or measurement) in a single message. Adopt the TP as baseline.

Proposal 2: Support UE to request LMF that how many PRU’s info the UE wants for AI/ML. Adopt the TP as baseline.

LPP-10a: Applicability of "batch reporting" for AI/ML positioning

Batch reporting is introduced to let UE report multiple measurements and/or multiple location estimates in a single ProvideLocationInformation signaling to LMF. The multiple location estimates are historical, considering currently LMF only forwards the latest PRU/UE’s part A/part B information to the target UE for training/monitoring purpose, it is not useful to let PRU/UE perform batch reporting associated with historical timestamps. Further, this is not an essential function when the target UE reports inference results, since periodical reporting can be used to achieve same effect but with more timely AI/ML location estimate provision.

Proposal 3: Do not support batch reporting in AI/ML positioning method.

LPP-11: Content of PositioningRequestLocationInformation

| LPP#11 | Details of IE NR-AI-ML-PositioningRequestLocationInformation | The IE NR-AI-ML-PositioningRequestLocationInformation contains (at least) the AssistanceAvailability flag. Additional details/information can be discussed via company contributions. | Open | Company contributions (if any) |
| --- | --- | --- | --- | --- |

Current NR-DL-AIML-ProvideLocationInformation does not contain additional content except for measurement time, and RAN1 does not provide any additional information that UE should report in case 1. So, both NR-DL-AIML-RequestLocationInformation and NR-DL-AIML-ProvideLocationInformation does not need any additional information.

Proposal 4: Both NR-DL-AIML-RequestLocationInformation and NR-DL-AIML-ProvideLocationInformation do not need any additional information.

LPP-14: Error cause in the new positioning method

In current LPP specification, the Error message is to indicate an error occurs at LMF or UE side during an on-going positioning method. For this new AI/ML positioning method, the Error message can be reused with some new error cause indication dedicated for AI/ML positioning use cases. RAN2#128 has already agreed to use Error message when the AIML based positioning method becomes non-applicable.

Since the new AI/ML method are applicable to all the LCM procedures, the target device error cause should be able to indicate the error occurred during training/inference/monitoring. If LMF knows the information, the LMF is able to provide more information to UE adaptively.

We propose to have the following target device error causes, in which the green one is already agreed and the blue ones should be further supported:

Error cause during training:

UE does not get enough training data to train an AI/ML model; -----LMF will be able to send more training data to UE to facilitate training

UE cannot download or acquire an AI/ML model now; ----- LMF will be able to wait a while then schedule the AI/ML method, or LMF will be able to change the method;

Error cause during inference:

AI/ML positioning is not available; ----- LMF will be able to change the method;

UE cannot generate an estimate location using an AI/ML model; ----- LMF will be able to change the method;

Error cause during performance monitoring:

AI/ML positioning is not available; ----- LMF will be able to change the method;

UE cannot generate a performance monitoring metric. ----- LMF will be able to abort the AI/ML method due to lack of monitoring outcome.

Proposal 5: Introduce the following additional target device error cause in AI/ML positioning, the error cause are indicated with respect to each LCM procedure (training/inference/monitoring):

- Error cause during training:

  - UE does not get enough training data to train a AI/ML model;

  - UE cannot download or acquire or train a AI/ML model;

- Error cause during inference:

  - UE cannot generate an estimate location using a AI/ML model;

- Error cause during performance monitoring:

  - UE cannot generate a performance monitoring metric.

LPP-18: Consistency between training and inference

RAN1#121 has agreed to use associated ID to provide TRP location implicitly:

Working Assumption For AI/ML based positioning Case 1, regarding info #7 in the assistance information from legacy UE-based DL-TDOA, it can be provided as in legacy UE-based DL-TDOA or implicitly. Agreement For AI/ML based positioning Case 1, regarding Info #7 in the assistance information from legacy UE-based DL-TDOA, - If implicitly provided, the implicit indication of Info #7 is via associated ID. - For given TRP(s), same associated ID implies that geographical coordinates of the TRP(s) can be understood as consistent by the UE. - The associated ID is not expected to provide the real value of Info #7 (i.e., geographical coordinates of the TRP(s) are not disclosed). - an associated ID is configured per-cell (e.g., NCGI-r15) - UE does not expect to receive different values of associated ID for TRPs belonging to the same NCGI-r15 - Associated ID can be realized by an identifier of N bits (e.g., 8 bits)

However, the associated ID is only used to implicitly represent the TRP location, not to represent all the TRP attributes that needs to be consistent between training and inference. For example, even if the location of TRPs in training and inference remain unchanged, the TRP sync error or TRP beam direction may change. This also has impact on the UE to maintain the consistency between training and inference. The UE with bad generalization performance cannot ensure the consistency if those TRP attributes have a change. So only relying on the agreed associated ID is not enough to ensure consistency between training and inference.

Observation 1: Only relying on the associated ID is not enough to ensure consistency between training and inference.

There is a discussion on how to ensure the consistency between training and inference. The basic assumption is that, each AI model that UE trains should be under some certain condition, and the trained AI model should also be used for inference under the same or similar condition to achieve higher accuracy. From specification perspective, this means the set of attribute/parameter/condition of the training data input of an AI model should be same or similar as the set of attribute/parameter/condition of the inference data input.

For AI/ML positioning usecase 1, UE receives and measures TRP’s PRS for training and inference, so the set of attribute/parameter/condition of the training data input or inference data input can include NW additional conditions such as TRP location, TRP synchronization error, TRP number, PRS configuration, etc.

LMF is the node to provide inference configuration to UE, UE should strictly follow NW’s configuration to perform inference. But LMF does not know which kind of inference configuration best suits the training configuration using which UE trained its AI model, e.g., how many number of TRPs used during training, which area the used TRPs locate on, the synchronization error range that the used TRP belongs to, etc. Therefore, UE should be able to request some specific TRP attributes/NW side additional conditions (for inference) that matches UE’s current well-trained AI model to ensure the training-inference consistency. The specific request can include the wanted number of TRPs, the wanted area of TRPs, and the wanted synchronization error between TRPs for inference.

Since different AI model has different generalization and it is only known by UE, so only the UE knows how much difference the training and inference owns that will not have big impact to the inference performance, and to ensure the QoS requirement. The UE is able to ensure the requested value of TRP attribute to be consistent with the value used during training, and the UE ensures such difference can be covered by the good generalization performance.

The above request can reuse the current on-demand PRS request or RequestAssistanceData procedure.

Observation 2: When providing inference configuration to UE, LMF does not know which kind of inference configuration best suits the training configuration that UE used to train an AI model.

Proposal 6: In order to ensure consistency between training and inference, UE should be able to request some specific assistance data for inference which suits the UE’s current trained AI model:

- UE can request the wanted number of TRPs/cells for inference;

- UE can request the wanted area of TRPs/cells for inference;

- UE can request the wanted synchronization error range between TRPs for inference.

LPP-19: Applicability of BM related agreements

- Open issue LPP-19 (new): Applicability of BM related agreements Issue description: As mentioned in [POST130][023][AI PHY] 38.305 CR (CATT), since the RAN2 intention is to have the same design for BM case and positioning case as much as possible, whether the agreements made for BM applicability reporting can also be applicable to positioning case 1. Whether the following agreements made for BM applicability reporting are also applicable to positioning case 1 and whether there are any associated LPP impacts: - UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. - Support the explicit reporting of applicability/inapplicability in initial report and subsequent reporting it reports only applicability it changed. Proposed resolution: Companies to provide contributions to the following meeting.

Based on the previous agreement, the applicable functionalities are reported via unsolicited ProvideCapabilities upon applicability change internally, or upon applicability change based on the change of NW configurations.

RAN2#128 agreement: For POS Case 1, RAN2 confirm that the existing unsolicited UE capability report mechanism in LPP can support UE to report the applicable functionality in both “proactive” and “reactive” as a baseline. - Proactive case: When the applicability change, UE can send an unsolicited LPP ProvideCapabilities message to LMF . - Reactive case: If the applicability changes based on the configuration in LPP ProvideAssistanceData message in step 3, UE can send an unsolicited LPP ProvideCapabilities message to LMF. Configuration details are FFS RAN2#129 agreement: UE reports the applicable functionality to the LMF by the LPP provide capabilities message if there is a change of applicable functionality. FFS if any additional LMF control is needed.

So we suggest to capture the above RAN2 agreement of applicable functionalities reporting in stage-2 spec.

From our point of view, the BW-related agreement that how UE decides the applicable functionalities (by NW side additional conditions, UE side additional conditions and model availability) is technically correct also for AI/ML positioning, but it does not have stage-3 spec impact.

Additionally, if the above understanding is to be captured in stage-2 spec, it should state that NW side additional condition in AI/ML positioning means the NR-DL-AIML-ProvideAssistanceData provided by LMF.

Proposal 7: Capture the RAN2 agreement on applicable functionalities reporting in stage-2 spec, and to state that NW-side additional condition in AI/ML positioning means the NR-DL-AIML-ProvideAssistanceData provided by LMF.

LPP-20: LPP Impacts related to Case 3a/3b

The open issue is added via the following comments:

| Samsung | new | According to the following agreement in RAN1, a training data sample contains both Part A (channel measurement) and Part B (ground truth location). Also, the training data can be for a same UE which can be either PRU or Non-PRU UE. RAN1 agreement: For training data collection of AI/ML based positioning, if a training data sample contains both Part A and Part B, RAN1 assumes that Part A and Part B in one training data sample are: - for a same UE (PRU or Non-PRU UE), and - for a same location associated with Part B. Meanwhile, with the current LPP, the LMF can request PRU to report both location estimate and measurement by setting the LocationInformationType field as ‘locationEstimateAndMeasurementsRequired’, which can be used for training data collection by LMF. However, the ‘locationEstimateAndMeasurementsRequired’ applies only to PRU UE now and thus there’s limitation on the training data collection for Non-PRU UE. New Open issue: whether/how the LMF can request Non-PRU UE to report both location estimate and measurement for training data collection. | I added a general Issue LPP-20 on possible LPP impacts related to Case 3a/3b (has also been mentioned by Ericsson in 'R2-25xxxxx_([POST130][025][AI PHY] LPP Running CR Discussion) '. On the cited agreement, the Part A/Part B terminology is for Case (3a)/3b (and initially for Case (2a)/2b, which however, are not supported in this Release). For Case 1, there is no "Part A" (i.e., model input is up to implementation). RAN1#116bis: Part A: - channel measurement - quality indicator of channel measurement - time stamp of channel measurement Part B: - ground truth label (or its approximation) - quality indicator of label - time stamp of label |
| --- | --- | --- | --- |

LMF can request non-PRU UE to report location estimate (part B) and report measurement (part A) via two independent messages. As long as the part A and the part B have same or similar/near timestamp, LMF can pair the part A and part B of the same non-PRU UE. So it seems not necessary to introduce a new LocationInformationType for non-PRU UE.

Besides, we do not observe other additional LPP spec impact on case 3a/3b.

Proposal 8: It is not necessary to introduce a new LocationInformationType for non-PRU UE, since LMF can pair the part A and part B of the same or near time stamp of the same non-PRU UE.

Proposal 9: There is no additional LPP spec impact on case 3a/3b.

LPP-21: "Associated ID" for TRP Location Coordinates

- Open issue LPP-21 (new): "Associated ID" for TRP Location Coordinates (IE TRP-ImplicitLocationInfo-r19) Issue description: The following issues have been discovered during LPP running CR review: - Must the "Associated ID" for the TRP coordinates be associated with a "cell" (e.g., NCGI)? If so, how should PRS-only TPs (which are not associated with a cell) being handled? And how to implement RAN1 agreement that UE does not expect to receive different values of associated ID for TRPs belonging to the same cell? - Is the "Associated ID" only for the TRP location coordinates, or also for the associated DL-PRS Resource Set/DL-PRS Resource ARPs? - Is the value range of 0..255 for the "Associated ID" sufficient? In particular if the "Associated ID" should be "configured per cell"? - Are the explicit TRP coordinates (NR-TRP-LocationInfo-r16) and implicit TRP coordinates (NR-TRP-ImplicitLocationInfo-r19) mutually exclusive, or can a deployment provide both? - If a UE supports explicit TRP location info (NR-TRP-LocationInfo-r16), must a UE support also implicit location info (nr-TRP-ImplicitLocationInfo-r19) (and vice versa)? Proposed resolution: Wait for more RAN1 input and/or companies to provide contributions to the following meeting.

Since the associated ID is agreed by RAN1 in their last meeting without further detailed explanation, it causes confusion in RAN2. However RAN1 has finished Rel-19 and further work should be LS-triggered. So we suggest to send LS to ask the above identified questions to RAN1. This will prevent RAN2 from endless debate.

Proposal 10: Send LS to RAN1 asking the detailed explanation of the associated ID.

Other issue: Training data collection request

For usecase 1, UE needs to get training data set for training. The training data set can be UE’s own channel PRS measurement (with the label of UE’s own location), or can be other PRU’s PRS channel measurement (with the label of other PRU’s location/intermediate feature).

To acquire the UE’s own PRS channel measurement and the UE’s own location, UE needs to get PRS configuration and/or TRP information in advance. So, the LMF should be able to provide PRS configuration and/or TRP information to a UE (i.e., green arrows in the figure 1).

LMF should also be able to provide other UE/PRU’s PRS channel measurement with PRU/UE location/intermediate feature to a UE, for AI/ML training or monitoring (see green arrows in the figure 1).

Figure 1. LMF provides PRS configuration, TRP information and other UE/PRU’s PRS channel measurement with PRU/UE location to target UE for AI training

The provision of such assistance data for AI/ML training can be embedded in ProvideAssistanceData message. And the request of such assistance data for AI/ML training can be embedded in RequestAssistanceData message.

Proposal 11: For usecase 1, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

Different training data set associated with different TRP information will have different impact on UE AI model’s performance, for example, the model trained using DL-PRS measurement results from 20 TRPs will be more accurate than using DL-PRS measurement results from 3 TRPs. To suit the current UE’s condition or given QoS requirement for AI/ML positioning, UE can request some specific assistance data that will have significant impact on model training, e.g.:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

LMF may also provide other PRU’s measurement and PRU label to target UE for training. So UE is better to request the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

Proposal 12: For case 1, UE can request some specific assistance data for training:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

- UE requests the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

- Conclusion

In this contribution, we propose the following observation and proposals:

LPP-6a: PRU info transfer

Proposal 1: Support LMF to send UE a list of multiple PRU info (i.e., PRU’s location and/or measurement) in a single message. Adopt the TP as baseline.

Proposal 2: Support UE to request LMF that how many PRU’s info the UE wants for AI/ML. Adopt the TP as baseline.

LPP-10a: Applicability of "batch reporting" for AI/ML positioning

Proposal 3: Do not support batch reporting in AI/ML positioning method.

LPP-11: Content of PositioningRequestLocationInformation

Proposal 4: Both NR-DL-AIML-RequestLocationInformation and NR-DL-AIML-ProvideLocationInformation do not need any additional information.

LPP-14: Error cause in the new positioning method

Proposal 5: Introduce the following additional target device error cause in AI/ML positioning, the error cause are indicated with respect to each LCM procedure (training/inference/monitoring):

- Error cause during training:

  - UE does not get enough training data to train a AI/ML model;

  - UE cannot download or acquire or train a AI/ML model;

- Error cause during inference:

  - UE cannot generate an estimate location using a AI/ML model;

- Error cause during performance monitoring:

  - UE cannot generate a performance monitoring metric.

LPP-18: Consistency between training and inference

Observation 1: Only relying on the associated ID is not enough to ensure consistency between training and inference.

Observation 2: When providing inference configuration to UE, LMF does not know which kind of inference configuration best suits the training configuration that UE used to train an AI model.

Proposal 6: In order to ensure consistency between training and inference, UE should be able to request some specific assistance data for inference which suits the UE’s current trained AI model:

- UE can request the wanted number of TRPs/cells for inference;

- UE can request the wanted area of TRPs/cells for inference;

- UE can request the wanted synchronization error range between TRPs for inference.

LPP-19: Applicability of BM related agreements

Proposal 7: Capture the RAN2 agreement on applicable functionalities reporting in stage-2 spec, and to state that NW-side additional condition in AI/ML positioning means the NR-DL-AIML-ProvideAssistanceData provided by LMF.

LPP-20: LPP Impacts related to Case 3a/3b

Proposal 8: It is not necessary to introduce a new LocationInformationType for non-PRU UE, since LMF can pair the part A and part B of the same or near time stamp of the same non-PRU UE.

Proposal 9: There is no additional LPP spec impact on case 3a/3b.

LPP-21: "Associated ID" for TRP Location Coordinates

Proposal 10: Send LS to RAN1 asking the detailed explanation of the associated ID.

Other issue: Training data collection request

Proposal 11: For usecase 1, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

Proposal 12: For case 1, UE can request some specific assistance data for training:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

- UE requests the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

- Reference

- R2-2505702, LPP open issues for feature "AI/ML for NR air interface", Qualcomm Incorporated (Rapporteur), 2025-08.