3GPP TSG-RAN2#131										                             R2-2505601

Bangalore, India, August 25th – 29th, 2025

Source: 			 ZTE Corporation, Sanechips

Title: 	Response to SA4 LS on the RAN simulation assumptions for ULBC

Agenda item:         8.20.2

Document for:       Discussion and Decision

- Introduction

Before RAN2#131, an LS on the RAN simulation assumptions for ULBC[1] has been provided from SA4 which is related to support IMS-based voice in NB-IoT NTN. The LS includes some questions to RAN2 as follows:

# To SA2 and RAN2: ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic.

In this contribution, we will discuss these questions and give our response.

- Discussion

## 2.1 Whether a packet overhead of 1 byte of MAC header is realistic

According to the TS 36.321 spec [2], the two types of minimal MAC subheader are defined as follows:

Figure 6.1.2-1: R/F2/E/LCID/F/L MAC subheader with 7-bits and 15-bits L field Figure 6.1.2-2: R/F2/E/LCID MAC subheader

Considering that “L” field is necessary for indicating the length of the following MAC SDU that encapsulates the voice packet, and the “R” field has already be used for QCDR (Downlink Channel Quality Report) function as follows, it is impossible to define a 1 byte MAC subheader for voice packet. So the 2 bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission.

- R: Except when this field is used for short DCQR, reserved bit, set to "0". For short DCQR, the mapping of R field to short DCQR value is described in table 6.2.1-5.

Observation 1: The “R” field in MAC subheader has already be used for DCQR function and the “L” field is necessary for indicating the length of voice MAC SDU, so it is impossible to define only a 1 byte MAC subheader for voice packet. Two bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission.

## 2.2 The header size of different protocol layers

Based on the current specifications, we list the minimum protocol header size that will be required for the different options listed in SA4 LS and transmitted over Uu as follows:

Table 1. Minimum header size of different protocol layers

| Transmission solutions Minimum header size for each layer Protocol layers |  | ## CP(CIoT EPS Control Plane optimization) | ## UP(CIoT EPS User Plane optimization) |
| --- | --- | --- | --- |
| AS layer | ## RRC | ## UL: 3 bytes (16bits UL DCCH MessageType and others) ## DL: 2 bytes (8bits DL DCCH MessageType and others) ## So, taking 2bytes as minimum value. | ## 0 byte |
|  | ## PDCP | ## 0 byte | ## 1 byte |
|  | ## RLC | ## UM: 1 byte ## AM: 2 bytes | ## UM: 1 byte ## AM: 2 bytes |
|  | ## MAC | ## 2 bytes (see analysis in section 2.1) | ## 2 bytes |
| NAS layer |  | ## 7 bytes (with NAS security) ## (note: CT1 still work in R19 to reduce NAS layer overhead) | ## 0 byte |
| Higher layers for Non-IP (NIDD) based option | ## IP | ## 0 byte | ## 0 byte |
|  | ## UDP | ## 0 byte | ## 0 byte |
|  | ## RTP | ## 12 bytes | ## 12 bytes |
| Higher layers for IP based option without RoHC | ## IP | ## 20 bytes | ## 20 bytes |
|  | ## UDP | ## 8 bytes | ## 8 bytes |
|  | ## RTP | ## 12 bytes | ## 12 bytes |
| Higher layers for IP based option with RoHC |  | ## 1 bytes (UO-0 mode) or ## 3 bytes (UOR-2 mode) | ## 1 bytes (UO-0 mode) or ## 3 bytes (UOR-2 mode) |

Based on this table, we have the following observation:

Observation 2: Based on the current specifications and under the same other conditions:

- The header size of UP solution is at least 8bytes less than that of CP solution;

- The header size of IP based option with RoHC is at least 9bytes or even 11bytes less than that of the Non-IP based option.

## 2.3 Potential AS layer optimizations to support voice over NB-IoT NTN and preferred option

There are two service flows for the IMS voice: IMS signalling flow and Voice flow, which have different QoS requirements and usually use different RLC transmission mode, e.g., IMS signalling usually needs to use RLC-AM mode to ensure reliable delivery of SIP signaling and successful session establishment, while voice flow usually needs to use RLC-UM mode to reduce transmission latency of the voice packets. Therefore, from RAN2 perspective, the following optimizations may be necessary for CP solution or UP solution respectively for efficiently support IMS-based voice in NB-IoT NTN:

|  | ## CP(CIoT EPS Control Plane optimization) | ## UP(CIoT EPS User Plane optimization) |
| --- | --- | --- |
| QoS parameters | There is only one QoS flow for CP in the current specification. At least another QoS flow should be introduced for CP to support Voice. | Multiple QoS flows can already be supported for UP in the current specification. No change is necessary. |
| RRC reconfiguration | Not supported, but necessary for the case that voice preempt the on-going data transmission. RRC reconfiguration should be retook for CP. | RRC reconfiguration is already supported for UP. No change is necessary. |
| RB | There is only one SRB with AM mode in NB-IoT, it cannot provide QoS differentiation between Voice and IMS signalling. New SRB with UM mode should be supported for CP. | Two DRBs with configurable RLC mode can already be supported for UP. No change is necessary. |
| RRC signalling | Only one RLC-AM mode DL RRC signalling(DLInformationTransfer-NB) and one RLC-AM mode UL RRC signalling(ULInformationTransfer-NB) can be used for data transmission in CP. New RRC DL/UL signalling with UM mode should be introduced for CP. | No RRC signalling is necessary |
| RoHC | IP/UDP/RTP RoHC profile (e.g., profile 0x0001 and 0x0101) should be introduced for Header compression configuration for CP solution by SA2. | IP/UDP/RTP RoHC profile (e.g., profile 0x0001 and 0x0101) should be introduced for PDCP configuration for UP solution by RAN2. |

From above table, it can be seen that, since the UP solution supports relatively rich bearer types and relatively comprehensive QoS guarantees, supporting IMS voice over UP solution impacts AS layer specifications very little, meanwhile supporting IMS voice over CP solution impacts AS layer specifications very much.

Observation 3: Based on the current specifications, supporting IMS voice over CP solution impacts AS layer specifications much more than that over UP solution.

Based on the Observation 2 and Observation 3, UP solution is preferred for support IMS-based voice in NB-IoT NTN.

Proposal 1: To use UP solution for supporting voice service over NB-IoT NTN.

Based on the Observations and proposal above, RAN2 can conclude and provide reply LS to SA4 as following:

Proposal 2: RAN2 sends reply LS to the SA4 LS, indicating that:

- A packet overhead of 1 byte of MAC subheader for voice transmission is not realistic. Two bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission.

- UP solution is more suitable to be used for supporting voice than CP solution because of its less protocol header size and less specification impacts.

- IP solution with RoHC is more suitable to be used for supporting voice than Non-IP solution because of its less protocol header size, unless new header compression mechanism is introduced, which may need much evaluation and work in other working groups.

- To include the header size evaluation table as reference.

Based on the Proposal 2, the draft reply LS from RAN2 is provided in the appendix.

- Conclusion

The following observations and proposals are made:

Observation 1: The “R” field in MAC subheader has already be used for DCQR function and the “L” field is necessary for indicating the length of voice MAC SDU, so it is impossible to define only a 1 byte MAC subheader for voice packet. Two bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission.

Observation 2: Based on the current specifications and under the same other conditions:

- The header size of UP solution is at least 8bytes less than that of CP solution;

- The header size of IP based option with RoHC is at least 9bytes or even 11bytes less than that of the Non-IP based option.

Observation 3: Based on the current specifications, supporting IMS voice over CP solution impacts AS layer specifications much more than that over UP solution.

Proposal 1: To use UP solution for supporting voice service over NB-IoT NTN.

Proposal 2: RAN2 sends reply LS to the SA4 LS, indicating that:

- A packet overhead of 1 byte of MAC subheader for voice transmission is not realistic. Two bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission.

- UP solution is more suitable to be used for supporting voice than CP solution because of its less protocol header size and less specification impacts.

- IP solution with RoHC is more suitable to be used for supporting voice than Non-IP solution because of its less protocol header size, unless new header compression mechanism is introduced, which may need much evaluation and work in other working groups.

- To include the header size evaluation table as reference.

Based on the Proposal 2, the draft Reply LS to SA4 LS is provided in the appendix.

- References

- R2-2505068_S4-251584 LS on the RAN simulation assumptions for ULBC, RAN2#131, August, 2025

- TS 36.321, V18.4.0 (2025-03), Evolved Universal Terrestrial Radio Access (E-UTRA); Medium Access Control (MAC) protocol specification(Release 18)

- TS 36.322, V18.0.0 (2024-03), Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Link Control (RLC) protocol specification(Release 18)

- TS 36.323, V18.0.0 (2024-03), Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification(Release 18)

- TS 36.331, V18.6.0 (2025-06), Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification(Release 18)

- Appendix: Draft reply LS

The following is draft reply LS to the SA4 LS (R2-2505068_S4-251584) on the RAN simulation assumptions for ULBC.

# 1 Overall description RAN2 thanks SA4 for the LS on the RAN simulation assumptions for ULBC, RAN2 can provide the following answers to the questions raised for RAN2/SA2. ACTION: SA4 kindly asks SA2 and RAN2 to comment on - the different options among UP/CP and IP/Non-IP and the respective overall packet overhead (including RTP/UDP/IP with RoHC, PDCP, RLC and MAC and any potential AS layer optimization if applicable), and if there is any preferred option, and - specifically, whether a packet overhead of 1 byte of MAC header is realistic. Answer: RAN2 discuss the related issue and made the following agreements: - A packet overhead of 1 byte of MAC subheader for voice transmission is not realistic. Two bytes MAC subheader with 7-bits L field is the minimum MAC header length that can be used for voice transmission. - UP solution is more suitable to be used for supporting voice than CP solution because of its less protocol header size and less specification impacts. - IP solution with RoHC is more suitable to be used for supporting voice than Non-IP solution because of its less protocol header size, unless new header compression mechanism is introduced, which may need much evaluation and work in other working groups. The following table on minimum header size of different protocol layers is provided for reference: Transmission solutions Minimum header size for each layer ## Protocol layers; ## CP(CIoT EPS Control Plane optimization); ## UP(CIoT EPS User Plane optimization). AS layer; ## RRC; ## UL: 3 bytes (16bits UL DCCH MessageType and others) ## DL: 2 bytes (8bits DL DCCH MessageType and others) ## So, taking 2bytes as minimum value.; ## 0 byte. ## PDCP; ## 0 byte; ## 1 byte. ## RLC; ## UM: 1 byte ## AM: 2 bytes; ## UM: 1 byte ## AM: 2 bytes. ## MAC; ## 2 bytes (see analysis in section 2.1); ## 2 bytes. NAS layer; ## 7 bytes (with NAS security) ## (note: CT1 still work in R19 to reduce NAS layer overhead); ## 0 byte. Higher layers for Non-IP (NIDD) based option; ## IP; ## 0 byte; ## 0 byte. ## UDP; ## 0 byte; ## 0 byte. ## RTP; ## 12 bytes; ## 12 bytes. Higher layers for IP based option without RoHC; ## IP; ## 20 bytes; ## 20 bytes. ## UDP; ## 8 bytes; ## 8 bytes. ## RTP; ## 12 bytes; ## 12 bytes. Higher layers for IP based option with RoHC; ## 1 bytes (UO-0 mode) or ## 3 bytes (UOR-2 mode); ## 1 bytes (UO-0 mode) or ## 3 bytes (UOR-2 mode). # 2 Actions To SA4: RAN2 respectfully asks SA4 to take the above information into account in related works.