3GPP TSG RAN WG2 #131                                   R2-2505613

Bengaluru, India, August 25th - 29th, 2025

Source:	Sharp

Title:	Remaining issues on A-IoT paging

Agenda Item:	8.2.2

Document for:	Discussion and Decision

Introduction

In this document, we share our views on a few remaining issues of A-IoT paging.

Discussion

(Issue 1-2) Transaction ID

In [3], Issue 1-2 is described as follows

Whether/how to specify how the reader generate Transaction ID, and the size - Relevant agreements: - The “transaction ID” can be generated by reader based on CN corelation ID. FFS how reader will generate “transaction ID”. FFS the size of transaction ID - 1 bit solution is excluded. FFS the size. Aim to have a reasonable size - Status in running CR: captured as Editor’s Note in 6.2.1.1.

And the following proposal was provided in [3] after the email discussion “[POST130][027][AIoT] MAC Running CR (Huawei)”,

### Proposal 1-2 (Issue 1-2 Transaction ID): RAN2 to make quick decision on 4 bits (15/25) or 6 bits (12/25) of Transaction ID length.

Transaction ID is not only used to identify a service from the CN, but also used to distinguish between paging messages from different Readers in the “overlapping reader” scenario. In our view, the decision on Transaction ID length should take into account the fact that UE Readers will be supported in Rel-20 where there will be no “coordination between UEs operating as UE readers” (see [2]). Unlike gNB Readers, UE Readers are more likely to change positions over time, resulting in potentially more severe “overlapping reader” problem. Therefore, we propose to adopt a relatively more conservative option of 6 bits.

- (Issue 1-2) Transaction ID length is 6 bits.

(Issue 4-4) MAC spec implementation

In [3], Issue 4-4 is described as follows,

For some easy FFS (e.g., how to implement agreement in spec), the Rapp took the liberty to propose some implementation resolution, and invite companies to check and review in the running CR. - AS ID release: FFS for CFRA - Msg2 retransmission: How to capture device behavior is FFS - Segmentation: This implies that the R2D message will either have command or offset (but not both).FFS whether we define two message types or one message type with optional fields. - NACK: FFS how to specify. - Paging message format: FFS if more than one R bit is required. - Access occasion number: value range FFS.

Regarding the number of access occasions indicated in an A-IoT paging message, the following has been captured in the latest running CR to TS 38.391 [4],

#### 6.2.1.1 A-IoT Paging message Figure 6.2.1.1-1 and 6.2.1.1-2 show the formats of the A-IoT Paging message. [...] For CBRA, the following fields are further included: - [...] - Number of Access Occasions: This field indicates the number of access occasions. The length of the field is 4 bits. The value 0 (i.e., 0000) indicates the number of access occasions is 20. The value 1 (i.e., 0001) indicates the number of access occasions is 21. The value 2 (i.e., 0010) indicates the number of access occasions is 22. And so on. The maximum number of access occasions is 215 when this field is set to 15 (i.e., 1111).

And the use of the indicated number of access occasions has been captured in clause 5.3.1.1 of [4] as follows,

#### 5.3.1.1 Selection of access occasion for D2R transmission of Access Random ID message […] The A-IoT MAC entity shall: 1> apply the D2R Scheduling Info received from the A-IoT Paging message; 1> generate a random number 'i' in the range: 0 ≤ i ≤ n-1, where n is the number of access occasions configured in A-IoT Paging message; The A-IoT MAC entity should: 1> set the ACCESS_OCCASION_COUNTER to 'i'; 1> if ACCESS_OCCASION_COUNTER < m, where m equals to X*NSFS (where X and NSFS are defined in clause 6.2.1.6):

During the email discussion “[POST130][027][AIoT] MAC Running CR (Huawei)”, it was correctly pointed out by some companies that there is a mismatch between the value indicated by the “Number of Access Occasions” field (i.e. n) in an A-IoT paging message and the actual total number of available access occasions (e.g. denoted by navail here, where navail is an integer multiple of X⋅NSFS, and X and NSFS are respectively the number of time resources and the number of frequency resources triggered by the A-IoT paging message or each following Access Trigger message).

For example, suppose X=1 and NSFS=6, the Reader may choose to set the “Number of Access Occasions” field in an A-IoT paging message to 3, and then transmit a total number of one Access Trigger message in the paging round, resulting in navail=2⋅6=12 available access occasions, and n=23=8 indicated access occasions. Due to the mismatch between navail and n, there are navail−n=12−8=4 access occasions which will never be selected by any device and will thus be always wasted. When a large number of Access Trigger messages are to be transmitted, the number of wasted access occasions may be considered insignificant. However, this number may become quite considerable in cases where no or only one Access Trigger message is to be transmitted.

In our understanding, the root cause of the above-mentioned issue is the incorrectly specified value of n supposed to be indicated by the “Number of Access Occasions” field. The issue can be resolved (based on current RAN2 agreements) for the above example by correcting the value of n to 12, and more generally for clause 6.2.1.1 of TS 38.391 by correcting the value n indicated by the “Number of Access Occasions” field (e.g. with field value v∈0, 1, …, 15) to a minimum integer number of times of X⋅NSFS satisfying n≥2v. We provide a TP for TS 38.391 in section 5.1 of this document as an example of such a correction.

- (Issue 4-4) Adopt TP#1 in section 5.1 of this document to correct the values indicated by the “Number of Access Occasions” field in A-IoT paging message in clause 6.2.1.1 of TS 38.391.

Conclusion

In this contribution, we discuss a few remaining issues of A-IoT paging, and make the following proposals:

- (Issue 1-2) Transaction ID length is 6 bits.

- (Issue 4-4) Adopt TP#1 in section 5.1 of this document to correct the values indicated by the “Number of Access Occasions” field in A-IoT paging message in clause 6.2.1.1 of TS 38.391.

References

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), RAN#106.

- RP-251885, “New WID on Solutions for Ambient IoT (Internet of Things) in NR Phase 2”, Moderator (RAN1 Vice-Chair), RAN#108.

- R2-2505521, “Remaining A-IoT MAC open issues”, Huawei, HiSilicon, RAN2#131.

- R2-2505520, “A-IoT MAC running CR”, Huawei, HiSilicon, RAN2#131.

- Appendix

  - TP#1 for TS 38.391

--------------------------------------------- Start of change to TS 38.391 -----------------------------------------

#### 6.2.1.1	A-IoT Paging message

#### < Unchanged parts are omitted >

For CBRA, the following fields are further included:

-	Paging ID Presence Indication (PIPI): This field indicates whether Paging ID and Length of Paging ID are present (when set to 1) or absent (when set to 0). The length of the field is 1 bit.

-	Paging ID Length: This field indicates the length of the Paging ID field in unit of bit when Paging ID field is present. The length of the field is 8 bits.

-	Paging ID: This field contains AIoT Identification Information (as defined in TS 23.369 [4], clause 5 and TS 23.003 [5]). .

-	Transaction ID: This field associates an inventory procedure or command procedure as specified in TS 38.300 [3]. The length of the field is xxx bits.

-	Number of Access Occasions: This field indicates the number of access occasions. The length of the field is 4 bits. The value v indicates the number of access occasions is 2vX⋅NSFS⋅X⋅NSFS where X and NSFS are defined in clause 6.2.1.6.

---------------------------------------------- End of change to TS 38.391 ------------------------------------------