3GPP TSG-RAN2 Meeting #131	R2-2505631

Bengaluru, India, Aug 19th – 23rd, 2025

Agenda Item:	8.10.2

Source: 	LG Electronics

Title:  	Remaining open issues for MOB MRO

Document for:	Discussion

Introduction

In this contribution, we focus on the remaining open issues for MOB MRO based on [1].

Discussion

Open issue RRC-1

- [Open issue RRC-1] For correlation of SHR and SHR generated for a CHO with Candidate SCG, RAN2 discuss and agree to one of the following options - Correlation indication in RRC_Complete message (a flag beside availability indication) - Correlation indication inside the SHR and SPR - Logging C-RNTI of PCell and timeSinceSPR in SPR - The duplicated information in correlated SHR and SPR is sufficient. no further information is needed for correlation.

We agree that correlation between SHR and SPR reports in a CHO with candidate SCG scenario is conceptually useful. However, we do not support adding explicit correlation indication in signaling (Option B).

Our reasoning is simple:

- SHR and SPR use the same ChoWithCandidateSCGInfo structure and contain identical CHO+CPAC results.

- Duplicating the same information in both reports brings no benefit and increases signaling overhead.

- If duplication is avoided at UE side, correlation indication becomes unnecessary.

Given the above reasoning, we believe that Option D can be enhanced with a UE-side duplicate logging avoidance mechanism.

This approach:

- Eliminates redundancy at the source, keeping signaling simple.

- Avoids post-processing complexity in the network.

- Can be extended to other duplication cases (e.g., RLF and SCG failure).

In summary, our proposal provides the same analytical value as Option B, but with simpler procedures and no additional flags in the specification. The table below compares Option B and our proposed UE-side duplicate logging avoidance:

|  | Option B – Correlation Indication in Reports | Our proposal – UE Duplicate Logging Avoidance |
| --- | --- | --- |
| NW analysis convenience | Explicit indication helps correlate reports in post-processing | No need for correlation; no duplicates to process |
| Specification complexity | Requires synchronization if reports are replaced or updated | Simple: add duplicate suppression condition to UE logging |
| UE implementation effort | Simple flag setting | Requires comparison of CHO+CPAC results (slightly more logic) |
| Signaling overhead | Sends same information twice | No duplicate transmission |
| Risk of misinterpretation | Possible if correlation flag is wrong or outdated | No duplicates means no correlation errors |
| Scalability to other scenarios | Limited to SHR–SPR | Can apply to RLF–SCG failure and other duplication cases |

Proposal 1:	(RRC-1) The UE is allowed to avoid duplicate SHR and SPR logging at the UE side to remove the need for extra correlation indication.

If the normative specification is considered too restrictive, a note can be added allowing the UE to suppress such duplication. Please refer to the Annex for the detailed text proposal.

Proposal 2:	(RRC-1) For proposal 1, to keep the procedures simple and efficient, add a NOTE to the relevant procedure text to suppress duplication logging, as described in the TP in Annex.

Open issue RRC-2

- [Open issue RRC-2] For the scenario of CHO with candidate SCG with complementary CHO only configuration, companies are invited to submit contribution with a text proposal addressing the issue and their proposed solution.

CHO with candidate SCG configuration is further enhanced configuration from CHO with SCG configuration. this is because SCG failure could be occurred frequently by not checking whether PSCell’s signal quality is suitable when initiating CHO. At that stage, even though RAN2 didn’t specify anything but CHO without SCG configuration (i.e. exactly same as CHO-only configuration) is allowed to be configured together.

In that sense, we think executing CHO-only configuration is the intended and normal behavior. The network configures the CHO-only for the case when the SCG conditions are not satisfied, the UE doesn’t add or change the PSCell but the UE applies the PCell only for the handover. So, the network will know that the UE has executed the CHO-only instead of CHO+CPAC even though the UE doesn’t report any thing to the network after the handover.

Thus, we propose the following.

Proposal 3:	(RRC-2) The UE doesn’t log indicating whether CHO-only configuration has been configured for the scenario of CHO with candidate SCG.

Open issue RRC-6

- [Open issue RRC-6] RAN2 agree that RAN3 based solution in logging LTM candidate cells can be applicable/used in both RLF and SHR scenarios. RAN2 remove logging LTM candidate flag from SHR procedure.

The RAN3-based network-side logging solution has been agreed and validated for RLF scenarios, whereas for SHR, a RAN2-based UE-side solution has been agreed, i.e., adding a candidate cell flag for LTM candidate cell logging.

The WI rapporteur proposes to remove this candidate cell flag from the SHR procedure and to handle LTM candidate cell logging for both RLF and SHR using the same RAN3-based network-side solution.

In SHR, the UE reports SHR information to the target cell immediately after the successful completion of the handover. As a result, the network can clearly recognize SHR in a post-handover manner, similar to how it becomes aware of RLF via the RLF report.

Furthermore, except for the candidate cell flag, the logging information reported in SHR is essentially equivalent to that reported in RLF, allowing the same processing logic to be applied in both cases.

For these reasons, the RRC-6 proposal is considered technically and operationally justified.

Thus, we propose the following.

Proposal 4:	(RRC-6) A common RAN3-based network-side logging solution can be applied to both SHR and RLF-report scenarios

Open issue RRC-7

- [Open issue RRC-7] If time allows RAN2 discuss duplicated CHO with Candidate SCG information (ChoWithCandidateSCGInfo) in the RLF and SCGFailureInformation.

We have identified another duplication scenario similar to Open issue RRC-1. When MCG RLF and SCG RLF occur in close succession while the UE is evaluating CHO and CPAC execution conditions, the same evaluation results may be logged in both the SCG failure information and the RLF report:

- SCG RLF occurs → UE logs CHO and CPAC evaluation results in SCG failure information

- UE sends the information to the source MN

- Source MN receives this as the first log sample

- Later, MCG RLF occurs → UE logs the same CHO and CPAC evaluation results in the RLF report

- After RRC re-establishment, a new PCell (possibly under a new MN) receives the RLF report and forwards it to the source MN

- The source MN receives this data again, treating it as a second log sample. Due to the time gap between the two log deliveries, the network may not recognize them as duplicates, leading to erroneous parameter updates.

Since these logs are reported at different times, the network may treat them as two separate samples, potentially leading to incorrect mobility optimization decisions.

Our reasoning is simple:

- RLF report and SCG failure information use the same ChoWithCandidateSCGInfo structure and can contain identical CHO+CPAC results.

- Duplicating the same information in both reports brings no benefit and increases signaling overhead.

- If duplication is avoided at UE side, there is no need for the network to perform complex correlation or post-processing.

This approach:

- Eliminates redundancy at the source, keeping signaling simple.

- Avoids post-processing complexity in the network.

- Aligns with the same principle proposed in Open issue RRC-1 for SHR and SPR correlation.

Thus, we propose the following.

Proposal 5:	(RRC-7) The UE is allowed to avoid duplicate CHO+CPAC logging across RLF report and SCG failure information.

Proposal 6:	(RRC-7) Similar to Open issue RRC-1, to keep the procedures simple and efficient, add a NOTE to the relevant procedure text to suppress duplication logging, as described in the TP in Annex.

Conclusion

It is proposed to discuss and decide on the following proposals:

Proposal 1:	(RRC-1)	The UE is allowed to avoid duplicate SHR and SPR logging at the UE side to remove the need for extra correlation indication.

Proposal 2:	(RRC-1)	For proposal 1, to keep the procedures simple and efficient, add a NOTE to the relevant procedure text to suppress duplication logging, as described in the TP in Annex.

Proposal 3:	(RRC-2)	The UE doesn’t log indicating whether CHO-only configuration has been configured for the scenario of CHO with candidate SCG.

Proposal 4:	(RRC-6)	A common RAN3-based network-side logging solution can be applied to both SHR and RLF-report scenarios

Proposal 5:	(RRC-7)	The UE is allowed to avoid duplicate CHO+CPAC logging across RLF report and SCG failure information.

Proposal 6:	(RRC-7)	Similar to Open issue RRC-1, to keep the procedures simple and efficient, add a NOTE to the relevant procedure text to suppress duplication logging, as described in the TP in Annex.

References

- [Post130][605][SONMDT] Remaining RRC Open issues for SONMDT feature (Ericsson)

Annexes

TP for open issue RRC-1

5.7.10.6 Actions for the successful handover report determination The UE shall for the PCell: 1> if the procedure is triggered due to successful completion of reconfiguration with sync, and if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync; or 1> if the procedure is triggered due to successful completion of reconfiguration with sync, and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or 1> if the procedure is triggered due to successful completion of reconfiguration with sync, and if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync; or 1> if the procedure is triggered due to successful completion of reconfiguration with sync, and if sourceDAPS-FailureReporting is included in the successHO-Config before executing the last reconfiguration with sync and is set to true and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running; or: 1> if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PCell before executing the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last Mobility from NR to E-UTRA; or 1> if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, and if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the Mobility from NR to E-UTRA and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PCell before executing the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last Mobility from NR to E-UTRA: 2> store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows: 3> clear the information included in VarSuccessHO-Report, if any; 3> if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e., includes the RPLMN); 3> else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPNs stored by the UE (i.e., including the registered SNPN identity), if available; 3> for intra-NR handover, set the c-RNTI to the C-RNTI assigned by the target PCell of the handover; 3> if the procedure is triggered due to successful completion of reconfiguration with sync, for the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; or 3> if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for the source PCell in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied: 4> set the sourcePCellID in sourceCellInfo to the global cell identity and tracking area code, if available, of the source PCell; 4> set the sourceCellMeas in sourceCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA; 4> set the rsIndexResults in sourceCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PCell collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA; 4> if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config including LTM-CSI-ReportConfig associated with the source PCell when connected to the source PCell: 5> set the resultsSSB-Indexes in sourceCellMeasL1 to include all the available SS/PBCH block L1-RSRP measurement results of the source PCell collected up to the moment the UE sends RRCReconfigurationComplete message; 4> if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running: 5> set the rlf-InSourceDAPS in sourceCellInfo to true; 3> if the procedure is triggered due to successful completion of reconfiguration with sync and if the UE was configured with condExecutionCond and condExecutionCondPScell, for the source PSCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; 4> set the sourcePSCellId in sourcePSCellInfo to the global cell identity and tracking area code, if available, of the source PSCell; 4> set the sourcePSCellMeas in sourcePSCellInfo to include the available cell level RSRP, RSRQ and the SINR, of the source PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync; 4> set the rsIndexResults in sourcePSCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PSCell collected up to the moment the UE sends RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync; 4> set the targetPSCellId to the global cell identity and tracking area code, if available, of the target PSCell, and otherwise to the physical cell identity and carrier frequency of the target PSCell; 3> if the procedure is triggered due to successful completion of reconfiguration with sync, for the target PCell indicated in the last applied RRCReconfiguration message including reconfigurationWithSync: 4> set the targetPCellID in targetCellInfo to the global cell identity and tracking area code, if available, of the target PCell; otherwise, set the targetCell-PCI-ARFCN to the physical cell identity and carrier frequency of the target PCell; NOTE 00: If targetCell-PCI-ARFCN is included, it is left to UE implementation how to set the targetPCellID. 4> set the targetCellMeas in targetCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PCell based on the available SSB and CSI-RS measurements collected up to the moment the UE sends RRCReconfigurationComplete message; 4> set the rsIndexResults in targetCellMeas to include all the available SSB and CSI-RS measurement quantities of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message; 4> if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config including LTM-CSI-ResourceConfig associated with the target PCell when connected to the source PCell: 5> set the resultsSSB-Indexes in targetCellMeasL1 to include all the available SS/PBCH block L1-RSRP measurement results of the target PCell collected up to the moment the UE sends RRCReconfigurationComplete message; 4> if the last applied RRCReconfiguration message including reconfigurationWithSync was included in the stored condRRCReconfig: 5> set the timeSinceCHO-Reconfig to the time elapsed between the initiation of the execution of conditional reconfiguration for the target PCell and the reception of the last applied conditionalReconfiguration including the condRRCReconfig of the target PCell in the source PCell; 3> if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for the target PCell indicated in the last applied MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA: 4> set the targetPCellId in eutra-TargetCellInfo to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell; 4> set the targetCellMeas in eutra-TargetCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PCell based on the available measurements collected up to the moment the UE sends RRCConnectionReconfigurationComplete message; NOTE 0: If eutra-TargetCellInfo is included, it is left to UE implementation how to set the targetCellInfo. 3> if the procedure is triggered due to successful completion of reconfiguration with sync and if the ratio between the value of the elapsed time of the timer T304 and the configured value of the T304 timer, included in the last applied RRCReconfiguration message including the reconfigurationWithSync, is greater than thresholdPercentageT304 if included in the successHO-Config received before executing the last reconfiguration with sync: 4> set t304-cause in shr-Cause to true;4> if the procedure is triggered due to successful completion of RACH-based reconfiguration with sync: 5> set the ra-InformationCommon to include the random-access related information associated to the random access procedure in the target PCell, as specified in clause 5.7.10.5; 3> if the ratio between the value of the elapsed time of the timer T310 and the configured value of the T310 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync or the last Mobility from NR to E-UTRA, is greater than thresholdPercentageT310 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync or Mobility from NR to E-UTRA: 4> set t310-cause in shr-Cause to true; 3> if the T312 associated to the measurement identity of the target cell was running at the time of initiating the execution of the reconfiguration with sync procedure or Mobility from NR to E-UTRA, and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the T312 timer, configured while the UE was connected to the source PCell before executing the last reconfiguration with sync or Mobility from NR to E-UTRA, is greater than thresholdPercentageT312 included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync, or Mobility from NR to E-UTRA: 4> set t312-cause in shr-Cause to true; 3> if the procedure is triggered due to successful completion of reconfiguration with sync and if sourceDAPS-FailureReporting included in the successHO-Config if configured by the source PCell before executing the last reconfiguration with sync is set to true, and if the last executed handover was a DAPS handover and if an RLF occurred at the source PCell during the DAPS handover while T304 was running: 4> set sourceDAPS-Failure in shr-Cause to true; 3> if the procedure is triggered due to successful completion of reconfiguration with sync, for each of the measObjectNR, configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied;or: 3> if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for each of the measObjectNR, configured by the source PCell, in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied: 4> if measRSSI-ReportConfig is configured for the frequency of the source PCell: 5> if the procedure is triggered due to successful completion of reconfiguration with sync: 6> set the measResultServCellRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequency of the source PCell up to the moment the UE sends the RRCReconfigurationComplete message; 5> else if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA: 6> set the measResultServCellRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequency of the source PCell up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message; 4> for each of the configured measObjectNR if measRSSI-ReportConfig is configured for the configured frequency: 5> if the procedure is triggered due to successful completion of reconfiguration with sync: 6> set the measResultNeighFreqRSSI in the measResultNeighFreqListRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the associated neighbouring frequency up to the moment the UE sends the RRCReconfigurationComplete message: 7> for each neighbour frequency included, include the optional fields that are available; 5> else if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA: 6> set the measResultNeighFreqRSSI in the measResultNeighFreqListRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the associated neighbouring frequency up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message: 7> for each neighbour frequency included, include the optional fields that are available; 4> if measurements are available for the measObjectNR: 5> if the SS/PBCH block-based measurement quantities are available: 6> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell or target PCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA; 6> for each neighbour cell included, include the optional fields that are available; NOTE 1: For the neighboring cells set included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, the UE includes also the CSI-RS based measurement quantities, if available. 5> if the CSI-RS measurement quantities are available: 6> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell and target PCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE sends the RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA; 6> for each neighbour cell included, include the optional fields that are available; NOTE 2: For the neighboring cells set ordered based on the CSI-RS measurement quantities, the UE includes measurements only for the cells not yet included in measResultListNR in measResultNeighCells to avoid overriding SS/PBCH block-based ordered measurements. 3> if the UE supports successful handover report for MCG LTM cell switch and if the UE was configured with ltm-Config including LTM-CSI-ReportConfig associated with the MCG when connected to the source PCell: 4> for each neighbour MCG LTM candidate cell: 5> if SS/PBCH block-based L1-RSRP measurement results are available: 6> set the neighCellsMeasL1ListNR to include all the available SS/PBCH block-based L1-RSRP measurement results of the best measured cells, other than the source PCell or target PCell, ordered such that the cell with highest SS/PBCH block-based L1-RSRP (of all SS/PBCH block-based L1-RSRP measurement results for the cell) is listed first, based on the available SS/PBCH block-based L1-RSRP measurements collected up to the moment the UE sends the RRCReconfigurationComplete message; 3> if the procedure is triggered due to successful completion of reconfiguration with sync, for each of the measObjectEUTRA, configured by the source PCell in which the last RRCReconfiguration message including reconfigurationWithSync was applied; or: 3> if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA, for each of the measObjectEUTRA, configured by the source PCell in which the last MobilityFromNRCommand concerning an inter-RAT handover from NR to E-UTRA was applied: 4> if measurements are available for the measObjectEUTRA: 5> set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, based on measurements collected up to the moment the UE sends the RRCReconfigurationComplete message if the procedure is triggered due to successful completion of reconfiguration with sync, or up to the moment the UE sends the EUTRA RRCConnectionReconfigurationComplete message if the procedure is triggered due to successful completion of Mobility from NR to E-UTRA; 5> for each neighbour cell included, include the optional fields that are available; 3> for each of the neighbour cells included in measResultNeighCells: 4> if the cell was a candidate target cell included in the condRRCReconfig within the conditionalReconfiguration configured by the source PCell, in which the last RRCReconfiguration message including reconfigurationWithSync was applied: 5> set the choCandidate to true in measResultNR; 4> if the UE supports successful handover report for MCG LTM cell switch and the cell was an LTM candidate cell in the LTM-Candidate within ltm-Config associated with the MCG when connected to the source PCell: 5> set the ltm-Candidate to true in measResultNR; 3> if the UE supports successful handover report for MCG LTM cell switch and the procedure is triggered due to successful completion of reconfiguration with sync concerning an LTM cell switch: 4> if the last executed LTM cell switch is a RACH-less LTM cell switch: 5> include the rach-Less; 3> for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows: 4> set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first in time; 4> if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled: 5> set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions; 4> set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList; 4> set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;3> if available, set the locationInfo as in 5.3.3.7; 1> release successHO-Config configured by the source PCell and thresholdPercentageT304 if configured by the target PCell. The UE may discard the successful handover information, i.e., release the UE variable VarSuccessHO-Report, 48 hours after the last successful handover information is added to the VarSuccessHO-Report. NOTE 3: For CHO with candidate SCG, the UE may omit logging information if identical information has already been reported or included via another report. (omitted..) 5.7.10.7 Actions for the successful PSCell change or addition report determination The UE shall for the PSCell: 1> if the ratio between the value of the elapsed time of the timer T304 and the configured value of the timer T304, included in the last applied RRCReconfiguration message for the SCG including the reconfigurationWithSync, is greater than thresholdPercentageT304-SCG if included in the successPSCell-Config received before executing the last reconfiguration with sync for the SCG; or 1> if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync for the SCG, is greater than thresholdPercentageT310-SCG included in the successPSCell-Config if configured by the source PSCell before executing the last reconfiguration with sync for the SCG; or 1> if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured and if the T312 associated to the measurement identity of the target PSCell included in a measConfig associated with the SCG was running at the time of initiating the execution of the reconfiguration with sync procedure for the SCG and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312-SCG included in the successPSCell-Config if configured by the source PSCell before executing the last reconfiguration with sync for the SCG: 1> if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is not configured and if the ratio between the value of the elapsed time of the timer T310 and the configured value of the timer T310, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync for the SCG, is greater than thresholdPercentageT310-SCG included in the successPSCell-Config if configured by the PCell before executing the last reconfiguration with sync for the SCG; or 1> if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is not configured and if the T312 associated to the measurement identity of the target PSCell included in a measConfig associated with the SCG was running at the time of initiating the execution of the reconfiguration with sync procedure for the SCG and if the ratio between the value of the elapsed time of the timer T312 and the configured value of the timer T312, configured while the UE was connected to the source PSCell before executing the last reconfiguration with sync, is greater than thresholdPercentageT312-SCG included in the successPSCell-Config if configured by the PCell before executing the last reconfiguration with sync for the SCG: 2> clear the information included in VarSuccessPSCell-Report, if any; 2> store the successful PSCell change or addition information in VarSuccessPSCell-Report and determine the content in VarSuccessPSCell-Report as follows: 3> if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs (including the RPLMN) stored by the UE; 3> else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPN identities (including the registered SNPN identity) stored by the UE, if available; 3> if the procedure is triggered due to successful completion of CHO with candidate SCG: 4> set the pCellId to the global cell identity and if available the tracking area code of the source PCell; 4> set the targetPCellId to the global cell identity and tracking area code, if available, of the target PCell, and otherwise to the physical cell identity and carrier frequency of the target PCell; 3> else: 4> set the pCellId to the global cell identity and tracking area code, if available, of the PCell; 3> for the source PSCell (in case of PSCell change procedure) in which the last RRCReconfiguration message for the SCG including reconfigurationWithSync was applied: 4> set the sourcePSCellId in sourcePSCellInfo to the global cell identity and tracking area code, and otherwise to the physical cell identity and carrier frequency of the source PSCell; 4> set the sourcePSCellMeas in sourcePSCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE successfully completed the random access procedure for the SCG; 4> set the rsIndexResults in sourcePSCellMeas to include all the available SSB and CSI-RS measurement quantities of the source PSCell collected up to the moment the UE successfully completed the random access procedure for the SCG; 3> for the target PSCell indicated in the last applied RRCReconfiguration message for the SCG including reconfigurationWithSync: 4> set the targetPSCellID in targetPSCellInfo to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PSCell; 4> set the targetPSCellMeas in targetPSCellInfo to include the cell level RSRP, RSRQ and the available SINR, of the target PSCell based on the available SSB and CSI-RS measurements collected up to the moment the UE successfully completed the random access procedure for the SCG; 4> set the rsIndexResults in targetPSCellMeas to include all the available SSB and CSI-RS measurement quantities of the target PSCell collected up to the moment the UE successfully completed the random access procedure for the SCG; 4> if the last applied RRCReconfiguration message for the SCG including reconfigurationWithSync was included in the stored condRRCReconfig: 5> set the timeSinceCPAC-Reconfig to the time elapsed between the initiation of the execution of conditional reconfiguration for the target PSCell and the reception of the last applied conditionalReconfiguration for the SCG including the condRRCReconfig of the target PSCell; 3> if triggering threshold for storing the successful PSCell change or addition information in VarSuccessPSCell-Report based on the thresholdPercentageT304-SCG is met: 4> set t304-cause in spr-Cause to true; 4> set the ra-InformationCommon to include the random-access related information associated to the random access procedure in the target PSCell, as specified in clause 5.7.10.5; 3> if triggering threshold for storing the successful PSCell change or addition information in VarSuccessPSCell-Report based on the thresholdPercentageT310-SCG is met: 4> set t310-cause in spr-Cause to true; 3> if triggering threshold for storing the successful PSCell change or addition information in VarSuccessPSCell-Report based on the thresholdPercentageT312-SCG is met: 4> set t312-cause in spr-Cause to true; 3> if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured: 4> consider all measObjectNR configured by the source PSCell; 3> else: 4> consider all measObjectNR configured by the PCell; 3> for each of the considered measObjectNR: 4> if measurements are available for the measObjectNR: 5> if the SS/PBCH block-based measurement quantities are available: 6> include in the measResultListNR in measResultNeighCells all the available measurement quantities of the best measured cells, other than the source PSCell (in case of PSCell change procedure) or target PSCell, ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE successfully completed the random access procedure; 6> for each neighbour cell included, include the optional fields that are available (including the CSI-RS based measurement quantities, if available); 5> if the CSI-RS measurement quantities are available for the cells not yet included in measResultListNR in measResultNeighCells: 6> include in the measResultListNR in measResultNeighCells all the available measurement quantities of the best measured cells, other than the source PSCell (in case of PSCell change procedure) and target PSCell, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE successfully completed the random access procedure; 6> for each neighbour cell included, include the optional fields that are available; 3> for each of the neighbour cells included in measResultNeighCells: 4> if the cell was a candidate target cell included in the condRRCReconfig within the conditionalReconfiguration, configured by the source PCell including the condExecutionCond within the conditionalReconfiguration associated to condEventA4 or by the source PSCell (in case of PSCell change) when the last RRCReconfiguration message for the SCG including reconfigurationWithSync was applied: 5> set the choCandidate to true in measResultNR; 3> for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows: 4> set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first in time; 4> if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled: 5> set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions; 4> set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList; 4> set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList; 3> if sn-InitiatedPSCellChange associated to the last applied RRCReconfiguration with reconfigurationWithSync for the SCG is configured: 4> if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the source PSCell; 4> include sn-InitiatedPSCellChange; 3> else: 4> if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the PCell; 1> release successPSCell-Config configured by the source PSCell if available and thresholdPercentageT304 if configured by the target PSCell. The UE may discard the successful PSCell change or addition information, i.e., release the UE variable VarSuccessPSCell-Report, 48 hours after the last successful PSCell change or addition information is added to the VarSuccessPSCell-Report or upon deregistration from the network as specified in TS 23.502 [43]. NOTE 1: For CHO with candidate SCG, the UE may omit logging information if identical information has already been reported or included via another report.

TP for open issue RRC-7

5.3.10.5 RLF report content determination The UE shall determine the content in the VarRLF-Report as follows: 1> clear the information included in VarRLF-Report, if any; 1> if the UE is not in SNPN access mode, set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. including the RPLMN); 1> else if the UE is in SNPN access mode, set the snpn-IdentityList to include the list of equivalent SNPNs stored by the UE (i.e., including the registered SNPN identity); 1> set the measResultLastServCell to include the cell level RSRP, RSRQ and the available SINR, of the source PCell (in case HO failure) or PCell (in case RLF) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected failure; 1> if the UE supports RLF-Report for conditional handover with candidate SCG and if the UE was configured with condExecutionCond and condExecutionCondPSCell; 2> set the measResultLastServPSCell to include the cell level RSRP, RSRQ and the available SINR, of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) based on the available SSB and CSI-RS measurements collected up to the moment the UE detected the failure; 2> if the UE does not support RLF-Report for fast MCG recovery procedure as specified in TS 38.306 [26] or if T316 is not configured: 3> set pSCellId to the the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change); 1> if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition and if one entry of choConfig concerns condEventD2; 2> set distanceFromReference1 to the measured distance between the UE and the serving cell moving reference location, 1> if measRSSI-ReportConfig is configured for the measObject indicated as the servingCellMO of the source PCell (in case HO failure) or PCell (in case of RLF), set the measResultLastServCellRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequency of the source PCell (in case HO failure) or PCell (in case of RLF) up to the moment the UE detected the failure; 1> if the SS/PBCH block-based measurement quantities are available: 2> set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure; 2> if the UE supports RLF-Report for conditional handover with candidate SCG and if the UE was configured with condExecutionCond and condExecutionCondPSCell: 3> set the rsIndexResults in measResultLastServPSCell to include all the available measurement quantities of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPSCell, ordered such that the highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure; 1> if the CSI-RS based measurement quantities are available: 2> set the rsIndexResults in measResultLastServCell to include all the available measurement quantities of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected failure; 2> if the UE supports RLF-Report for conditional handover with candidate SCG and if the UE was configured with condExecutionCond and condExecutionCondPSCell: 3> set the rsIndexResults in measResultLastServPSCell to include all the available measurement quantities of the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change), ordered such that the highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected failure; 1> if the UE supports RLF-Report for MCG LTM cell switch and if the UE was configured with ltm-Config associated with the MCG when connected to the source PCell (in case of HO failure) or PCell (in case of RLF) and if the SS/PBCH block-based L1-RSRP measurements performed based on LTM-CSI-ReportConfig are available: 2> set the resultsSSB-Indexes in measResultL1-LastServCell to include all the available SS/PBCH block-based L1-RSRP values of the source PCell (in case HO failure) or PCell (in case RLF), ordered such that the highest SS/PBCH block L1-RSRP measurement is listed first, based on the available SS/PBCH block-based L1-RSRP collected up to the moment the UE detected failure; 1> for each of the configured measObjectNR in which measurements are available or in which the associated reportConfigNR is configured as conditional handover with time-based or location-based trigger condition: 2> if the SS/PBCH block-based measurement quantities are available: 3> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), and other than the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPSCell and if the UE supports RLF-Report for conditional handover with candidate SCG ordered such that the cell with highest SS/PBCH block RSRP is listed first if SS/PBCH block RSRP measurement results are available, otherwise the cell with highest SS/PBCH block RSRQ is listed first if SS/PBCH block RSRQ measurement results are available, otherwise the cell with highest SS/PBCH block SINR is listed first, based on the available SS/PBCH block based measurements collected up to the moment the UE detected failure; 3> for each neighbour cell included, include the optional fields that are available; NOTE 0a: For the neighboring cells included in measResultListNR in measResultNeighCells ordered based on the SS/PBCH block measurement quantities, UE also includes the CSI-RS based measurement quantities, if available. 2> if the CSI-RS based measurement quantities are available: 3> set the measResultListNR in measResultNeighCells to include all the available measurement quantities of the best measured cells, other than the source PCell (in case HO failure) or PCell (in case RLF), and other than the source PSCell (in case of PSCell change) or PSCell (in case of no PSCell change) if the UE was configured with condExecutionCond and condExecutionCondPSCell and if the UE supports RLF-Report for conditional handover with candidate SCG, ordered such that the cell with highest CSI-RS RSRP is listed first if CSI-RS RSRP measurement results are available, otherwise the cell with highest CSI-RS RSRQ is listed first if CSI-RS RSRQ measurement results are available, otherwise the cell with highest CSI-RS SINR is listed first, based on the available CSI-RS based measurements collected up to the moment the UE detected radio link failure; 3> for each neighbour cell included, include the optional fields that are available; NOTE 0b: For ordering the neighboring cells based on the CSI-RS measurement quantities, UE includes measurements only for the cells not yet included in measResultListNR in measResultNeighCells to avoid overriding SS/PBCH block-based ordered measurements.2> if measurement quantities are not available: 3> set physCellId in measResultListNR in measResultNeighCells to include the physical cell identity of the neighbour cells that are candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure; 3> for each neighbour cell included, include the optional fields that are available; 2> for each neighbour cell, if any, included in measResultListNR in measResultNeighCells: 3> if the UE supports RLF-Report for conditional handover and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure and if the related MCG VarConditionalReconfig only concerns measurement-based trigger condition; or 3> if the UE supports RLF-Report for conditional handover with time-based and location-based trigger conditions in NTN and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure; or 3> if the UE supports RLF-Report for conditional handover with candidate SCG and if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig at the moment of the detected failure: 4> set choConfig in MeasResult2NR to the execution condition for each measId within condTriggerConfig associated to the neighbour cell within the MCG VarConditionalReconfig; 4> if the first entry of choConfig corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure; or 4> if the second entry of choConfig, if available, corresponds to a fulfilled execution condition at the moment of handover failure, or radio link failure: 5> set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of choConfig or to the execution condition condSecondEvent corresponding to the second entry of choConfig, whichever execution condition was fulfilled first in time; 5> set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in choConfig that was fulfilled first in time, and the point in time of fulfilling the condition in choConfig that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the choConfig were fulfilled; 4> if the UE supports RLF-Report for conditional handover with time-based or location-based trigger condition and if one entry of choConfig concerns condEventD2; 5> set distanceFromReference2 to the measured distance between the UE and the moving reference location of the neighbour cell, at the moment of handover failure, or radio link failure; 1> for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows: 2> if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled: 3> set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first; 3> set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions; 2> else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList is fulfilled: 3> set firstFulfilledConfig to cho or cpc, whichever was fulfilled; 3> set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the RLF; 2> set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList; 2> set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList;1> if the UE supports RLF-Report for MCG LTM cell switch, for each neighbour MCG LTM candidate cell: 2> if SS/PBCH block-based L1-RSRP measurement quantities performed based on LTM-CSI-ReportConfig are available: 3> set the measResultL1-NeighCells to include all the available SS/PBCH block-based L1-RSRP measurement results, ordered such that the cell with highest SS/PBCH block-based L1-RSRP (of all SS/PBCH block-based L1-RSRP measurement results for the cell) is listed first; 3> for each neighbour frequency included, include the optional fields that are available; 1> for each of the configured measObjectNR associated with neighboring cells if the associated reportConfigNR includes measRSSI-ReportConfig: 2> set the measResultNeighFreqRSSI in the measResultNeighFreqListRSSI to the linear average of the available RSSI sample value(s) provided by lower layers for the frequencies other than the frequency of the source PCell (in case HO failure) or of the PCell (in case RLF), up to the moment the UE detected failure: 3> for each neighbour frequency included, include the optional fields that are available; 1> for each of the configured EUTRA frequencies in which measurements are available; 2> set the measResultListEUTRA in measResultNeighCells to include the best measured cells ordered such that the cell with highest RSRP is listed first if RSRP measurement results are available, otherwise the cell with highest RSRQ is listed first, and based on measurements collected up to the moment the UE detected failure; 3> for each neighbour cell included, include the optional fields that are available; NOTE 1: The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported. 1> set the c-RNTI to the C-RNTI used in the source PCell (in case HO failure) or PCell (in case RLF); 1> if the failure is detected due to reconfiguration with sync failure as described in 5.3.5.8.3, set the fields in VarRLF-report as follows: 2> set the connectionFailureType to hof; 2> if the UE supports RLF-Report for DAPS handover and if any DAPS bearer was configured while T304 was running: 3> set lastHO-Type to daps; 3> if radio link failure was detected in the source PCell, according to clause 5.3.10.3: 4> set timeConnSourceDAPS-Failure to the time between the initiation of the DAPS handover execution and the radio link failure detected in the source PCell while T304 was running; 4> set the rlf-Cause to the trigger for detecting the source radio link failure in accordance with clause 5.3.10.4; 2> if the UE supports RLF-Report for conditional handover and if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of the handover failure: 3> if the UE executed a conditional handover toward target PCell according to the condRRCReconfig of the target PCell: 4> set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed conditional handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig of the target PCell of the failed conditional handover; 3> else: 4> set timeSinceCHO-Reconfig to the time elapsed between the execution of the last RRCReconfiguration message including reconfigurationWithSync for the target PCell of the failed handover, and the reception in the source PCell of the last conditionalReconfiguration including the condRRCReconfig; 3> set choCandidateCellList to include the global cell identity, if available, and otherwise to the physical cell identity and carrier frequency of each of the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of the failed handover, excluding the candidate target cells included in measResultNeighCells; 2> if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover: 3> set lastHO-Type to cho; 2> else if the UE supports RLF-Report for MCG LTM cell switch and the last executed RRCReconfiguration message including reconfigurationWithSync was concerning an LTM cell switch: 3> set lastHO-Type to ltm; 2> if the UE supports RLF-Report for conditional handover with candidate SCG and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning conditional handover with candidate SCG: 3> set lastHO-Type to choWithCandidateSCG; 2> set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover or a failed LTM cell switch; 2> include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last RRCReconfiguration message including reconfigurationWithSync was applied; 2> set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync; 1> else if the failure is detected due to Mobility from NR failure as described in 5.4.3.5, set the fields in VarRLF-report as follows: 2> set the connectionFailureType to hof; 2> if last MobilityFromNRCommand concerned a failed inter-RAT handover from NR to E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA (NR to EUTRA): 3> set the eutraFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover; 2> include nrPreviousCell in previousPCellId and set it to the global cell identity and tracking area code of the PCell where the last MobilityFromNRCommand message was received; 2> set the timeConnFailure to the elapsed time since the initialization of the handover associated to the last MobilityFromNRCommand message; 2> if the UE supports RLF report for inter-system handover for voice fallback and if voiceFallbackIndication is included in the last MobilityFromNRCommand: 3> include the voiceFallbackHO; 1> else if the failure is detected due to radio link failure as described in 5.3.10.3, set the fields in VarRLF-report as follows: 2> set the connectionFailureType to rlf; 2> set the rlf-Cause to the trigger for detecting radio link failure in accordance with clause 5.3.10.4; 2> set the nrFailedPCellId in failedPCellId to the global cell identity and the tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the PCell where radio link failure is detected; 2> if an RRCReconfiguration message including the reconfigurationWithSync was received before the connection failure: 3> if the last successfully executed RRCReconfiguration message including the reconfigurationWithSync concerned an intra NR handover or an LTM cell switch and the target cell of the intra NR handover or LTM cell switch was the PCell where radio link failure is detected; and 3> if T316 was not running before entering the PCell in which the radio link failure was detected; and 3> if T311 was not running before entering the PCell in which the radio link failure was detected: 4> include the nrPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the source PCell of the intra NR handover or LTM cell switch concerning the last successfully executed RRCReconfiguration message including reconfigurationWithSync; 4> if the UE supports RLF-Report for DAPS handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a DAPS handover: 5> set lastHO-Type to daps; 4> else if the UE supports RLF-Report for conditional handover and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning a conditional handover: 5> set lastHO-Type to cho; 4> else if the UE supports RLF-Report for MCG LTM cell switch and the last executed RRCReconfiguration message including reconfigurationWithSync was concerning an LTM cell switch: 5> set lastHO-Type to ltm; 4> if the UE supports RLF-Report for conditional handover with candidate SCG and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning conditional handover with candidate SCG: 5> set lastHO-Type to choWithCandidateSCG; 4> set the timeConnFailure to the elapsed time since the execution of the last RRCReconfiguration message including the reconfigurationWithSync; 3> else if the last RRCReconfiguration message including the reconfigurationWithSync concerned a handover to NR from E-UTRA and if the UE supports Radio Link Failure Report for Inter-RAT MRO EUTRA: 4> include the eutraPreviousCell in previousPCellId and set it to the global cell identity and the tracking area code of the E-UTRA PCell where the last RRCReconfiguration message including reconfigurationWithSync was received embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3; 4> set the timeConnFailure to the elapsed time since reception of the last RRCReconfiguration message including the reconfigurationWithSync embedded in E-UTRA RRC message MobilityFromEUTRACommand message as specified in TS 36.331 [10] clause 5.4.3.3; 2> if configuration of the conditional handover is available in the MCG VarConditionalReconfig at the moment of declaring the radio link failure: 3> set timeSinceCHO-Reconfig to the time elapsed between the detection of the radio link failure, and the reception, in the source PCell, of the last conditionalReconfiguration including the condRRCReconfig message; 3> set choCandidateCellList to include the global cell identity if available, and otherwise to the physical cell identity and carrier frequency of each of all the candidate target cells for conditional handover included in condRRCReconfig within the MCG VarConditionalReconfig at the time of radio link failure, excluding the candidate target cells included in measResultNeighCells; 1> if connectionFailureType is rlf and the rlf-Cause is set to randomAccessProblem or beamFailureRecoveryFailure; or 1> if connectionFailureType is rlf and the rlf-Cause is set to lbtFailure and the radio link failure is detected during the random access procedure; or 1> if connectionFailureType is hof and if the failed handover is an intra-RAT reconfiguration with sync and if a random-access procedure was trigged for the failed reconfiguration with sync: 2> set the ra-InformationCommon to include the random-access related information as described in clause 5.7.10.5; 1> if connectionFailureType is rlf and the rlf-Cause is set to lbtFailure, and the radio link failure is not detected during the random access procedure: 2> set the locationAndBandwidth and subcarrierSpacing in bwp-Info associated to the UL BWP in which the consistent uplink LBT failure was detected; 1> if the rlf-Cause is set to t310-Expiry or t312-Expiry: 2> set the ssbRLMConfigBitmap and/or csi-rsRLMConfigBitmap in measResultLastServCell to include the radio link monitoring configuration of the last serving cell, if available; 1> if available, set the locationInfo as in 5.3.3.7. The UE may discard the radio link failure information or handover failure information, i.e. release the UE variable VarRLF-Report, 48 hours after the radio link failure/handover failure is detected. NOTE 2: In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'. NOTE 3: For CHO with candidate SCG, the UE may omit logging information if identical information has already been reported or included via another report. (omitted..) 5.7.3.5 Actions related to transmission of SCGFailureInformation message The UE shall set the contents of the SCGFailureInformation message as follows: 1> if the UE initiates transmission of the SCGFailureInformation message due to T310 expiry: 2> set the failureType as t310-Expiry; 1> else if the UE initiates transmission of the SCGFailureInformation message due to T312 expiry: 2> set the failureType as other and set the failureType-v1610 as t312-Expiry; 1> else if the UE initiates transmission of the SCGFailureInformation message to provide reconfiguration with sync failure information for an SCG: 2> set the failureType as synchReconfigFailureSCG; 1> else if the UE initiates transmission of the SCGFailureInformation message to provide random access problem indication from SCG MAC: 2> if the random access procedure was initiated for beam failure recovery: 3> set the failureType as other and set the failureType-v1610 as beamFailureRecoveryFailure; 2> else: 3> set the failureType as randomAccessProblem; 1> else if the UE initiates transmission of the SCGFailureInformation message to provide indication from SCG RLC that the maximum number of retransmissions has been reached: 2> set the failureType as rlc-MaxNumRetx; 1> else if the UE initiates transmission of the SCGFailureInformation message due to SRB3 IP check failure: 2> set the failureType as srb3-IntegrityFailure; 1> else if the UE initiates transmission of the SCGFailureInformation message due to Reconfiguration failure of NR RRC reconfiguration message: 2> set the failureType as scg-reconfigFailure; 1> else if the UE initiates transmission of the SCGFailureInformation message due to consistent uplink LBT failures: 2> set the failureType as other and set the failureType-v1610 as scg-lbtFailure; 1> else if connected as an IAB-node and the SCGFailureInformation is initiated due to the reception of a BH RLF indication on BAP entity from the SCG: 2> set the failureType as other and set failureType-v1610 as bh-RLF; 1> else if the UE initiates transmission of the SCGFailureInformation message due to beam failure of the PSCell while the SCG is deactivated: 2> set the failureType as other and set failureType-v1610 as beamFailure; 1> include and set MeasResultSCG-Failure in accordance with 5.7.3.4; 1> for each MeasObjectNR configured by a MeasConfig associated with the MCG, and for which measurement results are available: 2> include an entry in measResultFreqList; 2> if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to ssb: 3> set ssbFrequency in measResultFreqList to the value indicated by ssbFrequency as included in the MeasObjectNR; 2> if there is a measId configured with the MeasObjectNR and a reportConfig which has rsType set to csi-rs: 3> set refFreqCSI-RS in measResultFreqList to the value indicated by refFreqCSI-RS as included in the associated measurement object; 2> if a serving cell is associated with the MeasObjectNR: 3> set measResultServingCell in measResultFreqList to include the available quantities of the concerned cell and in accordance with the performance requirements in TS 38.133 [14]; 2> set the measResultNeighCellList in measResultFreqList to include the best measured cells,ordered such that the best cell is listed first, and based on measurements collected up to the moment the UE detected the failure, and set its fields as follows; 3> ordering the cells with sorting as follows: 4> based on SS/PBCH block if SS/PBCH block measurement results are available and otherwise based on CSI-RS; 4> using RSRP if RSRP measurement results are available, otherwise using RSRQ if RSRQ measurement results are available, otherwise using SINR; 3> if the UE supports SCG failure information for mobility robustness optimization for conditional PSCell change or addition, for each neighbour cell, if any, included in measResultListNR in measResultFreqList: 4> if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is included in the secondaryCellGroup in the MCG VarConditionalReconfig (for CPA or inter-SN CPC in NR-DC) or SCG VarConditionalReconfig (for intra-SN CPC) at the moment of the detected SCG failure (radio link failure at PSCell or PSCell change or addition failure): 5> if the first entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell corresponds to a fulfilled execution condition at the moment of SCG failure; or 5> if the second entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell, if available, corresponds to a fulfilled execution condition at the moment of SCG failure: 6> set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell or to the execution condition condSecondEvent corresponding to the second entry of condExecutionCond or condExecutionCondSCG associated to the neighbour cell, whichever execution condition was fulfilled first in time; 6> set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in condExecutionCond or condExecutionCondSCG associated to the neighbour cell that was fulfilled first in time, and the point in time of fulfilling the condition in condExecutionCond or condExecutionCondSCG associated to the neighbour cell that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the condExecutionCond or condExecutionCondSCG associated to the neighbour cell were fulfilled; 3> if the UE supports SCG failure information for mobility robustness optimization for conditional handover with candidate SCG, for each neighbour cell, if any, included in measResultListNR in measResultFreqList: 4> if the neighbour cell is one of the candidate cells for which the reconfigurationWithSync is associated with both condExecutionCond and condExecutionCondPSCell in the MCG VarConditionalReconfig at the moment of the detected SCG failure (radio link failure at PSCell or PSCell change or addition failure): 5> if the first entry of condExecutionCond associated to the neighbour cell corresponds to a fulfilled execution condition at the moment of SCG failure; or 5> if the second entry of condExecutionCond associated to the neighbour cell, if available, corresponds to a fulfilled execution condition at the moment of SCG failure: 6> set firstTriggeredEvent to the execution condition condFirstEvent corresponding to the first entry of condExecutionCond associated to the neighbour cell or to the execution condition condSecondEvent corresponding to the second entry of condExecutionCond associated to the neighbour cell, whichever execution condition was fulfilled first in time; 6> set timeBetweenEvents to the elapsed time between the point in time of fulfilling the condition in condExecutionCond associated to the neighbour cell that was fulfilled first in time, and the point in time of fulfilling the condition in condExecutionCond associated to the neighbour cell that was fulfilled second in time, if both the first execution condition corresponding to the first entry and the second execution condition corresponding to the second entry in the condExecutionCond associated to the neighbour cell were fulfilled; 3> for each neighbour cell included: 4> include the optional fields that are available. NOTE 1: The measured quantities are filtered by the L3 filter as configured in the mobility measurement configuration. The measurements are based on the time domain measurement resource restriction, if configured. Exclude-listed cells are not required to be reported. NOTE 2: Field measResultSCG-Failure is used to report available results for NR frequencies the UE is configured to measure by SCG RRC signalling. 1> for each entry of condReconfigList in the MCG VarConditionalReconfig including both condExecutionCond and condExecutionCondPSCell, include an entry in choWithCandidateSCGInfoList and set the values as follows: 2> if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled: 3> set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first; 3> set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions; 2> else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList is fulfilled: 3> set firstFulfilledConfig to cho or cpc, whichever was fulfilled; 3> set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and the SCG failure; 2> set the pCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PCell stored in the condRRCReconfig of the concerned entry of condReconfigList; 2> set the psCellId to the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency, of the target candidate PSCell stored in the condRRCReconfig of the concerned entry of condReconfigList; 1> if available, set the locationInfo as in 5.3.3.7 according to the otherConfig associated with the NR MCG. 1> if the UE supports SCG failure for mobility robustness optimization: 2> if the failureType is set to synchReconfigFailureSCG; or 2> if the failureType is set to randomAccessProblem and the SCG failure was declared while T304 was running: 3> set perRAInfoList to indicate the performed random access procedure related information as specified in 5.7.10.5. 3> set the failedPSCellId to the physical cell identity and carrier frequency of the target PSCell of the failed PSCell change or failed PSCell addition; 3> if the failure occurred during a subsequent CPC: 4> set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG, if available; 3> else: 4> set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG, if available; 3> set the timeSCGFailure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure; 2> else: 3> set the failedPSCellId to the physical cell identity and carrier frequency of the PSCell in which the SCG failure was declared; 3> if the last RRCReconfiguration message including the reconfigurationWithSync for the SCG was received to enter the PSCell in which the SCG failure was declared: 4> set the timeSCGFailure to the elapsed time since the last execution of RRCReconfiguration message including the reconfigurationWithSync for the SCG until declaring the SCG failure; 4> if the failure occurred after a subsequent CPC: 5> set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last execution of RRCReconfiguration message including reconfigurationWithSync for the SCG, if available; 4> else: 5> set the previousPSCellId to the physical cell identity and carrier frequency of the source PSCell associated to the last received RRCReconfiguration message including reconfigurationWithSync for the SCG; 1> release successPSCell-Config configured by the source PSCell, if available. The UE shall submit the SCGFailureInformation message to lower layers for transmission. NOTE 3: For CHO with candidate SCG, the UE may omit logging information if identical information has already been reported or included via another report.