3GPP TSG-RAN WG2 Meeting #131	R2-2505634

Bengaluru, India, 25 – 29 August 2025

Agenda item:	8.17

Source:	Nokia, Nokia Shanghai Bell

Title:	Remaining issues on support of TDD mode for IoT-NTN

WID/SID:	IoT_NTN_TDD - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN #106 the work item on NB-IoT TDD NTN mode [1] was agreed with the following objective:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

RAN2 progressed well on this topic since RAN2-129 meeting. In this contribution we discuss remaining issues to support TDD NTN mode, which impacts RAN2.

# 2	Discussion

According to the WID and RAN1 discussions the NB-IoT TDD NTN frame structure is as illustrated in Figure 1.

Figure 1 NB-IoT TDD NTN frame structure.

RAN1 has agreed that the UL and DL occasions are paired, i.e. if UE has received scheduling information in DL1 the corresponding UL occasion is UL1. In general, the sparsity of DL and UL subframes impacts the network capacity, especially when considering the mandatory signalling (NPSS, NSSS, NPBCH, SIB1, SIB31). According to RAN1 conclusion the overhead of transmitting the aforementioned, excluding SIB31, is about 40 % of the anchor carrier. Thus the number of subframes, which can carry control- and user-plane data is limited. For example, the NB-IoT PRACH preamble has a high capacity due to the use of 3.75 kHz subcarrier. However, the RAR/Msg2 will require a larger bandwidth and thus it may be challenging for the eNB to respond timely to all UEs if many preambles were transmitted in the preceding UL occasion.

Observation 1: The 8 DL subframes of a DL occasion may have insufficient capacity to carry RAR for all UEs transmitting a preamble in the preceding UL occasion.

Based on the RAN1 agreements, if the UE has transmitted in UL1 the UE will have to monitor for a response in DL1. According to the frame structure definition, the time-separation is 3*8.28 ms + 3 guard periods which approximates 25 ms. The WID assumptions define LEO 600 km and 1200 km orbits and thus the RTT is in the range 4-12.88 ms for a regenerative LEO600 deployment and 8-25.76 ms for a transparent LEO600 deployment depending on the UE’s elevation angle to the satellite. This means that the UL-DL occasion gap in some scenarios is significantly longer than the RTT and in other scenarios the RTT and the gap is quite comparable.

Observation 2: The time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is about 25 ms.

The large UL-DL time separation impacts the RAN2 specification, because some MAC procedures are based on the UE-eNB RTT. For example, the 4-step Random Access procedure utilize the UE-eNB RTT when defining the start of the RAR window after Msg1 transmission and for start of the Contention Resolution timer after Msg3 transmission.

As an example, the RAR window (TS 36.321) starts based on X subframes + UE-eNB RTT. Starting the RAR window based on X (of table 5.1.4-1) and the UE-eNB RTT may result in most of the window occurring during the UL-DL occasions gap. This is problematic from UE energy consumption point of view and also limits the eNB scheduling flexibility, because it may reduce the overlap between the RAR window and the DL subframes in the DL occasion.

Observation 3: The RAR window may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

A similar issue also exists for the Contention Resolution timer, because it is started based on the UE-eNB RTT as well.

Observation 4: The Contention Resolution timer may run primarily during the UL-DL occasion gap if the start of the timer is based on UE-eNB RTT.

Based on this issue we propose that RAN2 discuss how to better align the UE’s PDCCH monitoring, based on the RAR window and Contention Resolution timer, with the DL occasions. We consider a feasible solution is to ensure the RAR window and Contention Resolution timer does not start before the next DL occasion.

This can be achieved by implementing the TP provided below (with the modification in red). Using this update, the RAR window and Contention Resolution timer will either start at the first DL subframe of the next DL occasion or start later if the UE-eNB RTT is long.

Proposal 1: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap. RAN2 to discuss following TP implementation.

### 5.1.4 Random Access Response reception … If the UE is an NB-IoT UE: - if the random access preamble was transmitted in a non-terrestrial network: - if the non-terrestrial network operates in IoT-NTN TDD mode - RA Response window starts at the subframe that contains the end of the last preamble repetition plus max (X subframes plus UE-eNB RTT, time distance from the last preamble repetition to the next DL subframe) and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions; - else - RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions; …

Within the 90ms, for one cell, only one pair of UL and DL occasions can be used for the UEs served by the cell. For the other occasions and the gaps between the occasions, it can be regarded as unavailable subframes. For UE in RRC idle state, the UE may however still be able to measure neighbour cells in the subframes, which are unavailable on the serving cell, because the neighbour cell(s) may use different pair(s) of UL and DL.

At RAN4 #114bis it was agreed that cell reselection requirements apply if the UE is provided with information on the active periods of the neighbour cells configured for the measurements. A similar statement was also discussed for RRC Connected mode measurements.

Agreement: For the cell reselection for TDD NB-IoT in NTN, - The requirements are applicable only if UE is provided with the active periods of the neighbour cells configured for the measurements. - For earth fixed cell and earth moving cell: - The existing values for Tdetect, Tmeasure, and Tevaluate in Table 4.6A.2.2-1 and Table 4.6A.2.2-2 can be applicable if it is confirmed that the current requirement is achievable without requiring more than one NPSS/NSSS sample for half the DRX cycle length [due to the TDD pattern] - Further discuss the values if -1dB will be used. Ksatellite = 1 for the cells measured in DL opportunities of a neighbour satellite that are in non-overlapping with DL opportunities to be measured in the serving satellite

Observation 5: RAN4 has agreed measurement requirements are applicable if the UE is provided with information on the active periods of the neighbour cells configured for the measurements.

RAN1 has defined the NTN TDD frame structure in TS 36.211 as follows:

## 4.4 Frame structure type 1 for NTN-TDD Frame structure type 1 is applicable to NTN-TDD in band 249. Each radio frame is Tf=307200Ts=10 ms long and consists of 10 subframes of length 30720Ts=1 ms, numbered from 0 to 9. Subframe i in frame nf has an absolute subframe number nsfabs=10nf+i where nf is the system frame number. The frame structure for NTN-TDD , at the uplink time synchronization reference point defined in clause 16.1.2 of TS 36.213 [4] consists of 8 consecutive downlink subframes, followed by 50 consecutive guard period subframes, followed by 8 consecutive uplink subframes in each 90 ms interval. - The UE shall not assume any signal or channel being transmitted in subframes other than downlink subframes 3, 4, 5, 6, 7, 8, 9, and 0 across two consecutive radio frames. - The UE shall not transmit any signal or channel on a subframe other than the 8 consecutive uplink subframes.

For a UE to understand, whether and when a neighbor cell can be measured the UE will need to know when the 8 consecutive downlink subframes occur in the neighbor cell. Therefore, we propose the delta between the serving cell first DL subframe and the neighbor cell first DL subframe can be indicated to the UE as part of the assistance information, which already includes satellite ephemeris and epoch time.

In current TS 36.331 the SIB33 includes the NeighSatelliteInfoList, which can be updated using the following TP. We propose to make a sequence of four offsets, since each satellite can use one or more of the 4 possible IoT NTN TDD frame structure configurations. The offset has the same range as the frame periodicity (90 ms).

#### – SystemInformationBlockType33-NB The IE SystemInformationBlockType33-NB contains satellite assistance information for neighbour cells. SystemInformationBlockType33-NB information element -- ASN1START SystemInformationBlockType33-NB-r18 ::= SEQUENCE { neighSatelliteInfoList-r18 NeighSatelliteInfoList-r18 OPTIONAL, -- Need OR neighValidityDuration-r18 ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120, s180, s240, s900} OPTIONAL, -- Need OP lateNonCriticalExtension OCTET STRING OPTIONAL, ... } -- ASN1STOP #### – SystemInformationBlockType33 The IE SystemInformationBlockType33 contains satellite assistance information for neighbour cells. SystemInformationBlockType33 information element -- ASN1START SystemInformationBlockType33-r18 ::= SEQUENCE { neighSatelliteInfoList-r18 NeighSatelliteInfoList-r18 OPTIONAL, -- Need OR neighValidityDuration-r18 ENUMERATED {s5, s10, s15, s20, s25, s30, s35, s40, s45, s50, s55, s60, s120, s180, s240, s900} OPTIONAL, -- Need OP lateNonCriticalExtension OCTET STRING OPTIONAL, ... } NeighSatelliteInfoList-r18 ::= SEQUENCE (SIZE(1..maxSat-r17)) OF NeighSatelliteInfo-r18 NeighSatelliteInfo-r18 ::= SEQUENCE { satelliteId-r18 SatelliteId-r18, ephemerisInfo-r18 CHOICE { stateVectors-r18 EphemerisStateVectors-r17, orbitalParameters-r18 EphemerisOrbitalParameters-r17 }, nta-CommonParameters-r18 SEQUENCE { nta-Common-r18 INTEGER (0..8316827) OPTIONAL, -- Need OP nta-CommonDrift-r18 INTEGER (-261935..261935) OPTIONAL, -- Need OP nta-CommonDriftVariation-r18 INTEGER (0..29479) OPTIONAL -- Need OP }, epochTime-r18 SEQUENCE { startSFN-r18 INTEGER (0..1023), startSubFrame-r18 INTEGER (0..9) } OPTIONAL, -- Need OP k-Mac-r18 INTEGER (1..512) OPTIONAL, -- Need OP t-ServiceStartNeigh-r18 TimeOffsetUTC-r17 OPTIONAL -- Need OR tdd-DLstartOffsetList-r19 ::= SEQUENCE (SIZE(1..4) OF tdd-DLstartOffset-r19 tdd-DLstartOffset INTEGER (0..90) OPTIONAL } -- ASN1STOP SystemInformationBlockType33 field descriptions. tdd-DLstartOffset Indicates the time offset between the start of the serving cell first DL subframe and a neighbor cell first DL subframe. Unit of ms. This field is only present when the cells are operating in IoT NTN TDD mode..

Proposal 2: Severing cell to provide assistance information on the active periods of the neighbour cells configured for measurements. RAN2 to discuss the TS 36.331 TP.

For TDD UE AS operation in RRC Connected state, we think one approach can be to base the UE behavior on what was agreed for GNSS measurement gaps for NB-IoT. The stage 2 TS 36.300 defines the following:

NOTE: The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is performing GNSS acquisition and resumed when the GNSS acquisition is completed.

Differently from the NOTE, for the NB-IoT TDD NTN operation the neighbor cell measurement can still be performed outside the UL-DL pair of the serving cell, while the other UE AS operations such as RLM and RACH shall be suspended.

Proposal 3: RAN2 to discuss reusing the AS operation from GNSS measurement gap for RRC Connected UE working in NB-IoT TDD NTN, except the neighbor cell measurement which is possible outside the serving cell’s UL-DL pair. The below TP for TS 36.300 can be a starting point:

NOTE: The AS operations (e.g. RLM related timers, dataInactivityTimer, CHO execution, neighbour cell measurement, RACH, SR, and BSR) are suspended when UE is served in a IoT NTN TDD cell and the current subframe is not one of the 8 consecutive downlink subframes or 8 consecutive uplink subframes.

At RAN2 #129bis it was agreed that when a PUR or SPS resource is in a non-U NB-IoT subframe, the UE postpones the PUR or SPS resource to the next valid UL subframe. Furthermore, at RAN1 #120 it was agreed that NPUSCH is mapped per current specifications, i.e. postponed to valid NB-IoT UL subframes.

RAN1 #120 agreement The non-U NB-IoT subframes are not considered by the UE as “NB-IoT UL subframes” from the specifications - NPUSCH is mapped as per current specifications. - NOTE: “non-U NB-IoT subframes” are subframes not contained within the set of U=8 usable consecutive uplink subframes in the TDD structure.

This is a good starting point to handle the sparse uplink resource availability for the individual UE, but from the network point of view it may still happen that multiple UEs postpone their semi-static uplink resource (PUR or SPS), and also dynamically scheduled NPUSCH, to the next valid uplink subframe and therefore there will be resource scarcity and in the worst-case scenario even collisions.

Observation 6: when multiple UEs postpone their PUR/SPS resource and dynamically scheduled NPUSCH to the next valid UL subframe there may be collisions between different UEs.

One may argue that the postponement issue can be addressed by network implementation, for example, by allocating different UEs in different frequency domain resources.  However, since the NB-IoT has only 1 PRB (12 tone) when the subcarrier spacing is 15 kHz, it is not possible to allocate two UEs with different frequency domain resource if the UE is allocated with 12 tone transmission. It is worth noting that any UE having a PUR or SPS resource or a dynamically scheduled NPUSCH resource, which occurs within the time between a first UL occasion and the start of a second UL occasion, will all postpone the resource to the first valid UL subframe of the second UL occasion.

Therefore, we propose RAN2 to discuss alternative solutions to handle the PUR and SPS postponement issue. We think one useful way forward can be to define that some PUR or SPS resources are not postponed to the next valid UL subframe but further offset to another valid UL subframe.

Proposal 4: RAN2 to discuss handling of the PUR & SPS postponement including postponement to another valid UL subframe to avoid UL collision between different UEs.

# 3	Conclusion

This document has made the following observations:

Observation 1: The 8 DL subframes of a DL occasion may have insufficient capacity to carry RAR for all UEs transmitting a preamble in the preceding UL occasion.

Observation 2: The time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is about 25 ms.

Observation 3: The RAR window may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

Observation 4: The Contention Resolution timer may run primarily during the UL-DL occasion gap if the start of the timer is based on UE-eNB RTT.

Observation 5: RAN4 has agreed measurement requirements are applicable if the UE is provided with information on the active periods of the neighbour cells configured for the measurements.

Observation 6: when multiple UEs postpone their PUR/SPS resource and dynamically scheduled NPUSCH to the next valid UL subframe there may be collisions between different UEs.

And proposed the following:

Proposal 1: The start of the RAR window and Contention Resolution timer shall be aligned with the start of the DL occasions, when the UE-eNB RTT is shorter than the UL-DL gap. RAN2 to discuss the TP implementation.

Proposal 2: Severing cell to provide assistance information on the active periods of the neighbour cells configured for measurements. RAN2 to discuss the TS 36.331 TP.

Proposal 3: RAN2 to discuss reusing the AS operation from GNSS measurement gap for RRC Connected UE working in NB-IoT TDD NTN, except the neighbor cell measurement which is possible outside the serving cell’s UL-DL pair. The TP for TS 36.300 can be a starting point.

Proposal 4: RAN2 to discuss handling of the PUR & SPS postponement including postponement to another valid UL subframe to avoid UL collision between different UEs.

# Reference

- RP-243293, Iridum, “Revised WID on introduction of IoT-NTN TDD mode”, December 2024

# RAN2 Agreements:

At RAN2 #129 the following was agreed:

Agreements: 1. RAN2 will continue studying paging aspects based, on RAN1 progress 2. RAN2 assumes Kmac has to be extended (or a new parameter with higher range introduced) to address the case where the number of hops exceeds a certain limit. We continue the discussion in the next meeting to investigate if there are any other implications and in case of any decisions we send an LS to other groups if needed. 3. RAN2 confirms that idle mode eDRX is supported in IoT-NTN TDD network. 4. RAN2 thinks that a change of H-SFN duration (Option 1-1) and/or H-SFN total number (option 2-2) will impact RAN2 and SA2 specification regarding the support of idle mode eDRX in IoT-NTN TDD network and the impact should be evaluated. 5. RAN2 assumes that legacy coverage enhancement techniques (i.e. transmission with repetitions) are supported in IoT-NTN TDD system. 6. RAN2 can continue the discussion also on RAR window 7. Legacy barring bit will be used (FFS is cellBarred or cellBarred-NTN)

At RAN2 #129bis the following was agreed:

Agreements: 1. Regarding paging occasion determination, legacy NB-IoT PO determination mechanism is used. When the determined paging subframe is not a valid downlink subframe, the Paging monitoring is postponed to the nearest valid downlink subframe. 2. In IoT-NTN TDD mode, existing cell barring mechanism using the IE cellBarred-r13 and cellBarred-NTN-r17 in SIB1 is sufficient to control access to the IoT-NTN TDD cell. 3. Existing value ranges of timers in unit of PDCCH periods are reused for IoT NTN TDD (FFS on the possible clarification to take into account the impact of invalid subframes 4. When PUR resource start subframe does not align with the UL subframes in the H-SFN, UE postpones the PUR resource start subframe to the next valid UL subframe 5. When the UL SPS overlaps with non-U NB-IoT subframes UE postpones the UL SPS resource to the next valid UL subframe 6. For IoT NTN TDD mode, support k-Mac with a value range up to 1023 ms (add corresponding a restriction in the field description)

At RAN2 #130 the following was agreed:

Agreements: 1. The SI-message transmission can be postponed to the next valid D frame within the SI-Window 2. It is up to NW implementation to avoid SI-window overlap 3. SI repetitions will not overlap (in case of collision the subsequent SI repetition is postponed) 4. In IoT-NTN TDD mode, the RA-RNTI should be calculated based on the SFN of the first radio frame in which the Random-Access Preamble is transmitted (i.e. no spec change) 5. For the timer of ra-ResponseWindowSize and mac-ContentionResolutionTimer, the absolute value limitation for FDD (i.e., 10.24s) is used for IoT NTN TDD. 6. In IoT-NTN TDD mode the same formula as for RA-RNTI calculation for FDD is reused 7. No extension is needed on the value range of timer in unit of ms or s for IoT NTN TDD 8. The remaining paging repetitions falling on the invalid DL SFNs are postponed to the next valid DL SFNs. 9. It is up to network to configure the gap between two POs (i.e., parameter NB) to be sufficiently long such that it includes enough number of valid DL subframes for NumRepetitionPaging-r13 (no spec impact) 10. Introduce the following definition for IoT-NTN TDD mode in the impacted RAN2 specifications: IoT-NTN TDD mode: allows use of NB-IoT channels with TDD mode for NTN with fixed values of D non-overlapping usable contiguous DL subframes and set of U usable contiguous UL subframes separated by fixed guard period (can revisit this based on the TP being prepared by RAN1) 11. In Rel19, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode 12. In IoT_NTN_Ph3-Core, RAN2 will not work on any specific enhancements to ensure that the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode. RAN2 understands that, as part of the IoT_NTN_TDD WI, we can discuss on a case by case basis whether minor specific enhancements – not affecting other WGs - can be supported to ensure that (some of) the features being specified in IoT_NTN_Ph3-Core will also work for IoT NTN TDD mode