3GPP TSG-RAN WG2 meeting #131	R2-2505639

Bengaluru, India, Aug 25th – 29th, 2025

Source:		NEC

Agenda item:		8.7.6	XR rate control

Title:		Uplink rate control for XR

Document for:		Discussion and decision

# 1. Introduction

In the RAN2#130 meeting, agreements [1] on XR rate control were made as below:

- (MAC-05) XR rate control MAC CE can comprise multiple QoS flows (which may belong e.g. to different LCHs) unless it becomes very complicated to specify. - (MAC-04) We rule out explicit signalling of PDU session ID + QFI - (MAC-04) We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping - (RRC-1) The granularity of bit rate query prohibit timer is QoS flow. - FFS The value of the prohibit timer is the same for all flows - (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow. - (MAC-12) The index 0 in the new bit rate table does not have a special meaning, i.e., indicate 0 bit rate or make it reserved bit. - (MAC-12) The available bit rate query is not supported and the bit rate query from the UE always includes a desired bit rate. - (MAC-09) No optimization is needed to address DC case for XR rate control MAC CE - (MAC-09) We capture in stage-2 that XR rate indication is for application and not for MAC entity. - (MAC-11) RAN2 will not specify DL rate control and DL rate control query

In this contribution, we will further discuss the remaining issues of XR rate control.

# 2. Discussion

2.1 [MAC-3] Design of identifier for QoS flows indicated in the UL Rate Control MAC CE.

Regarding the indication of QFI in the new recommended bit rate MAC CE, 2 options were down-selected in RAN2 #130 meeting:

- Option 1: Explicit DRB ID + QFI;

- Option 2: Implicit, e.g. index or mapping.

First, we aim to analyse the potential implementation of Option 2. As specified in TS 38.331, the maximum number of QFIs per PDU session (i.e., maxNrofQFIs) is 64, and the maximum number of DRBs (i.e., maxDRB) is 29. This would lead to a substantial bit overhead if a static mapping between QFI and bitmap is used—specifically, 64 × 29 = 1856 bits would be required for such indication. Thus, the static mapping for Option 2 should be ruled out.

If the network dynamically configures the mapping relationship for the UE, only QoS flows that support bit rate adaptation need to be considered. However, DRBs and QoS flows may be dynamically added or released, which would force the network and UE to update the mapping relationship in real time. For instance, suppose a bitmap currently indicates 8 QoS flows; if an additional QoS flow supporting bit rate adaptation is added, both the mapping relationship and the MAC CE structure would need to be updated. As a result, the bitmap-based solution will introduce extra signaling overhead and increase implementation complexity.

Based on the above analysis, Option 2 would either result in significant bit overhead or introduce extra signaling overhead and implementation complexity. In contrast, for Option 1, since the number of QoS flows requiring simultaneous bit rate indication is limited, its bit overhead remains controllable. Therefore, we propose that RAN2 agrees to indicate “QFI + DRB ID” in the new recommended bit rate MAC CE.

Proposal 1: [MAC-3] RAN2 agrees to indicate “QFI + DRB ID” in the new recommended bit rate MAC CE.

2.2 Other potential issues.

When the indication of QoS flows supporting bit rate adaptation from CN is available, RAN node should also configure this information to UE. UE should consider only the indicated QoS flows when calculating or querying the desired recommended bit rate in the recommended bit rate query MAC CE.

Proposal 2: RAN node configures UE with information regarding which QoS flows support bit rate adaptation.

Different QoS flows may have different preferences for bit rate recommendation granularity. For example, one might favor bit rate recommendation on a per-LCH basis, while another might prefer bit rate recommendation on a per-QoS flow basis. Consequently, UE should inform the network of its preference of bit rate recommendation granularity for a given QoS flow. Based on this, the network can indicate the recommended bit rate in the UE's preferred granularity.

Proposal 3: For a QoS flow, UE informs the network of its preference for bit rate recommendation granularity, either on a per-LCH basis or on a per-QoS flow basis.

# Summary

This contribution provides our analysis on the uplink rate control for XR, and has the following proposals:

Proposal 1: [MAC-3] RAN2 agrees to indicate “QFI + DRB ID” in the new recommended bit rate MAC CE.

Proposal 2: RAN node configures UE with information regarding which QoS flows support bit rate adaptation.

Proposal 3: For a QoS flow, UE informs the network of its preference for bit rate recommendation granularity, either on a per-LCH basis or on a per-QoS flow basis.

# References

[1] Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta, 19th May – 23rd May, 2025

[2] TS 38.331, NR; NR and NG-RAN Overall Description, v18.5.0