3GPP TSG-RAN WG2 Meeting #131	R2-2505646

Bengaluru, India, 25 – 29 August, 2025

Agenda item:	8.7.4

Source:	InterDigital

Title:	Remaining open issues of DSR enhancements

WID/SID:	NR_XR_Ph3-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

The rapporteur of email discussion “[POST130][505][XR] MAC running CR and open issues (Qualcomm)” has listed the following open issues for DSR:

[MAC-1] Discuss whether a pending DSR should be canceled if the UE has delay reporting SDUs but no delay critical SDUs or all delay critical SDUs have been reported. [MAC-2] Discuss whether all pending BSRs may be cancelled if all the data eligible for inclusion in a BSR MAC CE is reported in a DSR MAC CE.

This TDoc discusses these identified open issues.

# 2	Discussion

## 2.1	Cancelling DSR [MAC-1]

Currently, the MAC entity cancels a pending DSR only based on delay-critical PDCP SDUs (TS 38.321):

### 5.4.9 Delay status reporting A PDCP SDU is considered to be associated with a DSR if it has not been transmitted in any MAC PDU and is a delay-critical PDCP SDU (as defined in TS 38.323 [4]) associated with the logical channel which triggered the DSR. A MAC PDU shall contain at most one DSR MAC CE. A MAC PDU shall not contain any DSR MAC CE if it includes all PDCP SDUs associated with all the pending DSRs. After a DSR is triggered, it is considered as pending until it is cancelled. The MAC entity shall cancel a pending DSR, when all the PDCP SDUs associated with the DSR have been discarded, or when a MAC PDU is transmitted and this MAC PDU includes a DSR MAC CE that contains the delay information of all the PDCP SDUs associated with the DSR (as described in the clause 6.1.3.72), or when a MAC PDU is transmitted and this MAC PDU includes all the PDCP SDUs associated with the DSR.

The issue in [MAC-1] proposes to deviate from this principle by allowing to report the DSR when there is still delay-reporting PDCP SDUs in the buffer that could be reported. However, this seems to go against the principle of DSR reporting where the remainingTimeThreshold is used to define the delay-critical data. Configuring a reporting threshold above the reminingTimeThreshold was just to allow NW to identify how much data there could be imminent in being delay critical when there is also delay critical data to be reported. Absence of the DSR MAC CE means also that there is no (new) delay critical data in the UE buffer which is also useful information to the NW. Hence, the DSR cancellation conditions should not be changed.

Proposal 1: [MAC-1] The DSR cancelling conditions are not changed for Multiple Entry DSR MAC CE reporting.

## 2.2	Cancelling BSR [MAC-2]

The issue in [MAC-2] proposes that UE could cancel the pending BSR(s) in case the DSR reported in a MAC PDU contains buffer information of all the data to be reported in a BSR. While this could save a few bytes of overhead created by reporting also the BSR MAC CE, that would not allow NW to know that there was a BSR trigger at the UE at the same time. Furthermore, based on only the DSR, the NW would not know whether the UE has more data beyond the configured reporting thresholds unless the BSR is reported in this case. Hence, the BSR cancelling conditions should not be changed.

Proposal 2: [MAC-2] The BSR cancelling conditions are not changed based on the reported DSR.

# 3	Conclusion

This document has made the following proposals:

Proposal 1: [MAC-1] The DSR cancelling conditions are not changed for Multiple Entry DSR MAC CE reporting.

Proposal 2: [MAC-2] The BSR cancelling conditions are not changed based on the reported DSR.