3GPP TSG-RAN WG2 Meeting #131	R2-2505659

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item: 	8.7.5

Source:		Sony, Canon

Title:	 Timely retransmissions for RLC AM

Document for:	Discussion & Decision

# Introduction

RAN2_128 agreements for timely retransmissions of RLC AM enhancements

Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement, and NW can configure either or both of them.

RAN2_129 agreements for Autonomous retransmissions and polling enhancements

- Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC

- Only a single autonomous retransmission will be triggered per RLC SDU.

- There is no dynamic activation/deactivation of the autonomous retransmission mechanism.

- We have separate thresholds for autonomous reTx and for polling

RAN2_129bis agreements for Autonomous retransmissions and polling enhancements

- When the t-RxDiscard expires, the expiration of t-RxDiscard triggers an SR. FFS whether this is just usual SR or some changes are needed, or if UE implementation can decide (to be discussed during CR review)

- For autonomous retransmission and polling, the remaining time is determined based on discardTimer at PDCP. FFS whether/what additional conditions are needed to prevent too early and/or unnecessary retransmission

- Autonomous retransmission is triggered for an RLC SDU (segment) provided that the original RLC SDU has been submitted to lower layers.

- Autonomous retransmission is not triggered if the RLC SDU (segment) is already pending for retransmission. FFS specifications impact.

In this contribution, we discuss on how to reduce the time gap between the initial transmission and the re-transmissions for timely retransmissions in RLC AM.

# Timely retransmissions for RLC AM

In general, for RLC polling enhancements, the delay gap between the initial transmission and the re-transmission of the RLC SDU should be reduced. This means that both the frequency of RLC polling from Tx side and status report (SR) from Rx side must be fast enough.

From Rx side, when polling information is received, the speed of the status report (SR) depends on number of parameters: t-Reassembly, t-StatusProhibit timers. While both timers can be set to zero, in our view, it is not desirable to set t-Reassembly to zero because it will trigger premature NACKs to be reported in the status report, and completely resulting degraded radio link performance.

On one hand, the t-Reassembly timer must be set to an appropriate value by considering the time it takes to complete a number of HARQ retransmissions in the lower layers. On the other hand, the t-Reassembly timer creates long gap between the first ARQ transmission and retransmissions where it becomes more detrimental when the remaining lifetime of the SDU is very short.

One solution is to keep the t-Reassembly timer as in the current specification, but when generating the status report, the t-Reassembly timer is bypassed so that the receiver can immediately report the status of SDU or segments of SDU (i.e., one or more PDUs) that are still stuck in the lower layer HARQ retransmissions whilst t-Reassembly timer is still running (i.e.., the HARQ processing can still be done).

The point is to speed up the status report (SR), in a sense that at the point of reporting, if some SDUs are already confirmed to be ACK or NACK, they are reported as in legacy specification, however, if some SDUs or segments of SDU are still stuck in the HARQ retransmissions (e.g., t-Reassembly timer is still running), they are reported as UNKNOWN status in the report. In this case, after receiving the status report, if the remaining time is short, the Tx side can selectively retransmit (ARQ level) for these SDUs or segments of SDU with UNKNOWN status, hence reducing the gap between the first ARQ transmission and retransmissions.

Note that the t-Reassembly timer is still needed to oversee the HARQ retransmissions for the missing SDU, as it ensures that a sufficient time is given to the HARQ retransmissions to improve the radio link performance, so that the final result will be either late ACK or firm NACK which can be reported in the next status report.

An example is given on Figure 1 where there are three SDUs in the status report: SDU-5, SDU-6 and SDU-7.

In this example, at the receiver, at the time of reporting, the last SDU-7 is successfully received, SDU-5 has one segment/PDU that has failed (i.e. firm NACK), and SDU-6 has two segments/PDUs that are still under HARQ retransmissions at the time of reporting. This means that after SDU-7 is received successfully, the t-Reassembly timer is started because the middle SDU-6 is missing.

As proposed above, if receiver receives a polling information, the receiver should bypass the t-Reassembly timer, and provide status report including the status of SDU(s) that is stuck in the HARQ retransmissions at the time of reporting. In this case, SDU-6 is reported as UNKNOWN status by indicating/redefining the corresponding reserved bit as 1 (R = 1).

In detail as shown on Figure 1, the status report contains D/C = 0, CPT=0, ACK_SN = 8, blue E1 = 1 and R = 0. The following is NACK_SN = 5 to indicate the first lost PDU. In this case, E1 = 1 (to indicate there is another lost PDU following), E2 = 1, and E3 = 0 and R = 0. As E2 was set to 1, the lost PDUs for SDU-5 must be identified as SOstart = 0x0100 and SOend = 0x01FF (the last byte of the lost PDU). The following is UNKNOWN_SN = 6 to indicate that the some PDUs are still under HARQ retransmissions. In this case, E1 = 0, E2 = 1, and E3 = 0 and R = 1. As E2 was set to 1, PDUs that are under HARQ retransmissions for SDU-6 must be identified as SOstart = 0x0000 and SOend = 0x01FF. Note that in this example, during reporting, the t-Reassembly timer has been started due to SDU-7 is received successfully but segments of the middle SDU-6 are missing.

Figure 1. Example of STATUS REPORT (SR) with SDUs where some SDU segments (i.e., one or more PDUs) are still stuck in the HARQ retransmissions and given UNKNOWN status in the report.

Furthermore, we believe that the value to set for t-StatusProhibit timer depends on operator’s policy in order to avoid excessive SR reporting. In this case, it is also possible that when Rx side receives polling information (i.e., conditionally) the t-StatusProhibit timer can also be bypassed.

Proposal 1: After Rx side receives polling information, the Rx side should bypass/ignore the t-Reassembly timer for any SDU or segments of SDU if the t-Reassembly timer is running when generating status report.

Proposal 2: If proposal 1 is agreed, for any SDU or segments of SDU with bypassed t-Reassembly timer or still stuck in the lower layer retransmissions should be reported as UNKNOWN status in the status report (SR) by indicating/ redefining the corresponding reserved bit as 1 (R = 1).

Proposal 3: RAN2 to discuss when Rx side receives polling information whether the t-StatusProhibit timer can also be bypassed.

# Conclusion

In this contribution, we have provided a discussion on how to reduce the time gap between the initial transmission and the re-transmissions for timely retransmissions in RLC AM, and we have the following proposals.

Proposal 1: After Rx side receives polling information, the Rx side should bypass/ignore the t-Reassembly timer for any SDU or segments of SDU if the t-Reassembly timer is running when generating status report.

Proposal 2: If proposal 1 is agreed, for any SDU or segments of SDU with bypassed t-Reassembly timer or still stuck in the lower layer retransmissions should be reported as UNKNOWN status in the status report (SR) by indicating/ redefining the corresponding reserved bit as 1 (R = 1).

# References

- RP-243318, “Revised WID on XR (eXtended Reality) for NR Phase 3,” RAN#106.