3GPP TSG-RAN WG2 Meeting #131					           R2-2505672

Bangalore, India, 25 - 29 August, 2025

Agenda Item:		8.3.2.2

Source:			Huawei, HiSilicon

Title:			Functionality management for measurement event prediction

Document for:		Discussion and Decision

# 1   Introduction

In RAN2#129bis meeting, agreements related to functionality management were reached as follows [1]:

Agreements - Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology). Agreements - The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain • PW length (for temporal case A) • Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B ) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI • Measured and predicted frequency (For inter-freq domain prediction) Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them. Agreements 1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

In this Tdoc, we will discuss some remaining issues on functionality management for RRM measurement event prediction. The document is divided into two parts which focus on applicability reporting and performance monitoring issues (section 2.1), and reporting related issues (section 2.2).

# 2   Discussion

## 2.1 Applicability reporting and performance monitoring

Direct vs indirect measurement event prediction

RAN2 agreed that “Model related choices (i.e., cluster-based vs. cell-based, L1 filtering, RRM sub-use cases, OW length, direct vs. indirect) can be up to UE implementation unless a requirement is identified to specify them.” Since this agreement concluded that whether the UE uses direct or indirect event prediction method is up to UE implementation, our understanding is that this means RAN2 will specify only a single event prediction framework which will not distinguish indirect and direct methods, i.e. there will be no method specific configurations or discussions in the normative phase. Since we find this as an important guideline for the WI phase, we propose to agree the following:

Proposal 1: Only a single event prediction framework should be developed which will not distinguish indirect and direct prediction methods, i.e. method specific configurations/procedures should not be considered.

Performance metrics

In RAN2#130 meeting, it was agreed that RSRP differences can be used as the performance metric for RRM measurement prediction monitoring, but it remains to be decided what metric should be used for event prediction. During the simulation work for event prediction use case, RAN2 analyzed various metrics, including RSRP difference as well as F1 score and system level KPIs. Out of these metrics, it seems only RSRP difference can be used in practice. This is because F1 score and SLS metrics are not available in real time and by nature they need to be calculated over a long period of time. Taking an example, the F1 score calculation should be performed using at least several instances of predicted measurement event, and the system level KPIs calculation should be performed at least over several HO executions (and ideally it should be dozens of events/HOs). Therefore, if such metrics were used alone, i.e., without RSRP difference metric, the UE or the NW may still perform prediction for a very long time before realizing that the prediction accuracy is actually unacceptable. Furthermore, it does not seem feasible for F1 score and system level KPIs to be calculated at the UE side. Similarly as today, system KPIs such as HOF rate can be monitored by the network, e.g. in order to determine a long-term effect of using predictions on the system performance.

Observation 1: It is not feasible for the UE to measure F1 score and system level KPIs.

Observation 2: As today, such metrics should be monitored by the network on a long-term basis, but are not suitable for AIML functionality management.

Hence, the only feasible metric identified so far remains to be RSRP difference. It is worth noting that RSRP needs to be predicted by the UE anyway as the UE is required to provide RSRP results of the reported cells when it sends the measurement report to the network.

Observation 3: The UE needs to include RSRP results of the reported cells when it sends the measurement report to the network, so it needs to be able to predict cells’ RSRP when it predicts the measurement event.

Hence, we propose to use RSRP difference as the performance monitoring metric for both RRM measurement prediction and measurement event prediction. This would also allow to have a unified monitoring framework for both use cases.

Proposal 2: F1 score and system level KPIs are not used for instantaneous AIML model performance monitoring.

Proposal 3: RSRP difference is used as the performance monitoring metric for measurement event prediction.

## 2.2 Reporting aspects

RAN2 agreed the following:

5. For RRM prediction, UE sided model, the UE can be configured with periodic or event triggered reporting of predicted and/or actual RRM measurements. FFS details 6. For event prediction, UE-sided model, the UE can be configured with event-triggered reporting based on prediction and/or actual measurements. FFS details

As mentioned above, when measurement event triggering conditions are met, the UE sends a report containing cell and/or beam measurements. For periodic reporting of measurements, the UE reports predicted measurements according to the configured periodicity. However, when it comes to event triggered RRM measurement reporting, the question arises on which predicted measurements the UE should report. In legacy event triggered measurement reporting, the measurement result of only one moment is reported, i.e., the time when the TTT ends. However, for temporal domain prediction, the PW may be longer than TTT. So, predicted measurement result corresponding to the time when the PW ends may also be helpful for HO determination. Thus, it is suggested RAN2 to consider reporting of other measurement result(s), besides the one corresponding to the time when TTT ends, e.g., predicted measurement result corresponding to the time when PW ends. This is applicable to both event triggered reporting of predicted RRM measurements and measurement reporting during predicted event reporting.

Proposal 4: It should be possible for the UE to report predicted measurement results corresponding to multiple time instances, e.g. on top of the result corresponding to TTT expiry time, the one related to PW end.

# 3   Summary

In the paper, the functionality management for RRM measurement event prediction is discussed, and the following observations and proposals are provided:

Applicability reporting

Proposal 1: Only a single event prediction framework should be developed which will not distinguish indirect and direct prediction methods, i.e. method specific configurations/procedures should not be considered.

Performance monitoring

Observation 1: It is not feasible for the UE to measure F1 score and system level KPIs.

Observation 2: As today, such metrics should be monitored by the network on a long-term basis, but are not suitable for AIML functionality management.

Observation 3: The UE needs to include RSRP results of the reported cells when it sends the measurement report to the network, so it needs to be able to predict cells’ RSRP when it predicts the measurement event.

Proposal 2: F1 score and system level KPIs are not used for instantaneous AIML model performance monitoring.

Proposal 3: RSRP difference is used as the performance monitoring metric for measurement event prediction.

Reporting aspects

Proposal 4: It should be possible for the UE to report predicted measurement results corresponding to multiple time instances, e.g. on top of the result corresponding to TTT expiry time, the one related to PW end.

# 4   References

- Report of 3GPP TSG RAN WG2 meeting #130, St Julian's, Malta

- R2-2505671 Functionality management for RRM measurement prediction, Huawei, HiSilicon