3GPP TSG-RAN WG2 Meeting#131	R2-2505684

Bangalore, India Aug. 25th-29th, 2025

Agenda item:		8.4.4

Source:		Lenovo

Title:		 Open issues on LP-WUS in RRC connected mode

Document for:		Discussion and Decision

- Introduction

Currently, the UE’s Main Radio (MR) needs to periodically monitor PDCCH once per DRX cycle, which dominates the power consumption in periods with no signaling or data traffic. If the UE’s MR can wake up only when they are triggered (e.g. paging), power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it needs to monitor PDCCH [1]. The following agreements were made at the previous meetings:

Agreements at RAN2 #127: - For option 1-2, - After LP-WUS triggers the UE to perform PDCCH monitoring, the UE starts one timer. When the timer is running, the UE monitors PDCCH. FFS on the timer (e.g., newly defined timer or legacy timer.) - The timer is started at a time offset after receiving the LP-WUS indication for PDCCH monitoring. The range of time offset is left for RAN1. Agreements at RAN2 #127bis: - For option 1-2, LP-WUS monitoring is performed at least outside legacy C-DRX Active Time. FFS if the legacy drx-onDurationTimer is started or not if the new timer is configured in Option 1-2. - In option 1-2, a new timer triggered by LP-WUS is introduced. When this new timer is running, UE is in C-DRX active time. When UE is not in C-DRX active time, UE goes back to LP-WUS monitoring. - When UE is in C-DRX active time, UE PDCCH monitoring behaviors related to other legacy DRX timers (except for drx-onDurationTimer) are not affected. Agreements at RAN2#128 meeting: - The drx-onDurationTimer is not started with Option 1-2 LP-WUS. - For Option 1-2, network can configure whether UE reports periodic CSI/L1-RSRP during the time given by the configured drx-onDurationTimer, for the case when UE is outside C-DRX active time. - For option 1-2, if UE receives DRX command MAC CE or Long DRX command MAC CE, UE stops the new timer triggered by LP-WUS. - Don’t support Option 1-1 and Option 1-2 simultaneously configured for the same UE. Agreements at RAN2#129 meeting: - For Option 1-1, UE monitors LP-WUS outside C-DRX active time at least when long DRX cycle is used. FFS whether short DRX cycle is used. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-1 to stop drx-onDurationTimer and drx-InactivityTimer. - RAN2 confirm the (Long) DRX command MAC CE can be used with option 1-2 to stop the new timer and drx-InactivityTimer. Agreements at RAN2#129bis meeting: - For Option 1-1, the UE does not monitor LP-WUS when Short DRX cycle is used. - Working assumption: For option 1-2, it is up to network configuring short DRX cycle with LP-WUS. The UE monitors LP-WUS outside the Active Time regardless of if Short DRX cycle or Long DRX cycle is used. - Send an LS to RAN1 to inform the above conclusions, can revisit if needed based on RAN1 feedback. - Working assumption for the case of potential collision (if any): In Option 1-1, when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer (as if LP-WUS was detected). FFS for Option 1-2. - Send LS to inform this working assumption, can also ask a) what are the cases when UE cannot monitor LP-WUS, b) whether UE can monitor LR and MR simultaneously. - If configured, the UE can signal a preferred time offset via UAI signalling. - Ask RAN1 for further information regarding their conclusions. - FFS whether/how to support LP-WUS (including Option 1-1 and 1-2) and dual DRX group. - For NR-DC, the LP-WUS can be configured to be monitored at least on the PCell and PSCell. Wait for RAN1 progress on whether to allow LP-WUS configuration and monitoring on other Cells. - For NR-DC, the LP-WUS in MCG and SCG can be configured independently. - Apart from NR-DC, LP-WUS can also be supported in NE-DC, EN-DC, NGEN-DC. And proposal 1 and 2 also apply to NE-DC, EN-DC, NGEN-DC. Agreements at RAN2#130 meeting: - Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e., the UE monitors LP-WUS before the on-duration occasion or periodically outside Active Time. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups. - Check whether we need to capture in MAC that UE is not expected to monitor LP-WUS if not in Cell DTX active period.

This contribution looks at the possible RRC Connected Mode procedures for an LP-WUR enabled UE based on listed open issues.

- Discussion

  - UE operation for potential collision

- In RAN2#129bis meeting, it’s assumed that when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer for option 1-1, and a related LS is sent to RAN1. Based on the RAN1#121 meeting agreements and the reply LS to RAN2 as below, UE is not able to operate LR and MR simultaneously in Rel-19. Then the working assumption can be confirmed since LR is not able to monitor LP-WUS when MR is on.

RAN1#121 Meeting Agreements: - As the initial reply to RAN2 LS in R1-2503616, RAN1 confirms that at least the collision with Active Time, measurement gap, and RAR window monitoring for BFR can be considered for the cases/scenarios on when the UE is not able to monitor LP-WUS. - As the reply to RAN2 LS in R1-2503616, RAN1 assumes that UE is not able to operate LR and MR simultaneously in Rel-19. RAN1 understanding is that the terminology of LR and MR operations are for discussion purpose and will not be specified - LR operation is the UE operation for LP-WUS monitoring - MR operation is the UE operation for all other NR signals/channels transmissions/receptions in connected mode

- Proposal 1: (MAC-2) RAN2 to confirm the working assumption: when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer for Option 1-1.

- In last meeting, it’s still FFS whether the similar solution as option 1-1 can be applied to Option 1-2. From our point of view, similar solution can be applied to Option 1-2 to avoid UE missing potential scheduling. Some companies may argue that this will cause frequent start of lpwus-PDCCHMonitoringTimer due to the frequent LO configuration, thus reducing power saving gain. However, it’s not preferred to compromising scheduling performance to obtain power saving gain. In addition, in option 1-2, lpwus-PDCCHMonitoringTimer may be shorter than LP-WUS monitoring periodicity, and there will not be frequent collisions. In this case, it’s beneficial to start lpwus-PDCCHMonitoringTimer in collision case to guarantee the scheduling latency and robustness.

- Proposal 2: (MAC-2) Similar principle can be applied to option 1-2: when the UE is not able to monitor the LP-WUS occasion(s), the UE should start the lpwus-PDCCHMonitoringTimer.

  - Support LP-WUS with dual DRX group

- Serving cells of a MAC entity may be configured by RRC with two DRX groups. Each DRX group has separate DRX parameters to be applied for FR1 and FR2. Then RAN2 needs to identify whether dual DRX group can be configured along with the LP-WUS configuration. In last RAN2#130 meeting, following work assumption has been made on LP-WUS:

- Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

- It’s assumed that LP-WUS can be configured on the PCell with secondary DRX for both option 1-1 and 1-2. Generally, we think this working assumption can be confirmed since it is benefit on the power saving.

- Proposal 3: (RRC-3/MAC-1) RAN2 to confirm following working assumption on support of LP-WUS with dual DRX group.

- Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

- More specifically, regarding to how the two LP-WUS options may be configured if it is supported with secondary DRX simultaneously, we share the view that the same LP-WUS Configuration (i.e., Option 1-1 or Option 1-2) should be used across both DRX groups. That is, if Option 1-1 is configured on the PCell of default DRX group, then the secondary DRX group shall also follow Option 1-1. Similarly, if Option 1-2 is configured on the PCell of default DRX group, then the secondary DRX group shall also follow Option 1-2. With Option 1-1, this behaviour is quite straightforward and the secondary DRX group may be configured with shorter values of drx-onDuration and drx-Inactivity timers, as in legacy. With Option 1-2, the lpwus_PDCCHMonitoringTimer can either be configured with the same value in the secondary DRX group or it can be configured with a shorter timer value as compared to the default DRX group. Since Option 1-2 was designed to have a shorter latency benefit, we think it would be beneficial to have a shorter lpwus_PDCCHMonitoringTimer in the secondary DRX group for a better power saving gain. The cycle length and start of DRX in both DRX groups can be the same, similar to legacy behaviour. So we think, in principle the design of Option 1-2 with secondary DRX group (if supported) can reuse the legacy design as baseline.

- Proposal 4: (RRC-3/MAC-1): The default DRX group and secondary DRX groups are both configured with the same LP-WUS DRX Option (i.e., either Option 1-1 or Option 1-2).

- Proposal 5: (RRC-3/MAC-1): If secondary DRX group is configured with Option1-2 of LP-WUS, the secondary DRX group is configured with a shorter lpwus_PDCCHMonitoringTimer than default DRX group.

  - BWP switching timer

- In legacy, gNB can dynamically indicate UE to switch BWP through DCI. But if the UE has no data transmission for a long time, the bwp-InactivityTimer will trigger the UE to fallback to the default or initial BWP (usually configured with a smaller bandwidth), thereby reducing the UE's power consumption and RF complexity. From UE perspective, UE may star or re-start the bwp-InactivityTimer when receiving scheduling information from gNB (e.g., DCI format 0_1/1_1). An open issue of whether to (re)start the bwp-InactivityTimer when UE receives LP-WUS is raised during the open issue list. In legacy DCP design, the bwp-InactivityTimer is not (re)started when receiving legacy WUS. Generally, we think  same principle can also apply to LP-WUS, i.e., there’s no direct relationship between bwp-InactivityTimer start and LP-WUS reception. In the case that bwp-InactivityTimer expires after receiving LP-WUS and UE fallbacks to the default BWP, gNB can send explicit signalling to indicate UE to switch BWP when UE is in active time if needed. That is to say, the legacy mechanism is workable. Therefore, we prefer that UE does not start or re-start the bwp-InactivityTimer when receiving the LP-WUS.

- Proposal 6: (MAC-X1) UE does not start or re-start the bwp-InactivityTimer when receiving the LP-WUS.

  - Multiple LP-WUS cycles

- During the MAC Open issue discussion, another open issue raised is about whether to consider multiple LP-WUS cycles is proposed, e.g., long LP-WUS cycle and short LP-WUS cycle. From our side, it’s not essential to introduce multiple LP-WUS cycles in the last meeting, which just follow gNB’s configuration on the short or long LP-WUS cycle is workable and enough, considering the motivation is not clear and may cause additional spec impact if multiple LP-WUS cycles are supported. we suggested not to support multiple LP-WUS cycles in Rel-19.

- Proposal 7: (MAC-X2) It is suggested not to support multiple LP-WUS cycles in Rel-19.

- Conclusion

This document discusses some procedures for a LR capable UE in the RRC Connected Mode. Following are the proposals and observations made in this document:

- Proposal 1: (MAC-2) RAN2 to confirm the working assumption: when the UE is not able to monitor the LP-WUS occasion(s) the UE should start the drx-OnDurationTimer for Option 1-1.

- Proposal 2: (MAC-2) Similar principle can be applied to option 1-2: when the UE is not able to monitor the LP-WUS occasion(s), the UE should start the lpwus-PDCCHMonitoringTimer.

- Proposal 3: (RRC-3/MAC-1) RAN2 to confirm following working assumption on support of LP-WUS with dual DRX group.

  - Working assumption: LP-WUS can be configured on the PCell with secondary DRX. LP-WUS with secondary DRX is supported with option 1-1 and 1-2, i.e. the UE monitors LP-WUS before the on-duration occasion or periodically outside ActiveTime. When LP-WUS is detected, then UE starts the drx-onDurationTimer (with option 1-1) or the lpwus-PDCCHMonitoringTimer (with option 1-2) in both DRX groups.

- Proposal 4: (RRC-3/MAC-1) The default DRX group and secondary DRX groups are both configured with the same LP-WUS DRX Option (i.e., either Option 1-1 or Option 1-2).

- Proposal 5: (RRC-3/MAC-1) If secondary DRX group is configured with Option1-2 of LP-WUS, the secondary DRX group is configured with a shorter lpwus_PDCCHMonitoringTimer than default DRX group.

- Proposal 6: (MAC-X1) UE does not start or re-start the bwp-InactivityTimer when receiving the LP-WUS.

- Proposal 7: (MAC-X2) It is suggested not to support multiple LP-WUS cycles in Rel-19.

- References

[1] 3GPP TR 38.869 v0.2.0: Study on low-power wake up signal and receiver for NR (Release 18)