3GPP TSG-RAN WG2 #131	R2-2505687

Bangaluru, India, August 25th – 29th, 2025

Agenda item:	   8.1.3

Source:		Lenovo

Title:	Left issues on NW side data collection

Document for:		Discussion and Decision

# Introduction

In this paper, we discuss issues related to the training data collection for NW sided model for beam management and positioning use cases.

# Discussion

## Open issue RRC-24: Where to include the logging configuration from NW to UE

Issue description: It is FFS whether the logging configuration is included in the CSI framework (whether in CSI-ReportConfig or directly under CSI-MeasConfig) or at L3. This issue was discussed in RAN2#129bis and the following outcome was captured: Next meeting proponents should work together and bring complete proposals to show specification impact and consider future use cases.. The issue is captured as an editor’s note in the running CR, in clause 6.3.2. Configuration details of events for event-based logging configuration, e.g. whether we reuse configuration in reportConfig, can be included. Proposed resolution: This open issue is addressed in the email discussion [POST130][031][AI PHY] NW side data collection (Ericsson/ZTE). Companies can also discuss relevant aspects in their contributions.

One aspect raised by some company is that once the memory issue is resolve, i.e., no longer full, will UE autonomously resume the data logging. Following the previous RAN2 discussion, we understand that when the memory reserved for storing logged data becomes full, the UE stops measurement and logging for data collection. After UE reports the logged data to the network, UE can restart measurement and logging for data collection. And it’s better to be clarified in the specification.

UE can autonomously restart measurement and logging for data collection if the memory is no longer full, e.g., upon reporting the logged data to NW.

## Open issue RRC-31: The release of logged AIML data in UE’s buffer

Open issue RRC-31: The release of logged AIML data in UE’s buffer Issue description: RAN2 needs to discuss and capture in the spec when the logged data for AIML in UE buffer will be released. In addition to the retain/release of logged data during HO covered in Open issue RRC-7, the following can be also considered: - Power off or deregistration - 48 hrs after the release of logged measurement configuration - Explicit indication from the serving gNB. Proposed resolution: It is suggested that companies provide contributions to resolve this issue later.

It has not been discussed yet is the release of the logged data in the UE buffer. Since it could be possible that the logged data is retained in UE buffer, while the gNB does not fetch for a long time.

#### 5.5a.1.3 Reception of the LoggedMeasurementConfiguration by the UE Upon receiving the LoggedMeasurementConfiguration message the UE shall: 1> discard the logged measurement configuration as well as the logged measurement information as specified in 5.5a.2; 1> store the received loggingDuration, reportType and areaConfiguration, if included, in VarLogMeasConfig; #### 5.5a.1.4 T330 expiry Upon expiry of T330 the UE shall: 1> release VarLogMeasConfig; The UE is allowed to discard stored logged measurements, i.e. to release VarLogMeasReport, 48 hours after T330 expiry. ### 5.5a.2 Release of Logged Measurement Configuration #### 5.5a.2.1 General The purpose of this procedure is to release the logged measurement configuration as well as the logged measurement information. #### 5.5a.2.2 Initiation The UE shall initiate the procedure upon receiving a logged measurement configuration in same or another RAT. The UE shall also initiate the procedure upon power off or upon deregistration. The UE shall: 1> stop timer T330, if running; 1> if stored, discard the logged measurement configuration as well as the logged measurement information, i.e. release the UE variables VarLogMeasConfig and VarLogMeasReport.

In legacy logged MDT, the logged data will be released upon

- New logged measurement configuration is received

- Release of logged measurement configuration

- Power off or deregistration

- 48 hrs after logging duration is reached

For the logged AI/ML data in RRC connected state, since multiple data logging configurations (e.g., for different use cases) can be configured at UE, the receiving or release of logged measurement configuration is no longer a proper trigger of logged data release.

RAN2#128 Agreement: Duration is not supported

Also, since RAN2 agreed that the logging duration is not supported, then it is not a relevant factor in this case .

On the other hand, considering the handover scenario, where the UE connects to a new serving gNB with logged data retained before handover. The new serving gNB has to always fetch the retained logged data (imagining UE buffer is full) before configuring UE with any new data logging configuration. Considering the consumer of the logged data could be the old gNB, and it is after all the new serving gNB’s decision whether to fetch the logged data retained in UE’s buffer, it could be beneficial and a clean approach to introduce a dedicated indication from the serving gNB to discard (some of) the logged data in UE’s buffer.

The logged data in UE’s buffer will be released upon

  - Power off or deregistration

  - 48 hrs after the release of logged measurement configuration

  - Explicit indication from the serving gNB (while UE stays in RRC connected state)

## Open issue RRC-33: Whether separate user consent for gNB centric training is needed

Issue description: It can be discussed whether a separate user consent is required for gNB-centric training, as it may differ from the conventional MDT user consent. To address the potential impact on gNB and OAM regarding data collection, it may be necessary to send an LS to RAN3 and SA5, including agreements related to logging. From the rapporteur’s perspective, this issue is outside the scope of RAN2 and it is unclear why separate user consent is needed, given that the gNB can already obtain the same data with legacy reporting (without logging), for the beam management use case considered in Rel-19.

In immediate MDT framework, only UEs that have given MDT user consent can be configured to collect and report data for MDT purposes, which may not be enough to support the AIML driven data collection which can be initiated by OAM or gNB considering:

- MDT and AIML driven are considered as two kinds of data usage purposes. It may not be a good idea to directly reuse the MDT user consent (that UE may report its location) for the AIML driven data collection (UE will not report its location for AI based beam prediction use case).

- In case of gNB initiated training data collection, the MDT user consent may not be provided by the core network if OAM/CN is not prepared to perform MDT data collection.

Existing MDT user consent checking mechanism may not be enough to serve the purpose of data collection for beam prediction initiated by gNB or OAM.

In the meanwhile, in SA2 AIML related discussion, user consent for analytics (e.g., model inference) or ML model training is specified and stored in UDM, which is different than the MDT user consent. NWDAF will check the user consent before collecting or using the UE data for the AIML purpose.

We believe similar/same user consent for AIML purpose different than MDT user consent can be introduced and should be checked by the gNB before configuring the UE data collection either triggered by OAM or by gNB itself.

### TS 23.288 ### User consent for analytics Depending on local policy or regulations, to protect the privacy of user data, the data collection, ML model training and analytics generation for a SUPI or GPSI, Internal or External_Group_Id or "any UE" may be subject to user consent bound to a purpose, such as analytics or ML model training. The user consent is subscription information stored in the UDM, which includes: a) whether the user authorizes the collection and usage of its data for a particular purpose; b) the purpose for data collection, e.g. analytics or model training. The NWDAF retrieves the user consent to data collection and usage from UDM for a user, i.e. SUPI prior to collecting user data from an NF as described in clause 6.2.2 and from a DCCF as described in clause 6.2.6.

User consent has been defined for analytics or model training, which will be checked by the NWDAF before collecting or using UE side data.

RAN2 sends LS to SA3/SA5 consulting if existing MDT user consent can be reused or AIML related user consent shall be provided for the gNB to initiated gNB-centric data collection.

## Open issue RRC-34: Whether enhancements for NW-side data collection are per use case or common for all AI-related use cases

Issue description: It can be discussed whether the enhancements for NW-side data collection are per use case or common for all AI-related use cases, e.g., AS layer memory, logged data availability, low-power state indication. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

The issue of AS layer memory per use case is discussed in the email discussion [POST130][038][AI PHY] UE capabilities, thus not repeated here.

In the last RAN2 meeting, it was agreed that the data is collected on per data logging configuration basis and UE indicates the data logging configuration ID. Following the agreement, it seems straightforward to support the data request/report per logging configuration.

RAN2#130 Agreement: Data is collected on per data logging configuration basis and UE indicates data logging configuration ID. An indication of the “gap” is needed. “Gap” is time interval larger than the configured logging periodicity. FFS if timestamp and relative time stamp for each group is needed per “group”.

In particular, when UE indicates data availability, UE can also indicate the list of data logging configuration ID  that has data available. Then, when gNB requests logged data from UE via UEInformationRequest message, gNB can also include the interested data logging configuration ID.

When UE indicates the data availability to NW, UE also indicate the list of data logging configuration ID for which data is available.

When gNB requests logged data from UE via UEInformationRequest message, gNB can also include the interested data logging configuration ID.

On the other hand, the cause value shall remain generic for all data logging, i.e., threshold, power state.

The cause value (i.e., buffer full, buffer threshold reached, low power state) of data availability indication remain generic for all data logging.

Besides, during the handover, the source gNB may also indicate to the target gNB if the logged data available in UE buffer should be reported from target gNB to the source gNB after handover. Following the same design, the retaining/releasing of available data in UE buffer during handover should also be handled in a per data logging configuration manner. In particular, the source gNB may indicate if UE should retain the available data in UE buffer for a particular data logging configuration id in the HandoverPreparationInformation. And the target gNB should include the indication in the handover command correspondingly.

The source gNB may indicate to the target gNB if UE should retain the available data in UE buffer for a particular data logging configuration during handover. And the target gNB shall include the indication in the handover command correspondingly.

## Open issue RRC-36: How data is forwarded to OAM or source gNB after HO

Issue description: RAN2 agreed that target gNB can fetch data collected in the source gNB after HO. We need to decide how this data is forwarded to OAM or source gNB, e.g. via inter-node RRC message or in some other way. RAN3 involvement may be needed. Proposed resolution: It is suggested that companies provide contributions to resolve the issue.

In case of gNB-centric NW side data collection, it’s clear that the data fetched after HO should be forwarded to the source gNB, just it needs to be decided in which message, e.g., in an inter-node RRC message or a Xn message with RRC IE container. In our understanding, given there is no existing inter-node RRC message that serve the purpose, the most straightforward way is to transfer the logged data as a RRC IE container (e.g., CSI-LogMeasReport IE) from target gNB to source gNB conveyed in a Xn message, similar as how SuccessHO-Report is transmitted over Xn interface. The exact Xn message used to transfer the data is upon RAN3’s decision.

For gNB-centric NW side data collection, after handover, the target gNB may transfer the logged data from UE to source gNB via a Xn message conveying the corresponding RRC IE container (e.g., CSI-LogMeasReport IE).

It is upon RAN3’s discussion that which Xn message should be for target gNB to transfer the logged data from UE to source gNB after handover.

In case of OAM-centric NW side data collection, since OAM is the consumer of the data, there are two approaches for target gNB to transfer the logged data after handover:

- Approach#1: target gNB transfer the logged data from UE to source gNB, then source gNB further transfer to OAM

- Approach#2: target gNB transfer the logged data directly to OAM.

From specification impact point of view, Approach#1 does not have additional RAN2/RAN3 impact, while it will consume additional resources over Xn interface. Approach#2 can avoid the data transmission over Xn interface, while it would require the corresponding TraceReference related information being exchanged over RRC.

At this late stage, we suggest RAN2 assumes only Approach#1 is available, it can be upon RAN3’s further discussion if Approach#2 should be supported as well.

For OAM-centric NW side data collection, RAN2 assumes that after handover target gNB transfers the logged data from UE to source gNB, then source gNB further transfer to OAM.

It is upon RAN3’s discussion if it should be supported as well that gNB transfers the logged data from UE directly to OAM after handover.

## Open Issue RRC-x: LPP message over SRBx

SRBx is for RRC messages which include logged measurement information for network data collection, all using DCCH logical channel. SRBx has a lower priority than SRB1 and can only be configured by the network after AS security activation.

In the current CR, the new SRBx is only used to transfer RRC messages that restrict the use of the new SRBx for AI based beam management and CSI prediction use case only. Since SRBx is introduced for low priority training data collection, we understand it applies to the training data collection for LMF as well. The transmission of LPP message containing training data collection via SRBx should also be allowed.

The transfer of NAS (containing LPP message) over SRBx for data collection for AI based positioning is also supported.

# 3	Conclusion

Based on the discussion above, we observe:

Observation 1	Existing MDT user consent checking mechanism may not be enough to serve the purpose of data collection for beam prediction initiated by gNB or OAM.

Observation 2	User consent has been defined for analytics or model training, which will be checked by the NWDAF before collecting or using UE side data.

Based on the discussion above, we propose:

Proposal 1	UE can autonomously restart measurement and logging for data collection if the memory is no longer full, e.g., upon reporting the logged data to NW.

Proposal 2	The logged data in UE’s buffer will be released upon

a.	Power off or deregistration

b.	48 hrs after the release of logged measurement configuration

c.	Explicit indication from the serving gNB (while UE stays in RRC connected state)

Proposal 3	RAN2 sends LS to SA3/SA5 consulting if existing MDT user consent can be reused or AIML related user consent shall be provided for the gNB to initiated gNB-centric data collection.

Proposal 4	When UE indicates the data availability to NW, UE also indicate the list of data logging configuration ID for which data is available.

Proposal 5	When gNB requests logged data from UE via UEInformationRequest message, gNB can also include the interested data logging configuration ID.

Proposal 6	The cause value (i.e., buffer full, buffer threshold reached, low power state) of data availability indication remain generic for all data logging.

Proposal 7	The source gNB may indicate to the target gNB if UE should retain the available data in UE buffer for a particular data logging configuration during handover. And the target gNB shall include the indication in the handover command correspondingly.

Proposal 8	For gNB-centric NW side data collection, after handover, the target gNB may transfer the logged data from UE to source gNB via a Xn message conveying the corresponding RRC IE container (e.g., CSI-LogMeasReport IE).

Proposal 9	It is upon RAN3’s discussion that which Xn message should be for target gNB to transfer the logged data from UE to source gNB after handover.

Proposal 10	For OAM-centric NW side data collection, RAN2 assumes that after handover target gNB transfers the logged data from UE to source gNB, then source gNB further transfer to OAM.

Proposal 11	It is upon RAN3’s discussion if it should be supported as well that gNB transfers the logged data from UE directly to OAM after handover.

Proposal 12	The transfer of NAS (containing LPP message) over SRBx for data collection for AI based positioning is also supported.