3GPP TSG-RAN WG2 Meeting #131	R2-2505690

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:		8.9.2

Source:		Lenovo

Title:		Some remaining issues for S&F operation mode and transition time

Document for:		Discussion

- Introduction

The latest RAN2#130 meeting made the following agreements regarding Store and Forward (S&F) operations in IoT NTN:

RAN2#130 Agreements - When the S&F operation indication is present, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the normal mode will start; otherwise, the agreed time information in SIB31 (i.e., t-ModeSwitching) indicates when the S&F mode will start. - In a S&F network deployment which also exhibits discontinuous coverage, existing mechanisms to handle discontinuous coverage can be leveraged (e.g. satellite assistance information, UE not needing to perform idle mode tasks when the UE determines that is out of coverage, etc.). There is no need to modify existing discontinuous coverage features due to the addition of S&F Satellite operation. FFS if we clarify in discontinuous coverage procedure in idle mode that the UE also takes into account the information about NAS configured S&F monitoring list. - Clarify in Stage 2 from Rel-18 (so in general, not only for S&F) that the values used in “satelliteId-r17” are expected to correspond to the values used in “satelliteId-r18” in other SIBs (e.g. the value used to identify the serving satellite in SIB33, SIB32 and SIB31 should be the same). Come back in the next meeting with an actual Rel-18 CR for this. FFS if in Rel-19 we need to say anything about the correspondence between the “satelliteId” IE and “S&F Monitoring List”. RAN2#130 Working Assumption - In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not (FFS if we also include the transition time). This WA can only be confirmed if we converge on the corresponding UE behaviour.

In this contribution we discuss some remaining issues for S&F operation mode and transition time.

- Discussion

## Serving cell mode transition time and neighbour cell measurement

For the store and forward (S&F) operation in IoT NTN, it has been agreed to introduce an “S&F operation” indication in SIB1 to indicate that whether a cell is operating in S&F mode or normal mode, while its value can also act as a cell barring indication for S&F-capable UEs. Meanwhile to help UE in preparation for the mode transition, a mode transition time indication (t-ModeSwitching) is also introduced, which indicates the when the normal mode will start if “S&F operation” indication is present, and indicates when the S&F mode will start if “S&F operation” indication is absent. Based on the previous agreements, it is foreseen that the agreed mode transition time indication (t-ModeSwitching) may have impact on the neighbour cell measurement triggering.

On one hand (where less or no impact is expected) for a cell operating in S&F mode, only the S&F-capable UE may camp on. It is reasonable that all the services that can be delivered in S&F mode can also be delivered in normal mode (even with better QoS), and as a result it is unnecessary to trigger neighbour cell measurement for these UEs before the time when the normal mode will start.

Proposal 1: An S&F-capable UE is not required to trigger neighbour cell measurement for cell reselection before the mode transition time from SF to normal.

On the other hand (where more impact is expected) for a cell operating in normal mode, both the S&F-capable UE and the S&F-incapable UE may camp on. Some services that can be delivered in normal mode (e.g., time-sensitive services) may not be delivered in S&F mode due to large delivery delay. As a result, the S&F-capable UE may choose to camp on the cell when it transits to S&F mode, while the S&F-incapable UE has to triggered neighbour cell measurement before the mode transition time so that it may reselect to another cell e.g., one operating in normal mode.

Proposal 2: S&F-capable UE is not required to trigger neighbour cell measurement for cell reselection before the mode transition time from normal to SF.

Proposal 3: S&F-incapable UE is required to trigger neighbour cell measurement for cell reselection before the mode transition time from normal to SF.

## Neighbour cell operation mode indication

For the neighbour cells, the similar logic as above applies, i.e. at least the S&F-incapable UE does not expect to reselect to a cell operating in S&F mode. From this perspective, at least the operation indication of neighbour cell is useful for cell reselection.

Proposal 4: Confirm the following part of the previous Working Assumption “In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not”.

And such information can at least be used by the S&F-incapable UE to avoid cell reselection to a cell operating in S&F mode.

Proposal 5: S&F-incapable UE can deprioritize the frequency of a neighbour cell operating in SF mode for cell reselection.

A followed question is whether the mode transition time information of neighbour cells is also useful. In our understanding the S&F-incapable UE does not expect to reselect to a cell going to transit to S&F mode and thus such information can be useful, but we also think that how to use such information can be UE implementation to minimize the specification impact for the last meeting, i.e. there is no need to specify how UE use the mode transition time information of neighbour cells.

Proposal 6: Confirm the FFS part of the previous Working Assumption, e.g. in the neighbour cell list we introduce the mode transition time information of the cell. How to use such information is up to UE implementation.

- Conclusion

In this contribution we discuss further issues and enhancements for the mode transitions in S&F operation.. The following proposal are provided:

Proposal 1: An S&F-capable UE is not required to trigger neighbour cell measurement for cell reselection before the mode transition time from SF to normal.

Proposal 2: S&F-capable UE is not required to trigger neighbour cell measurement for cell reselection before the mode transition time from normal to SF.

Proposal 3: S&F-incapable UE is required to trigger neighbour cell measurement for cell reselection before the mode transition time from normal to SF.

Proposal 4: Confirm the following part of the previous Working Assumption “In the neighbour cell list we introduce an indication whether the cell operates in S&F mode or not”.

Proposal 5: S&F-incapable UE can deprioritize the frequency of a neighbour cell operating in SF mode for cell reselection.

Proposal 6: Confirm the FFS part of the previous Working Assumption, e.g. in the neighbour cell list we introduce the mode transition time information of the cell. How to use such information is up to UE implementation.