3GPP TSG-RAN WG2 Meeting #131	R2-2505692

Bangalore, India, Aug 25th – 29th, 2025                       Revision of R2-2503910

Agenda item:		8.9.4

Source:		Lenovo

Title:		Further considerations on PWS broadcast support in IoT NTN

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements to support PWS broadcast for NB-IoT UEs in NTN:

RAN2#129 agreements: 1. Introduce geographic information in ETWS messages for geo-fencing in IoT NTN cells 2. Introduce the Warning Area Coordinates IE for ETWS to describe an emergency area in system information. 3. Introduce the Warning Area Coordinates IE to existing ETWS SIBs, i.e., SIB10-NB/11-NB, without introducing the need for segmentation of SIB10-NB 4. We don’t introduce a new SIB for carrying geographical information for emergency areas RAN2#130 Agreements: - A UE should be able to camp on an NB-IoT cell (at least for NTN) for the purpose of receiving PWS broadcast, without the cell being a part of UEs PLMNs. Come back in the next meeting to discuss the exact solution to make this possible (i.e. not necessarily introducing an acceptable cell category for NB-IoT) - Regarding support continued reception of PWS segmentation of a message from different cells, can come back in the next meeting with an actual TP. In any case we will not target other scenarios than the intra-gNB case

In this contribution we discuss further issues and potential solutions to support PWS broadcast for NB-IoT in NTN.

- Discussion

In previous releases before Rel-19 the PWS mechanisms are designed for TN, and UE can simply judge whether it is in an emergency broadcast service area by its serving cell and the corresponding PWS broadcast configuration it receives. This mechanism is workable based on the assumption that a PWS broadcast service area is larger than a TN cell coverage so that the service area can be mapped to one or more TN cells.

However, the diameter of an NTN cell could be much larger than an emergency broadcast service area. An NTN cell coverage may also across country border on one side of which an emergency broadcast service is not allowed or authorized. It is observed that, for ETWS there is no IE to describe a geographical area in the contents of legacy NR SIB10 and SIB11, while for CMAS as an optional IE warningAreaCoordinatesSegment was introduced in SIB12 to describe the alert area coordinates of a CMAS message.

Regarding the area indication for ETWS in NR NTN, in RAN2#129 it was agreed to reuse the IE warningAreaCoordinatesSegment in SIB12 for CMAS, to be included in SIB10-NB/11-NB, with its formats following ATIS-0700041. Based on the ATIS-0700041, it is further observed that in addition to circles and polygons, warningAreaCoordinatesSegment can also be used to indicate the Geo-Fencing Maximum Wait Time (when the tag bits are set to 0001):

ATIS-0700041 … 5.2.3 WAC tag coding Table 8 identifies the WAC TLV tags defined for WEA. … 5.2.8 WAC Geo-Fencing Maximum Wait Time This WAC TLV describes the Geo-Fencing Maximum Wait Time a device shall allow to determine its position meeting operator policy. If the device is unable to determine its position meeting operator policy within the Geo-Fencing Maximum Wait Time, it shall present the alert to the user and discontinue further positioning determination for the alert. This WAC TLV is optional and when included shall only occur once in a WAC IE. If it is not included, the device shall assume the Geo-Fencing Maximum Wait Time value 255 described in Table 12. … Table 12 identifies the Geo-Fencing Maximum Wait Time values defined for WEA. The values have the following meaning: 0: Use existing device position meeting operator policy on position age. If existing device position does not meet that operator policy, then present the alert rather than initiating a new positioning determination. 1-254: Use the provided value for the maximum wait time to establish a device position meeting operator policy on position age. If a position meeting the operator policy is not established within the wait time, then present the alert to the user and discontinue further position determination. 255: Use the device default or implementation Geo-Fencing Maximum Wait Time to establish a device position meeting operator policy on position age. At the end of this time, if a position meeting the operator policy is not established, then present the alert to the user and discontinue further position determination.

Observation 1: The warningAreaCoordinatesSegment can be used to indicate Geo-Fencing Maximum Wait Time, which is the time a device shall allow to determine its position meeting operator policy.

Observation 2: If the device is unable to determine its position meeting operator policy within the Geo-Fencing Maximum Wait Time, it shall present the alert to the user and discontinue further positioning determination for the alert.

Based on the description in ATIS-0700041, our understanding is that reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time triggers a positioning procedure, and the positioning procedure is expected to success within the indicated time or it will be terminated. Meanwhile the alert seems to be for upper layers which is out of RAN scope or even 3GPP scope.

From RAN2 perspective, considering that the schemes introduced in Rel-18 IoT NTN for GNSS positioning (GNSs positioning triggering and reporting), there could be some impacts or conflicts between GNSS positioning for UL synchronization and the positioning triggered by warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time.

For the positioning procedure triggering, whether the reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time always triggers a new positioning procedure, or an on-going GNSS positioning for UL synchronization in NTN can fulfill the requirement (obtaining UE positioning within the Geo-Fencing Maximum Wait Time) as well. On the opposite, if a GNSS positioning is triggered by reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time, whether its result can be used for UL synchronization in NTN.

Proposal 1: RAN2 to discuss whether the outcome of GNSS positioning for UL synchronization in NTN at UE can be used to fulfill the requirement of the positioning from reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time.

Proposal 2: RAN2 to discuss whether the outcome of GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time can be used for UL synchronization in NTN.

For Proposal 2 if agreeable, a followed question is whether UE shall trigger report of gnss-ValidityDuration.

Proposal 2-bis: If the outcome of GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time can be used for UL synchronization in NTN, RAN2 to discuss whether UE shall trigger report of gnss-ValidityDuration.

In the GNSS positioning for UL synchronization at UE, there is a parameter gnss-PositionFixDuration reported by UE to indicate the time duration required for GNSS positioning. If the Geo-Fencing Maximum Wait Time indicated in warningAreaCoordinatesSegment is shorter than the gnss-PositionFixDuration, whether UE shall stop triggering GNSS positioning, or attempt for other positioning method, or present the alert to the user as described in ATIS-0700041 immediately.

Proposal 3: RAN2 to discuss if the Geo-Fencing Maximum Wait Time indicated in warningAreaCoordinatesSegment is shorter than the gnss-PositionFixDuration, whether UE shall stop triggering GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time. The UE may still attempt for other positioning methods.

- Conclusion

In this contribution we discuss further issues and potential solutions to support PWS broadcast for NB-IoT in NTN. The following observations are given:

Observation 1: The warningAreaCoordinatesSegment can be used to indicate Geo-Fencing Maximum Wait Time, which is the time a device shall allow to determine its position meeting operator policy.

Observation 2: If the device is unable to determine its position meeting operator policy within the Geo-Fencing Maximum Wait Time, it shall present the alert to the user and discontinue further positioning determination for the alert.

And we propose:

Proposal 1: RAN2 to discuss whether the outcome of GNSS positioning for UL synchronization in NTN at UE can be used to fulfill the requirement of the positioning from reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time.

Proposal 2: RAN2 to discuss whether the outcome of GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time can be used for UL synchronization in NTN.

Proposal 2-bis: If the outcome of GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time can be used for UL synchronization in NTN, RAN2 to discuss whether UE shall trigger report of gnss-ValidityDuration.

Proposal 3: RAN2 to discuss if the Geo-Fencing Maximum Wait Time indicated in warningAreaCoordinatesSegment is shorter than the gnss-PositionFixDuration, whether UE shall stop triggering GNSS positioning for reception of warningAreaCoordinatesSegment indicating Geo-Fencing Maximum Wait Time. The UE may still attempt for other positioning methods.