3GPP TSG-RAN WG2 Meeting #131	R2-2505695

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:		8.3.3

Source:		Lenovo

Title:		Discussion on network sided model

Document for:		Discussion and Decision

- Introduction

Based on the agreements made in the previous meetings, this contribution aims to discuss remaining issues related to Network sided model for AI mobility.

- Discussion

Configuration and report

In the RAN2#130 meeting, we had reached a consensus on enhancement of measurement report from one latest measurement result for each cell/beam to multiple time instances in one measurement report, e.g., “To support cell/beam level prediction, one enhancement is to report RRM measurement results per cell at multiple time instances in one measurement report for NW-sided model.” For temporal domain Case A, the input data for the NW-sided model would be the historical L3 cell/beam-level measurement results of multiple time instances within the observation window (OW). The number of multiple time instances (e.g., N) and the minimum interval value may be related to the OW length, and it can be configured by NW. Based on the OW, it is UE implementation to generate the measurement results. The minimum interval is used to avoid that the time gap between two measurement results is too close.

Proposal 1: When NW configures UE to log multiple time instances of L3 cell/beam measurement result, NW can configure the number of multiple time instances (e.g., N) and the minimum interval value between two consecutive instances.

Data collection

The availability indication can be triggered due to full buffer being reached, buffer threshold being reached or low power (if configured). In data collection for beam management, when the memory reserved for storing logged data becomes full, the UE stops measurement and logging for data collection. In data collection for RRM prediction or event prediction, the UE can stop measurement and logging for data collection when buffer is full. Furthermore, after the UE reports the logged data to the network, UE can restart measurement and logging for data collection since the buffer is not full.

Proposal 2: UE stops measurement and logging for data collection when full buffer being reached.

Proposal 3: UE resumes measurement and logging for data collection e.g., upon reporting the logged data to NW after stopping measurement and logging for data collection due to the full buffer.

As for the validity of data collection configuration for AI/ML-based beam management, the following descriptions based on agreements were captured in the TS 38.300 running CR for AI for air interface [2]:

When the memory reserved for storing logged data becomes full, the UE stops measurement and logging for data collection. When the memory reserved for storing logged data becomes full or reaches an absolute threshold, the UE indicates to the network, if configured, as specified in TS 38.331. When low power state is detected, the UE can indicate the low power state to the network, if configured. Upon reception of the low power state indication, the network should release the UE data collection configuration for NW-side model.

Based on the above agreements for AI/ML-based beam management, when the buffer is full, the UE can stop measurement and logging for data collection, however, the UE will not autonomously stop when low power state is detected until the network releases the configuration. Hence, for RRM prediction, the similar data collection configuration release mechanism can be reused.

Proposal 4: NW is expected to release the data collection configuration from UE upon receiving the low power status indication.

The availability indication can be triggered due to full buffer being reached or buffer threshold being reached. While whether the granularity of full buffer or buffer threshold is per use case is still FFS. If UE is configured to collect data with different logging intervals and the payload size for each quantity of the data could be different, a single buffer threshold may be unsuitable. For example, UE is configured to collect data for L1 beam-level results and L3 cell-level results with the different interval. Or UE is configured to collect data for measurement results and event occurrence, respectively. It is better to have a separate buffer threshold for each AI/ML feature/use case.

Proposal 5: RAN2 to support that the buffer-based trigger condition is per use case.

It was considered beneficial that UE will keep the logged data upon the detection of RLF. In 5G system, UE will performre-establishment or recovery. The cases are listed below.

- Case 1: Re-establishment procedure

- UE will initiate the re-establishment procedure upon RLF. After successful re-establishment towards a selected cell, UE can report the logged data to the re-established cell. If UE fails to re-establish, UE will enter idle state. UE will release the logged data when it moves to idle state, as agreed.

- Case 2: Handover based recovery (CHO based recovery, LTM based recovery, conditional LTM based recovery)

- UE will initiate the re-establishment procedure upon RLF. UE performs cell selection during re-establishment. Once the selected cell is configured with CHO, LTM or conditional LTM, UE will perform handover instead of re-establishment procedure. Once successful handover-based recovery, UE can report the logged data to the target cell. If the selected cell is not configured with handover configuration, UE will transmit re-establishment request to the selected cell which is same as Case#1. If UE fails to perform handover-based recovery, UE performs re-establishment procedure.

Proposal 6: To capture in TR above two cases for RLF case.

Performance monitoring

Based on last RAN2#130 meeting agreements, there is an FFS issue on performance monitoring for NW-sided model with NW-sided monitoring, e.g., “FFS on UE awareness and preference for NW sided model”. During online discussion, some companies would like to indicate a preference to the network on whether UE would like to do performance monitoring. Since for NW-sided model, the gNB is responsible for performance monitoring (i.e., calculate performance metrics), there are no additional impacts on the UE for monitoring and management, except for being configured to provide the required measurement/data. Hence, there may not be any need to introduce the UE awareness and preference to do performance monitoring.

Proposal 7: UE awareness and preference for performance monitoring of NW sided model is not needed.

- Conclusion

In this contribution, the following proposals are given based on the discussion:

Proposal 1: When NW configures UE to log multiple time instances of L3 cell/beam measurement result, NW can configure the number of multiple time instances (e.g., N) and the minimum interval value between two consecutive instances.

Proposal 2: UE stops measurement and logging for data collection when full buffer being reached.

Proposal 3: UE resumes measurement and logging for data collection e.g., upon reporting the logged data to NW after stopping measurement and logging for data collection due to the full buffer.

Proposal 4: NW is expected to release the data collection configuration from UE upon receiving the low power status indication.

Proposal 5: RAN2 to support that the buffer-based trigger condition is per use case.

Proposal 6: To capture in TR above two cases for RLF case.

Proposal 7: UE awareness and preference for performance monitoring of NW sided model is not needed.

4. References

[1] RAN2#130 RAN2 Chair notes

[2] TS 38.300 running CR for AI for air