3GPP TSG-RAN WG2 #131	R2-2505725

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.10.3

Source:	Nokia

Title:	SON/MDT enhancements for network slicing (RRC-3), MHI and NTN

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses three topics for Rel-19 SON/MDT enhancements: enhancements for network slicing, enhancements for MHI, and enhancements for NTN.

On the enhancement for network slicing this paper addresses the Open issue RRC-3 (logging highest ranked NSAG info associated with cell, freq etc.).

On the enhancements for MHI, this paper contains a proposal that is partially related to the Open issue RRC-8.

This paper also contains proposals to enhance SHR, and RLF report for RACH-less access for NTN, which has not been addressed earlier.

# 2	Discussion

## 2.1	RRC-3: logging highest ranked NSAG info associated with cell

At the RAN2#130 meeting the discussion on SON/MDT for slicing lead to the following conclusion:

- We aim to specify that: If the UE supports slice-based cell reselection does not find any suitable cell in the frequencies corresponding to the highest ranked NSAG, the UE logs the highest ranked NSAG. FFS the need of cell or frequency info.

Even if there was no explicit agreement on which report to capture slice specific information, the background assumption during the discussion at RAN2#130 was the use of logged MDT. Therefore, we think that an explicit agreement is needed to progress this work.

Proposal 1.1: Enhance the logged MDT report with slice-based cell reselection information.

The next step is to clarify the details of the MDT enhancements: what information is to be added, and when to perform this logging.

The UE should be configured via the LoggedMeasurementConfiguration message to perform the logged measurements. This message contains conditions (e.g., area configuration) when the UE should perform the logging. We think that the current configuration provides enough options (e.g., cell lists, TAC lists) to enable the MNOs to specify the areas where logging to be performed to discover issues with the configuration of slice-based cell reselection, and thus no changes are needed in the logged MDT configuration.

Observation 1: No change is needed in the logged MDT configuration (LoggedMeasurementConfiguration message) to support the logging of slicing related information logged MDT reports.

When the UE performs logging, the UE includes measurement results of neighbouring cells that became available during the last logging interval in order of decreasing ranking-criterion as used for cell re-selection. As this logged information is related to cell re-selection, this is an appropriate part of the report generation when slice-based cell reselection information can be logged.

As the measurement results of the neighbouring cells are added in order of decreasing ranking-criterion as used for cell re-selection, the network receiving the report has information about the frequency ordering by the UE. What the network cannot know based on the current report is the NSAG IDs that were used to create this order. If we would like a specification where all information that was used by the UE to rank the neighbouring cells, then all NSAG IDs in priority order should be provided in the report.

However, the discussion at RAN2#130 lead to conclusion to only add the most critical information, which is the NSAG ID with the highest priority, and whether the UE found a neighbouring cell based in this NSAG ID. Therefore, it is proposed to log the highest priority NSAG ID that was used for cell ranking and an indication whether any neighbouring cell was found based on this NSAG ID.

Providing slice-based cell reselection information is optional from the network side, and the support of slice-based cell reselection is optional for the UE. Therefore, we think the logging of this information is only meaningful if slice-based cell reselection information is advertised by the network, and it is only possible if the UE supports slice-based cell reselection. On the other hand, we do not think that a separate new capability for logging this information is necessary. Logging of slice based cell reselection information is not meaningful if the UE is in any cell selection state, as the slice-based cell reselection is only relevant when the UE can find suitable cells.

Proposal 1.2: If the UE supports slice-based cell reselection, and if it received slice-based cell reselection information and not in any cell selection state then the UE adds the highest priority NSAG ID that was used for cell ranking and an indication whether any neighbouring cell was found based on this NSAG ID in the LogMeasInfo. (Text proposal can be found in Annex 1 of the paper.)

## 2.2	MHI Enhancement for SCG Deactivation/Activation (related to RRC- 8)

The mechanism for SCG activation and deactivation was introduced in Rel-17 to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated.

SCG activation/deactivation can be triggered by UL or DL traffic on Data Radio Bearers (DRBs) with an SCG Radio Link Control (RLC) bearer (split or SCG bearer) by MN, SN or UE based on some activation/deactivation condition that is not clearly specified in 37.340. The following figure shows an example of how this could work assuming the activation/deactivation condition is based on a traffic threshold.

The concept of SCG activation and deactivation is most efficient for traffic where bursts of data large enough to benefit from SCG transmission are separated by periods of lower activity, during which the SCG can be deactivated. As the energy consumption in SCG active state is high, the criterion deciding upon SCG activation and deactivation shall be carefully selected. The simplest way is a UE traffic threshold where periods with data traffic above the threshold correspond to SCG activated state, while periods with data traffic below the threshold correspond to SCG deactivated state which may be configured for each UE or group of UEs with similar traffic profile.

In case the threshold is set as too low, the SCG activated period may contains also bursts of data which are not large enough to benefit from SCG transmission, hence leading to unnecessary power consumption. In case the threshold is set as too aggressively high, it could happen that not all bursts of data which are large enough benefit from SCG transmission are detected, which may lead to QoS degradation from end user perspective, though energy consumption may be reduced in this case.

Initially , the UHI and MHI for SCG included no information about SCG activation and deactivation. Despite majority of companies consider the information about the time of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity in our understanding, due to possible different setting of traffic thresholds between the PSCells of different nodes this information may be insufficient. The detailed information about relative time in SCG activated state may therefore need to be extended with some more information. For instance, if the relative time in SCG activated state is quite low, it means that DC is possibly not so necessary for the UE, and the network might not need configure DC for the UE, to reduce the UE’s battery consumption and the signaling overhead of DC configuration at network side. However, such a decision may not be optimal if the traffic threshold for the PSCells(s) reported within the MHI is set as too aggressively high. On the other hand, if the relative time in SCG activated state is quite high, it possibly means that the Dual Connectivity is highly demanded, and the MN could configure and/or activate the SCG more efficiently, which again may not be optimal if traffic threshold is set as too low.

Observation 2: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state together with additional assistance information related to the condition(s) for SCG activation/deactivation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

At RAN2#129bis it was agreed that “UE reports the absolute time it has spent in PSCell with SCG activated in MHI the following was agreed”.

Additionally, RAN2 has noted an open issue with regard to this topic during the email discussion [Post130][605][SONMDT] “Running NR RRC CR|:

[Open issue RRC-8] If time allows RAN2 discuss logging traffic information in MHI.

However, it is our opinion that, in relation to MHI/UHI neither the agreed addition of activation/deactivation time nor the traffic pattern as suggested in the open RRC issues are sufficient to help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity, as explained above.

Proposal 2: It is proposed to include additional assistance information related to the condition(s) for SCG activation/deactivation, in MHI such as DL and/or UL volume per second which was transmitted or received by UE during the SCG activation time for the given PSCell.

## 2.3	SON/MDT for NTN

As part of Rel-18, RAN2 specified different features to reduce the signalling overhead in NR NTN and reduce mobility latency. One of these features was the RACH-less Handover (HO) that aims at skipping Msg1 and Msg2 of the RACH procedure during the access of the target cell.

The RACH-less HO defined for NTN deployments has 2 main aspects Early TA and UL grant - using Configured Grant (CG) or Dynamic Grant (DG). We look at issues in the DG scenario below and propose SHR and RLF Report enhancements to address these issues.

Figure 3.1: Handover Procedure with RACH-less Access of T-Cell

To investigate the potential errors that could occur in the DG based RACH-less access mechanism please see the following

Step 3: After detecting the target cell and acquiring DL synchronization, UE uses quasi co-location (QCL) (SSB given in source as part of the RRC Reconfiguration vs DMRS of target) to monitor PDCCH of target. The UE monitors PDCCH waiting for a DCI assignment for PUSCH transmission. This UL grant also contains additional information to perform the transmission such as modulation and coding formats, time and frequency resource allocations, HARQ configuration or power parameters

The key thing to note here is that the UE does these actions on its own and the network is not aware of the time elapsed between successful reception of the HO command and the UE initiating the monitoring of the PDCCH.

Observation 3.1: The network is not aware of the time elapsed between the sending of the HO Command and the start of UE monitoring the PDCCH.

We discuss below the possible near-failure and failure scenarios that could result due to this.

Near failure scenario: Even though the RACH-less HO procedure may succeed, there could be issues like the UE having to monitor the PDCCH for a longer time for it to acquire PUSCH related configuration (i.e., NW delays the transmission of DCI) or the NW may have to transmit the UL grant for repeated times over a given search space until the UE acquires it (i.e., due to NW transmits the UL grant sooner than the UE is ready to receive). This issue may result in longer time for successful HO - resulting in higher UE energy consumption and/or sub-optimal utilization of allocated resources (for UL grant for example).

Observation 3.2: The DG based RACH-less access defined for NTN mobility could lead to sub-optimal scenarios (near failure scenarios) that would require constant measurement, monitoring and performance optimization.

The following are the possible NTN-specific failures scenarios that could happen in RACH-less access.

- Outdated/drifted SIB19 information for estimation of the target cell parameters received in the HO command. UE’s TA/doppler estimation may drift over time, if the estimation is off the optimal value, it may force the NW to allocate more resources to each PUSCH assignment to protect those resources.

- Power control parameters were not adjusted for latest channel impairments (a UE may receive sub-optimal PC parameters which may cause an unsuccessful PUSCH transmission).

- The ssb_index in HO Command could be changed/optimized. If a UE is located on the cell edge between two cells, it may result that the UE experiences better signal conditions on the adjacent cell (e.g., signals from both cells are fully uncorrelated since those belong to different satellites).

- Step 3 could fail because when the UE monitors for PDCCH, it is unable to decode the same due to bad radio conditions

- Step 4 could fail because the UE is unable to acquire the UL grant in time and timer T304 expires.

Observation 3.3: The DG based RACH-less access defined for NTN mobility can lead to many failure scenarios related to NTN specific parameters that would require some MRO enhancements.

Based on the above observations we think that enhancement of RLF report and SHR are needed to enable network optimization for RACH-less access for NTN.

Proposal 3.1: RAN2 to consider enhancing RLF Report and SHR for DG based RACH-less access for NTN with the addition of the following parameters:

- PDCCH Monitoring Time Until Success

- No of PUSCH Attempts Until Success

We also think that defining a new trigger to capture the newly defined measurements upon successful completion of NTN RACH-less HO as below would also be beneficial:

•	Time the UE spent monitoring the PDCCH channel is higher than the configured Threshold_1 AND/OR

•	Number of PUSCH Attempts is larger than Threshold_2

The Threshold-1 & Threshold-2 values can be configured via RRCReconfiguration message.

Proposal 3.2: Define a new SHR trigger (based on PDCCH monitoring time and/or number of PUSCH attempts) for the UE to log the enhancements measurements specific to NTN RACH-less access.

# 3	Conclusion

This document has made the following observation and proposals on SON/MDT for slicing (open issue RRC-3):

Proposal 1.1: Enhance the logged MDT report with slice-based cell reselection information.

Observation 1: No change is needed in the logged MDT configuration (LoggedMeasurementConfiguration message) to support the logging of slicing related information logged MDT reports.

Proposal 1.2: If the UE supports slice-based cell reselection, and if it received slice-based cell reselection information and not in any cell selection state then the UE adds the highest priority NSAG ID that was used for cell ranking and an indication whether any neighbouring cell was found based on this NSAG ID in the LogMeasInfo. (Text proposal can be found in Annex 1 of the paper.)

This document has made the following observation and proposal on MHI Enhancement for SCG Deactivation/Activation (related to open issue RRC- 8):

Observation 2: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state together with additional assistance information related to the condition(s) for SCG activation/deactivation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Proposal 2: It is proposed to include additional assistance information related to the condition(s) for SCG activation/deactivation, in MHI such as DL and/or UL volume per second which was transmitted or received by UE during the SCG activation time for the given PSCell.

This document has made the following observations and proposals on SON/MDT for NTN:

Observation 3.1: The network is not aware of the time elapsed between the sending of the HO Command and the start of UE monitoring the PDCCH.

Observation 3.2: The DG based RACH-less access defined for NTN mobility could lead to sub-optimal scenarios (near failure scenarios) that would require constant measurement, monitoring and performance optimization.

Observation 3.3: The DG based RACH-less access defined for NTN mobility can lead to many failure scenarios related to NTN specific parameters that would require some MRO enhancements.

Proposal 3.1: RAN2 to consider enhancing RLF Report and SHR for DG based RACH-less access for NTN with the addition of the following parameters:

- PDCCH Monitoring Time Until Success

- No of PUSCH Attempts Until Success

Proposal 3.2: Define a new SHR trigger (based on PDCCH monitoring time and/or number of PUSCH attempts) for the UE to log the enhancements measurements specific to NTN RACH-less access.

# Annex 1: TP for RRC-3 (Slicing support in SON/MDT)

The changes proposed in this Annex are implementing Proposal 1.2 in 38.331.

*** 1st Change ***

#### 5.5a.3.2	Initiation

<text omitted>

3>	if the UE is in any cell selection state (as specified in TS 38.304 [20]):

4>	set anyCellSelectionDetected to indicate the detection of no suitable or no acceptable cell found;

4>	if the reportType is set to eventTriggered in the VarLogMeasConfig; and

4>	if the RPLMN at the time of entering the any cell selection state is included in plmn-IdentityList stored in VarLogMeasReport or if the registered SNPN identity at the time of entering the any cell selection state is included in snpn-ConfigID-List stored in VarLogMeasReport; and

4>	if areaConfiguration is not included in VarLogMeasConfig or if the last suitable cell that the UE was camping on is part of the area indicated by areaConfig of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by cag-ConfigList of areaConfiguration in VarLogMeasConfig, or if last suitable cell that the UE was camping on is part of the area indicated by snpn-ConfigList of areaConfiguration in VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last suitable cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last suitable cell the UE was camping on;

4>	else if the reportType is set to periodical in the VarLogMeasConfig:

5>	set the servCellIdentity to indicate global cell identity of the last logged cell that the UE was camping on;

5>	set the measResultServingCell to include the quantities of the last logged cell the UE was camping on;

3>	else:

4>	set the servCellIdentity to indicate global cell identity of the cell the UE is camping on;

4>	set the measResultServingCell to include the quantities of the cell the UE is camping on;

4>	if the UE supports cell-based cell reselection and the UE is provided with NSAG information for cell selection and cell-based cell reselection information:

5>	set the highestRankedNSAG-ID to the highest ranked NSAG ID provided in the NSAG information for cell selection by NAS (TS 24.501 [23]);

5>	if the UE has not found any suitable cell that supports the highest ranked NSAG ID as specified in TS 38.304 [20]:

6>	include the noCellForHighestRankedNSAG;

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include measurements of neighbouring cell that became available during the last logging interval and according to the following:

<text omitted>

*** 2nd Change ***

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to network

UEInformationResponse message

-- ASN1START

-- TAG-UEINFORMATIONRESPONSE-START

UEInformationResponse-r16 ::=        SEQUENCE {

rrc-TransactionIdentifier            RRC-TransactionIdentifier,

criticalExtensions                   CHOICE {

ueInformationResponse-r16            UEInformationResponse-r16-IEs,

criticalExtensionsFuture             SEQUENCE {}

}

}

UEInformationResponse-r16-IEs ::=    SEQUENCE {

measResultIdleEUTRA-r16              MeasResultIdleEUTRA-r16             OPTIONAL,

measResultIdleNR-r16                 MeasResultIdleNR-r16                OPTIONAL,

logMeasReport-r16                    LogMeasReport-r16                   OPTIONAL,

connEstFailReport-r16                ConnEstFailReport-r16               OPTIONAL,

ra-ReportList-r16                    RA-ReportList-r16                   OPTIONAL,

rlf-Report-r16                       RLF-Report-r16                      OPTIONAL,

mobilityHistoryReport-r16            MobilityHistoryReport-r16           OPTIONAL,

lateNonCriticalExtension             OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1700-IEs     OPTIONAL

}

UEInformationResponse-v1700-IEs ::=  SEQUENCE {

successHO-Report-r17                 SuccessHO-Report-r17                OPTIONAL,

connEstFailReportList-r17            ConnEstFailReportList-r17           OPTIONAL,

coarseLocationInfo-r17               OCTET STRING                        OPTIONAL,

nonCriticalExtension                 UEInformationResponse-v1800-IEs     OPTIONAL

}

UEInformationResponse-v1800-IEs ::=  SEQUENCE {

flightPathInfoReport-r18             FlightPathInfoReport-r18            OPTIONAL,

successPSCell-Report-r18             SuccessPSCell-Report-r18            OPTIONAL,

measResultReselectionNR-r18          MeasResultIdleNR-r16                OPTIONAL,

nonCriticalExtension                 SEQUENCE {}                         OPTIONAL

}

FlightPathInfoReport-r18 ::=         SEQUENCE (SIZE (0..maxWayPoint-r18)) OF WayPoint-r18

WayPoint-r18 ::=                     SEQUENCE {

wayPointLocation-r18                 OCTET STRING,

timeStamp-r18                        AbsoluteTimeInfo-r16                OPTIONAL

}

LogMeasReport-r16 ::=                SEQUENCE {

absoluteTimeStamp-r16                AbsoluteTimeInfo-r16,

traceReference-r16                   TraceReference-r16,

traceRecordingSessionRef-r16         OCTET STRING (SIZE (2)),

tce-Id-r16                           OCTET STRING (SIZE (1)),

logMeasInfoList-r16                  LogMeasInfoList-r16,

logMeasAvailable-r16                 ENUMERATED {true}                   OPTIONAL,

logMeasAvailableBT-r16               ENUMERATED {true}                   OPTIONAL,

logMeasAvailableWLAN-r16             ENUMERATED {true}                   OPTIONAL,

...

}

LogMeasInfoList-r16 ::=              SEQUENCE (SIZE (1..maxLogMeasReport-r16)) OF LogMeasInfo-r16

LogMeasInfo-r16 ::=                  SEQUENCE {

locationInfo-r16                     LocationInfo-r16                    OPTIONAL,

relativeTimeStamp-r16                INTEGER (0..7200),

servCellIdentity-r16                 CGI-Info-Logging-r16                OPTIONAL,

measResultServingCell-r16            MeasResultServingCell-r16           OPTIONAL,

measResultNeighCells-r16             SEQUENCE {

measResultNeighCellListNR            MeasResultListLogging2NR-r16    OPTIONAL,

measResultNeighCellListEUTRA         MeasResultList2EUTRA-r16        OPTIONAL

},

anyCellSelectionDetected-r16         ENUMERATED {true}                   OPTIONAL,

...,

[[

inDeviceCoexDetected-r17             ENUMERATED {true}                   OPTIONAL

]],

[[

highestRankedNSAG-ID-r19             NSAG-ID-r17                         OPTIONAL,

noCellForHighestRankedNSAG-r19       ENUMERATED {true}                   OPTIONAL

]]

}

<text omitted>

*** End of Changes ***

LogMeasReport field descriptions

absoluteTimeStamp Indicates the absolute time when the logged measurement configuration logging is provided, as indicated by NR within absoluteTimeInfo.

anyCellSelectionDetected This field is used to indicate the detection of any cell selection state, as defined in TS 38.304 [20]. The UE sets this field when performing the logging of measurement results in RRC_IDLE or RRC_INACTIVE and there is no suitable cell or no acceptable cell.

highestRankedNSAG-ID Indicates the highest ranked NSAG ID provided in the NSAG information for cell selection by NAS (TS 24.501 [23]).

inDeviceCoexDetected Indicates that measurement logging is suspended due to IDC problem detection.

measResultServingCell This field refers to the log measurement results taken in the Serving cell.

noCellForHighestRankedNSAG Indicates that no suitable cell that supports the highest ranked NSAG ID has been found (TS 38.304 [20]).

numberOfGoodSSB Indicates the number of good beams (beams that are above absThreshSS-BlocksConsolidation, if configured by the network) associated to the cells within the R value range (which is configured by network for cell reselection) of the highest ranked cell as part of the beam level measurements. If the UE has no SSB of a neighbour cell whose measurement quantity is above the absThreshSS-BlocksConsolidation or if the network has not configured the absThreshSS-BlocksConsolidation, then the UE does not include numberOfGoodSSB for the corresponding neighbour cell. If the UE has no SSB of the serving cell whose measurement quantity is above the absThreshSS-BlocksConsolidation or if the network has not configured the absThreshSS-BlocksConsolidation, then the UE shall set the numberOfGoodSSB for the serving cell to one.

relativeTimeStamp Indicates the time of logging measurement results, measured relative to the absoluteTimeStamp. Value in seconds.

tce-Id Parameter Trace Collection Entity Id: See TS 32.422 [52].

traceRecordingSessionRef Parameter Trace Recording Session Reference: See TS 32.422 [52].

<text omitted>

*** End of changes ***