3GPP TSG RAN WG2 Meeting #131	 R2-2505728

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.2.3

Source: 	ITL

Title: 	Remaining issues on random access for A-IoT

Document for:	Discussion and decision

Introduction

In the previous RAN2 meeting [1], agreements related to A-IoT random access are as follows:

Agreements on parallel service request

1	For Msg1 resource selection procedure capture as guidance the countdown behaviour in the MAC specification (use TP in R2-2503952).  Capture a NOTE that other implementation are allowed.   X, Y will be signalled by paging message

2	The start of the first set of MSG1 resources is indicated by Paging message directly instead of the new R2D trigger messages.  R2D trigger message is not sent in CFRA procedure.   Come back if RAN1/4 sees any issues.  Send LS to RAN1/RAN4

3	FFS  R2D byte alignment dependent on TBS size discussion

Agreements on RA

1	Exclude the option of  MSG2 transmission and any retransmission of MSG2 happens within a predefined time window (based on timer)

2	A device expecting MSG2 assumes CBRA failure if its MSG2 is not received before a boundary, where the boundary can be further downselected between option B and C below.  A device receiving MSG2 within this boundary transmits MSG3. The device does not process MSG2 (re)transmission received after the boundary.

•	Option B – the boundary is the reception of either the next R2D trigger message or the subsequent paging message

•	Option C – the boundary is the reception of either the kth R2D trigger message or the subsequent paging message (K is FFS)

•	Option A (the boundary being the subsequent paging only) is excluded.

For option C, further discuss in terms of complexity at the device vs reader flexibility.

3	Including frequency index along with RN16 in MSG2 to reduce collisions of MSG1 between different devices is feasible.  FFS Discuss further whether to include it.

Agreements on NACK reception:

1	After MSG3 transmission, upon receiving NACK with its AS ID before subsequent paging or command addressed to this device from the reader, device determines it will perform re-access.   FFS how to specify.

Agreements on RN16/AS ID maintainance:

1	Confirm a device is not expected to maintain both AS ID and RN16.   After msg2 reception, RN16 becomes AS ID, if new AS ID was not assigned by reader.

This implies that the reader cannot change AS ID and RN16 pair across message 2 retransmission.  How to capture device behavior is FFS

This paper presents our views on the remaining issues that were not addressed in the previous meeting.

Discussion

Msg2 monitor window in CBRA

In the current procedure, MSG2 may be transmitted or retransmitted by the reader at any point during the paging frame. Without a defined reception limit, a device may receive MSG2 at any time within the frame and, upon reception, will retransmit MSG3. Since there is no ACK for MSG3 (only NACK), the device cannot distinguish between the loss of MSG2 and the loss of MSG3, and therefore retransmits MSG3 whenever MSG2 containing its own RN16 is received.

This behaviour leads to an increasing probability of MSG3 collisions toward the end of the paging frame. As more devices transmit MSG1 over the course of the frame, the likelihood of two devices having selected the same RN16 grows, especially in long paging frames (e.g., with large Q values).

In previous discussions on the MSG2 monitoring window for contention-based random access (CBRA), several alternative definitions for the reception boundary were considered.

It was agreed that a device regards the CBRA attempt as failed if MSG2 is not received before the defined boundary. If MSG2 is received within this boundary, the device transmit MSG3; ignore any MSG2 (re)transmissions received after the boundary.

Establishing such an MSG2 monitoring window (i.e., a defined reception boundary for MSG2) helps to avoid unnecessary MSG3 retransmissions in the later part of the paging frame. This, in turn, directly improves CBRA efficiency. The currently discussed Option B and Option C are the two candidate mechanisms for specifying this boundary.

- Option A (Excluded): Boundary defined solely as the reception of the subsequent paging message.

- Option B: The boundary is the reception of either the next R2D trigger message or the subsequent paging message

- Option C: The boundary is the reception of either the k-th R2D trigger message or the subsequent paging message (K is FFS)

| Aspect | Option B | Option C |
| --- | --- | --- |
| Boundary definition | Earlier of the next R2D trigger message or the subsequent paging message. | Earlier of the K-th R2D trigger message or the subsequent paging message (K is FFS). |
| Device Complexity | Low – only needs to detect next trigger as boundary. | Higher – device must count R2D triggers up to K |
| Reader Scheduling Flexibility | Limited – boundary is fixed to the next trigger. | Higher – K can be tuned to allow more (re)transmission opportunities. |
| Collision Mitigation | Strong – early boundary limits late-frame (re)transmissions. | Dependent on K – small K is similar to Option B; larger K reduces mitigation. |
| MSG3 Transmission Opportunity | Limited – fewer retransmission chances. | Greater when K is larger, but with increased late-frame collision risk. |

Moreover, it was agreed at RAN2#130 that including a frequency index in MSG2 is feasible to reduce MSG1 collisions among UEs; whether to include it and the associated details remain FFS. A UE transmits MSG3 only if both the echoed RN16 and the frequency index match; this reduces unintended MSG3 transmissions and partially mitigates the above issue.

We prefer Option C for CBRA failure determination, given that the reader may expect one or more MSG3 retransmissions and the device may not receive initial Msg2. The boundary is the reception of either the k-th R2D trigger message or the subsequent paging message. Furthermore, if K is configurable, Option C subsumes the behavior of Option B. In particular, setting K = 1 makes the boundary the earlier of the next R2D trigger or the subsequent paging message, which is identical to Option B.

Proposal 1: For CBRA failure detection boundary, the boundary is the reception of either the k-th R2D trigger message or the subsequent paging message

Random ID differentiation in Msg2

RAN2 supports the use of both time-division (TDM) and frequency-division (FDM) multiplexing of MSG1 resources following an access-occasion trigger. In line with this, it was agreed that a single MSG2 may echo multiple random IDs received from different A-IoT devices.

When two or more devices select the same random ID (e.g., RN16) but transmit MSG1 on different time/frequency resources, an MSG2 that echoes only the random ID creates ambiguity: a device cannot determine whether the echoed ID corresponds to its own MSG1 or to another device’s transmission. Because MSG3 is confined to FDM, this ambiguity can lead to concurrent MSG3 transmissions and thus undermines the benefit of provisioning multiple time/frequency resources for MSG1.

In the last meeting, two options were discussed, and it was agreed that including a frequency index in Msg2 is feasible

- Option 1: Time and/or Frequency index with each RN16

- Option 2: Bitmap of all time/frequency resources

Although some concerns were raised about UE complexity for a time index, considering Option C it may be appropriate for MSG2 to also include a time-resource index in addition to the frequency-resource index to ensure an unambiguous mapping to the device’s MSG1 transmission.

Although inclusion of a time index was not preferred owing to UE complexity, considering Option C it is proposed that MSG2 also include a time index (in addition to the frequency index) to unambiguously associate the echoed random ID with the device’s MSG1 attempt.

We support, at least, the inclusion of a frequency index with RN16 in Msg2; Subject to the CBRA failure-detection option adopted, inclusion of a time index should be considered.

Proposal 2: Introduce frequency index with RN16 in Msg2

Conclusion

In this contribution, we discussed the A-IoT random access. According to discussion in section 2, we have the following proposals:

Proposal 1: For CBRA failure detection boundary, the boundary is the reception of either the k-th R2D trigger message or the subsequent paging message

Proposal 2: Introduce frequency index with RN16 in Msg2

Reference

RAN2#130 meeting report