3GPP TSG-RAN WG2 Meeting #131	R2-2505750

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.7.6

Source:	OPPO

Title:	Discussion on XR Rate Control

Document for:	Discussion, Decision

Introduction

This paper addresses the open issues related to XR rate control.

- [MAC-3] Design of identifier for QoS flows indicated in the UL Rate Control MAC CE.

- [MAC-5]	Discuss UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE, e.g. whether the UE can transmit some of the queries or it must wait until it can send all pending queries together.

Discussion

  - [MAC-3] Identifier for QoS flows indicated in the UL Rate Control MAC CE

With the introduction of QoS flow-level bit rate signalling, RAN2 has explored various options for indicating a QoS flow, as detailed in RAN2#130.

- We rule out explicit signalling of PDU session ID + QFI - We will down-select between, considering the max number of flows we want to be able to indicate and trying to minimize overhead: - Explicit DRB ID + QFI (FFS if QFI is explicit or implicit) - Implicit, e.g. index or mapping

Proponents of the implicit indication method argue that it reduces the signalling overhead, as the explicit DRB ID and QFI currently occupy 5 bits and 6 bits, respectively. This method proposes a QoS flow bitmap in the UL Rate Control MAC CE to indicate the index of QoS flows for rate control, with the network configuring the mapping between these indexes and the corresponding QoS flow IDs + the DRB IDs.

maxDRB INTEGER ::= 29 -- Maximum number of DRBs (that can be added in DRB-ToAddModList). maxQFI INTEGER ::= 63

Theoretically, up to 960 QoS flows (15 * 64 = 960) can be supported in a UE, though only a subset may need rate control. If we assume 64 of these QoS flows are subject to rate control, 8 octets will need to be reserved for indicating the QoS flow index. The greater the number of QoS flows requiring rate control, the higher the signalling overhead due to the bitmap design. Additionally, if a QoS flow is remapped to another DRB afterwards, the network must reconfigure the mapping between the QoS flow index and the QoS flow ID + DRB ID. This reconfiguration introduces 1) the RRC signalling overhead, 2) potential confusion for the UE, if the UL Rate Control MAC CE and RRC reconfiguration message are received nearly simultaneously. Moreover, we believe that not many QoS flows related to an XR application need simultaneous bit rate adaptation. For instance, adapting the bit rate for just 2 QoS flows at a time may be sufficient. Therefore, we propose the explicit indication method, which uses the explicit DRB ID + QFI.

[MAC-3] In the UL Rate Control MAC CE, use explicit DRB ID + QFI to identify a specific QoS flow.

  - [MAC-5] UE behavior if available UL-SCH resources cannot accommodate all the pending bit rate queries in a single UL Rate Control MAC CE

In the existing RAN-assisted codec adaptation procedure, the MAC entity generates the Recommended bit rate MAC CE when it has UL resources for new transmission that can accommodate a Recommended bit rate MAC CE plus its subheader.

5.18.10 Recommended Bit Rate [omitted] If the MAC entity has UL resources allocated for new transmission the MAC entity shall: 1> for each Recommended bit rate query that the Recommended Bit Rate procedure determines has been triggered and not cancelled: 2> if bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query is configured, and it is not running; and 2> if the MAC entity has UL resources allocated for new transmission and the allocated UL resources can accommodate a Recommended bit rate MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1: 3> instruct the Multiplexing and Assembly procedure to generate the Recommended bit rate MAC CE for the logical channel and the direction of this Recommended bit rate query; 3> start the bitRateQueryProhibitTimer for the logical channel and the direction of this Recommended bit rate query; 3> cancel this Recommended bit rate query.

In RAN2#130, this multiplexing principle serves as a baseline for the UL Rate Control MAC CE for XR.

- (MAC-08, MAC-10) Legacy Recommended bit rate query procedure (i.e. triggering, multiplexing, cancellation, prohibit timer) is used as baseline for bit rate query, with the change of logical channel to QoS flow.

Since the UL Rate Control MAC CE can include information for multiple QoS flows, the UL-SCH resource may not always fit all bit rate queries in the MAC entity’s list.

Editor’s note: FFS whether a UL Rate Control MAC CE is transmitted only if the available UL-SCH resources can accommodate all the pending queries.

We are cautious about pursuing the optimization for this scenario, since reporting truncated or partial queries is generally not critical to the gNB.

[MAC-5] The MAC entity generates the UL Rate Control MAC CE only when the UL resources allocated for new transmission can accommodate the rate query MAC CE plus its subheader, including all the pending bit rate queries in the MAC entity’s list.

Conclusion

We have the following proposals:

Proposal 1	[MAC-3] In the UL Rate Control MAC CE, use explicit DRB ID + QFI to identify a specific QoS flow.

Proposal 2	[MAC-5] The MAC entity generates the UL Rate Control MAC CE only when the UL resources allocated for new transmission can accommodate the rate query MAC CE plus its subheader, including all the pending bit rate queries in the MAC entity’s list.

Reference

[1] RP-250107 Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur)