3GPP TSG-RAN WG2 #131	R2-2505764

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:	8.6.3 (NR_Mob_Ph4-Core)

Source:	LG Electronics Inc.

Title: 	Discussion on triggering of MR MAC CE for leaving beam

Document for:	Discussion and Decision

1. Introduction

In this contribution, we discuss the UE behaviours related to L1 MR for beams satisfying leaving condition.

2. Discussion

Issue1: Handling of MR for beam satisfying leaving condition

In general, if a beam satisfies leaving condition for TTT, this beam is expected to trigger a MR (a separate MR). However, this general understanding should be carefully checked, because the necessity of triggering a separate MR by the leaving beam depends on the cases, as discussed in the following.

Case (a) 	Let us consider a case that an initial MR has been triggered by entering beams B1, B2. These two beams are included in BEAM_ENTERING_LIST, and UE transmits a MR MAC CE including B1 and B2 as type1 beam. Upon sending the MR MAC CE, the B1 and B2 are included in BEAM_REPORTED_LIST, and BEAM_ENTERING_LIST is cleared. Then, one of the beams, say B2, included in the BEAM_REPORTED_LIST satisfies leaving condition for TTT.

Figure 1. Case (a) UE behaviours according to the MAC running CR (v22_rappor4)

In this case, for B2 which has been included in BEAM_REPORTED_LIST, it is clear that B2 should trigger a separate MR with the updated BEAM_LEAVING_LIST set to {B2}. Then, if UL grant is available and MR MAC CE is generated and sent, the MR MAC CE reports B2 as type2 beam and B1 as type3 beam. The current running CR is in line with the behaviours as described in this paragraph.

Case (b) 	Let us consider another case that an initial MR has been triggered by entering beams B1, B2. These two beams are included in BEAM_ENTERING_LIST, and UE transmits MR MAC CE for the initial MR including B1 and B2 as type1 beam. Upon sending the MR MAC CE, the B1 and B2 are included in BEAM_REPORTED_LIST, and BEAM_ENTERING_LIST is cleared. Then, a new beam (B3), which is neither in BEAM_ENTERING_LIST nor in BEAM_REPORTED_LIST, satisfies entering condition and hence triggers a subsequent MR. While the subsequent MR is pending, B3 satisfies leaving condition for TTT.

Figure 2. Case (b) UE behaviours according to the MAC running CR (v22_rappor4)

In the case, the question is whether the B3 should be included in BEAM_LEAVING_LIST and whether B3 should trigger a MR. If we follow the current running CR, satisfaction of leaving condition by B3 updates BEAM_LEAVING_LIST and then triggers a separate MR with the updated BEAM_LEAVING_LIST being set to {B3}. As a result, when UL grant is available and MR MAC CE for the subsequent MR is generated and sent, the MR MAC CE includes B1 and B2 as type1 beam and B3 as type2 beam.

In our view, since B3 has never been reported as type1 beam before for this MR entry within MR LIST (meaning that B3 has not been included in BEAM_REPORTED_LIST), B3 should not be reported as type2 beam for this MR.

Case (c) 	Now we consider an initial reporting case, where an initial MR has been triggered by entering B1 and B2, and while the initial MR is pending, one of these beams, say B2, included in the BEAM_ENTERING_LIST satisfies leaving condition for TTT.

Figure 3. Case (c) UE behaviours according to the MAC running CR (v22_rappor4)

For this case, if we follow the current running CR, satisfaction of leaving condition by B2 triggers a separate MR with BEAM_LEAVING_LIST being set to {B2}. As a result, when UL grant is available and hence the initial MR is generated and sent, the MR includes B1 as type1 beam and B2 as type2 beam.

In our view, however, B2 should not be reported as type2 beam for this MR, since B2 has never been reported as type1 beam before for this MR entry within MR LIST. That is, while initial MR triggered by beam(s) of BEAM_ENTERING_LIST is pending, satisfaction of leaving condition by any beam included in the BEAM_ENTERING_LIST should not update BEAM_LEAVING_LIST but only update BEAM_ENTERING_LIST (by removal of the beam), and it should not trigger a separate MR.

Based on the discussion, in order to avoid triggering MR by a beam satisfying leaving condition but not included in BEAM_REPORTED_LIST as described in case (b) and (c), we propose:

Proposal 1: When a beam satisfies leaving condition for TTT, the beam triggers MR if the beam has been included in BEAM_REPORTED_LIST.

In addition, in order to avoid reporting of type2 beam in MR MAC CE for initial MR as described in case (c), we propose:

Proposal 2: While initial MR is pending, satisfaction of leaving condition of any beam included in the BEAM_ENTERING_LIST does not update BEAM_LEAVING_LIST but only update BEAM_ENTERING_LIST/BEAM_REPORTED_LIST.

In the current running CR clause 5.x.3.1, if a beam satisfying leaving condition, UE includes an MR entry into MR LIST, if this is the initial MR (i.e., the case that no MR entry for this report config has existed in the MR LIST). In our view, this part should be removed, as triggering of MR by leaving condition for a leaving beam is only possible when the beam has been included in BEAM_REPORTED_LIST as proposed in proposal1.

Proposal 3: Remove the part in 5.x.3.1 of 38.321 running CR that UE includes an MR entry into MR LIST, if this is the initial MR (i.e., the case that no MR entry for this report config has existed in the MR LIST), as below.

2>	else if the leaving condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams included in the BEAM_ENTERING_LIST or BEAM_REPORTED_LIST for the measurement from lower layer during timeToTrigger defined for this event:

Issue2: Whether to reset reportAmount upon leaving condition being met

According to the current running CR, reportAmount is reset when MR MAC CE is transmitted.

In L3 MR report, reportAmount is reset if MR is triggered by entering cell. If MR is triggered by leaving cell, reportAmount is not reset, which is to avoid too frequent reset of reportAmount and hence to avoid too frequent MR triggering.

In our view, it is desirable for LTM MR to be aligned with L3 MR report in treating the Amount. That is, reportAmount should not be reset for MR purely triggered by a leaving condition, i.e., when generating MR MAC CE is generated, if BEAM_ENTERING_LIST is empty (i.e., no type1 beam in this MAC CE), the reportAmount should be continued.

Proposal 4: Once generating MR MAC CE, reportAmount is reset if BEAM_ENTERING_LIST is non-empty (before the list is cleared).

Conclusion

Proposal 1: If a beam satisfies leaving condition for TTT, the beam triggers MR if the beam has been included in BEAM_REPORTED_LIST.

Proposal 2: While initial MR is pending, satisfaction of leaving condition of any beam included in the BEAM_ENTERING_LIST does not update BEAM_LEAVING_LIST but only update BEAM_ENTERING_LIST/BEAM_REPORTED_LIST.

Proposal 3: Remove the part in 5.x.3.1 that UE includes an MR entry into MR LIST, if this is the initial MR (i.e., the case that no MR entry for this report config has existed in the MR LIST)

Proposal 4: Once generating MR MAC CE, reportAmount is reset if BEAM_ENTERING_LIST is non-empty (before the list is cleared).

Text proposal

A text proposal to the running MAC CR (v22_rappor4) is given below.

5.x.3.1 General … Unless explicitly specified otherwise, it is up to UE implementation how to store these variables. The MAC entity shall for LTM event evaluation procedure: 1> for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig: 2> if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration: 3> if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId: 4> consider only the current beam of serving cell, i.e. the beam corresponds to the RS configured in the indicated TCI State or the RS QCLed with the RS configured in the indicated TCI State indicated by TCI State in the serving cell as defined in clause 5.1.5 in TS 38.214, with the same RS type as the beam of LTM candidate cell, i.e. the RSs configured in LTM-CSI-ResourceConfig which associated with this ltm-CSI-ReportConfigId, to be applicable; 3> if the eventLTM3, eventLTM4, or eventLTM5 is configured in the corresponding ltm-CSI-ReportConfigId: 4> consider any beam of LTM candidate cell (except the serving cell) configured in ltm-CandidateReportConfigList, i.e. the RSs configured in LTM-CSI-ResourceConfig which associated with this ltm-CSI-ReportConfigId, to be applicable; 2> if the entry condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams, i.e. reference signalling associated with SSB-Index or NZP-CSI-RS-ResourceID in the LTM-CSI-ResourceConfig associated with the LTM-CSI-ReportConfig, which is not in the BEAM_ENTERING_LIST and not in the BEAM_REPORTED_LIST, for the measurement from lower layer during timeToTrigger defined for this event: 3> if the MR_LIST does not include a measurement reporting entry for this ltm-CSI-ReportConfigId (a first RS triggers the event): 4> include a measurement reporting entry in the MR_LIST for this ltm-CSI-ReportConfigId; 3> include the SSBRI or CRI of the concerned beam(s) in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; 3> initiate the measurement reporting procedure, as specified in 5.x.4. 2> else if the leaving condition for the event associated with ltm-CSI-ReportConfigId is fulfilled for one or more applicable beams included in the BEAM_ENTERING_LIST or BEAM_REPORTED_LIST for the measurement from lower layer during timeToTrigger defined for this event: 3> if BEAM_REPORTED_LIST is not empty, include the SSBRI or CRI of the concerned beam(s) in the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; 3> remove the concerned beam(s) in the BEAM_ENTERING_LIST or BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId; 3> if the concerned beam is included in BEAM_REPORTED_LIST and reportOnLeave is set to true for this ltm-CSI-ReportConfigId: 4> initiate the measurement reporting procedure, as specified in 5.x.4; 2> upon expiry of the periodical reporting timer for this ltm-CSI-ReportConfigId: 3> initiate the measurement reporting procedure, as specified in 5.x.4. NOTE x: timeToTrigger is not restarted if the current beam of serving cell changes and the entry condition is still met with the new current beam. NOTE y: To evaluate the L1 measurement reporting triggering event, the UE uses the latest L1-RSRP measurement from lower layer. 5.x.4 Measurement report ... For the event triggered L1 measurement reporting, for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig, the MAC entity shall: 1> if at least one L1 measurement report has been triggered as specified in 5.x.3 and not cancelled: 2> if UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as befined in clause 6.1.3.x according to the measurement report information in the MR_LIST; 3> if reportAmount is configured in the LTM-EventTriggeredPeriodicReport by RRC: 4> if at least one L1 measurement report associated with the ltm-CSI-ReportConfigId has been triggered due to the reason with non-empty BEAM_ENTERING_LIST other than the expiry of the periodical reporting timer and the L1 measurement report is not cancelled: 5> set the MR_SENT_COUNTER to 0 for this ltm-CSI-ReportConfigId; 4> increment the MR_SENT_COUNTER associated with the ltm-CSI-ReportConfigId by 1; 4> stop the periodical reporting timer, if running; 4> if reportAmount is configured in RRC, and the MR_SENT_COUNTER as defined for this ltm-CSI-ReportConfigId is less than reportAmount: 5> restart the periodical reporting timer with the value of reportInterval for this ltm-CSI-ReportConfigId as defined within the corresponding LTM-CSI-reportConfig; 3> include the SSBRI or CRI in the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId, if any, into the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId; 3> clear the BEAM_ENTERING_LIST for this ltm-CSI-ReportConfigId; 3> clear the BEAM_LEAVING_LIST for this ltm-CSI-ReportConfigId; 3> if the BEAM_REPORTED_LIST for this ltm-CSI-ReportConfigId is empty: 4> remove the measurement reporting entry within the MR_LIST for this ltm-CSI-ReportConfigId; 4> stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running; 3> cancel the triggered L1 measurement report; 2> else if the UL-SCH resources are available for a new transmission in the serving cell and these UL-SCH resources can accommodate the Truncated L1 measurement report MAC CE plus its subheader as a result of logical channel prioritization: 3> instruct the Multiplexing and Assembly procedure to generate the Truncated L1 measurement report MAC CE associated with the ltm-CSI-ReportConfigId as defined in clause 6.1.3.x according to the measurement report information in the MR_LIST be selecting the RS(s) based on a decreasing order of the priority for the type of beam: RS(s) in BEAM_ENTERING_LIST, RS(s) in BEAM_LEAVING_LIST, RS(s) in BEAM_REPORTED_LIST, and other RS(s) not in these three lists; 2> else: 3> if the dedicated SR configuration for L1 measurement report MAC CE transmission is configured: 4> trigger the SR using the dedicated SR configuration for L1 measurement report; 3> else: 4> initiate a Random Access procedure (see clause 5.1) on the SpCell and cancel the pending SR trigger; 3> stop the periodical reporting timer for this ltm-CSI-ReportConfigId, if running. NOTE X: After sending a Truncated L1 measurement report MAC CE, if the subsequent UL grant is still not big enough to fit all the remaining beams, it is up to UE implementation to select the beam(s) which were not included in the previous MAC CE. NOTE Y: If more than one triggering events for L1 measurement report are pending and the UL grant is not sufficient for all regular L1 measurement report MAC CEs, it is up to UE implementation how to handle/include the L1 measurement report MAC CEs and/or Truncated L1 measurement report MAC CE. NOTE Z: When a measurement report is triggered by entry condition for one or more RS(s), and included in the BEAM_ENTERING_LIST, another measurement report is triggered by exit condition for the same RS(s), all the corresponding measurement reports are cancelled. When a measurement report is triggered by exit condition for one or more RS(s), and included in the BEAM_ENTERING_LIST, another measurement report is triggered by entry condition for the same RS(s), all the corresponding measurement reports are cancelled.