3GPP TSG-RAN WG2 Meeting #131											        R2-2505767

Bengaluru, 25-29 August 2025

Agenda item:			8.8.4

Title:							Remaining open issues for MBS service continuity over NTN

Source:					Continental Automotive

Document for:			Discussion and Decision

# Introduction

The Non-Terrestrial Networks (NTN) for NR Phase 3 work item aims at specifying support for broadcast services via NR NTN in Rel-19 [1]:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

During RAN2#125bis [2] the following agreements were made:

Agreements:

- For MBS broadcast service we don’t restrict the work to any satellite constellation type

- We prioritize working on a solution for MBS broadcast but we don’t preclude other broadcast services, namely ETWS

- We will cover at least the case where the indicated intended service area covers a portion of a NTN cell

- The intended service area can cover the area of more than one NTN cells (or portions thereof)

- Can discuss next time whether the broadcast transmission can be limited to the intended service area only (i.e. no transmission happens outside of the intended serive area)

- At least the following geographical area formats to model service area can be further considered (the signalling of other information than the geographical information can be considered):

- Circles (like for TN coverage)

- Geographical area information, e.g. via polygons, to better approximate the intended shape of service area

Further, during RAN#126 [3], RAN2#129bis [4], and RAN#130 [5] the following agreements were made:

Agreements:

- For MBS broadcast service, both EFC and EMC are supported.

- RAN2 will not define means for the NW to prevent the reception of the content of the service outside of the intended service area.

- MBS broadcast intended service area is provided via system information

- For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

- When intended service area (e.g., geographical area/TN coverage) is provided for MBS broadcast service, it needs to be associated with MBS session (FFS on the details)

Agreements:

1.	RAN2 understands the Intended service areas of all MBS broadcast services of the current serving cell that need to be geo-fenced will be included in the new SIBxx (no spec impacts)

2.	If UE knows it’s not in any intended service areas of any MBS services the UE is interested into, the UE may not need to acquire MCCH

3.	If no intended service area is explicitly indicated (e.g. in SIBxx) for a MBS service the UE is interested into, existing behavior applies.

4.	The field warningAreaCoordinates is included in SIB6 while the field warningAreaCoordinatesSegment is included in SIB7 for ETWS primary/secondary notification to indicate Warning Area Coordinates IE.

Agreements:

1.	Do not introduce service area information specific for FSAI in SIB21.

2.	Do not introduce service area per neighbour cell in MCCH

3.	RAN2 assumes that the ISA can be valid for all the frequencies providing the MBS session

4.	The new SIBxx containing MBS ISA(s) can be cell or area specific, which is up to NW implementation (No specs change).

In this paper, we discuss remaining issues related to MBS service continuity over NTN, specifically concerning the provisioning of intended service areas and the MBS interest indication procedure, as identified in [6].

# 2. Discussion

RAN2 agreed [2][3][4] not to restrict the work to any satellite constellation type and to support both quasi-earth-fixed and earth-moving cells for MBS broadcast services The intended service area (ISA) may span multiple NTN cells or part of them.

In NGSO scenarios, ISAs may be covered by various, fast moving quasi-earth-fixed or earth-moving cells. As a result, the UE may frequently switch between satellites and, consequently, needs to frequently acquire MCCH updates.

Fig. 1 illustrates a scenario, where fast moving NTN cells cover several ISAs on ground. At T=t0, a stationary UE (located in ISA #0) is covered by cell #0, and at T=t1, the same UE (still in ISA #0) is now covered by cell #1.

Fig. 1 – A stationary UE at times T=t0 and T=t1 is in coverage of various, fast moving NTN cells.

Fig. 2 shows a moving UE. At T=t0, the UE is in ISA #0 and covered by cell #0. At T=t1, it moves to ISA #1, where it may be covered by cell #1 or cell #2.

Fig. 2 – A moving UE at times T=t0 and T=t1 is in coverage of various, fast moving NTN cells.

Observation 1: In NGSO scenarios, ISAs may be covered by various, fast moving NTN cells. A UE that is interested in a particular MBS service needs to frequently acquire MCCH updates.

While such updates help the UE to understand where the MBS service is available, it may also be beneficial for the UE to know how long the MBS service will be available in a given ISA.

For instance, if the estimated service time in an ISA is too short, the UE may choose to delay the MBS interest indication procedure to avoid sudden service interruptions.

Therefore, the network should provide additional guidance to help the UE select suitable ISAs for the MBS service. For example, along with MBS-related ISAs, the network may provide estimated service times for ISAs where satellite changes occur frequently. Based on these estimates, the UE can decide whether to initiate MBS service or not.

Proposal 1: The network should provide estimated service times for ISAs where satellite changes occur frequently. Based on these estimates, the UE can decide whether to initiate the MBS service or not.

# Conclusions

The following observation and proposal have been made in this contribution:

Observation 1: In NGSO scenarios, ISAs may be covered by various, fast moving NTN cells. A UE that is interested in a particular MBS service needs to frequently acquire MCCH updates.

Proposal 1: The network should provide estimated service times for ISAs where satellite changes occur frequently. Based on these estimates, the UE can decide whether to initiate the MBS service or not.

# References

- RP-234076, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN1 Vice-Chair, RAN#102, December 11-15, 2023, Edinburgh, UK.

- R2-2403733, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#125bis, Changsha, China, April 15th – 19th, 2024.

- R2-2405703, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#126, Fukuoka, Japan, May 20th-24th, 2024.

- R2-2502983, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#129bis, Wuhan, China, April 7th-11th, 2025.

- R2-2504673, Report from Break-out session on NR-NTN and IoT-NTN, Session Chair (ZTE Corporation), RAN2#130, Malta, May 19th-23rd, 2025

- [Post130][302][R19 NR NTN] Remaining RRC open issues for NR NTN Rel-19, Ericsson