3GPP TSG-RAN WG2 Meeting #131			         R2-2505780

Bangalore, India  August 25th – 29th , 2025

Agenda Item	: 8.4.3

Source	: Tejas Networks.

Title	: Remaining issues in LP-WUS based RRM relaxation and offloading

Document	: Discussion

# Introduction

This contribution discusses the open issues related to LP-WUS based RRM relaxation procedures in IDLE and INACTIVE mode.

# Discussion

## 2.1 Supporting Rel16 RRM relaxation based on LR measurement (RRC-15/38304-2)

Introducing new threshold based on LR measurement specifically for rel19 UEs to support rel16 RRM relaxation feature appears to be a corner case and new signalling needs to be introduced to support such cases. In case a rel19 UE is configured with both rel19 RRM relaxation and rel16 RRM relaxation, the rel19 UE applies only rel19 feature. This simplifies UE behaviour and avoids need for another threshold value to be configured by the network.

Proposal 1: If Rel-16 RRM relaxation and Rel-19 relaxation features are configured together, Rel-19 UEs applies Rel-19 RRM relaxation feature only.

### 2.2 Considering low mobility criteria for LR based Relaxation (RRC-10/38304-6)

RAN4 has not studied the use of low mobility criteria as an entry/exit condition for rel19 RRM relaxation/offloading procedure and given that it’s the last meeting for rel19 LP-WUS discussion, we should not introduce low mobility criteria in entry/exit condition evaluation

Proposal 2: Low mobility criteria is not considered for Rel19 RRM relaxation/offloading entry/exit condition evaluation criteria.

### 2.3 Behaviour when logged measurement is configured

TS 38.304 Section 5.2.4.9.0

<unrelated texts omited>

The above relaxed measurements and no measurement are not applicable for frequencies that are included in VarMeasIdleConfig, if configured and for which the UE supports dual connectivity or carrier aggregation between those frequencies and the frequency of the current serving cell.

Based on current 38.304 spec, in case the UE is configured to perform early measurement reporting in IDLE/INACTIVE mode, the UE does not perform legacy RRM relaxation on the configured frequencies. We should apply similar principle to LR based RRM relaxation and offloading as well.

Proposal 3: UE does not perform Rel19 RRM relaxation/offloading when Early Measurement Reporting is configured.

# Conclusion

Proposal 1: (RRC-15/38304-2) If Rel-16 RRM relaxation and Rel-19 relaxation features are configured together, Rel-19 UEs applies Rel-19 RRM relaxation feature only.

Proposal 2: (RRC-10/38304-6) Low mobility criteria is not considered for Rel19 RRM relaxation/offloading entry/exit condition evaluation criteria.

Proposal 3: UE does not perform Rel19 RRM relaxation/offloading when Early Measurement Reporting is configured.