3GPP RAN WG2 Meeting #130	R2-2505786

Bangalore, India, Aug 25th – 29th, 2025

Agenda Item:	8.6.2

Source:	Ofinno

Title:	Discussion on open issues of inter-CU LTM

Document for:	Discussion, Decision

# Introduction

In the previous RAN2 meetings, the following agreements were made:

RAN2 #129bis - If RLF: If the selected candidate cell has the same Rel-19 ID as source (no security key change), the UE performs fast RLF recovery (same as in Rel-18). - if reconfiguration failure (inter-CU LTM): If the selected candidate cell has the same Rel-19 ID as target, the UE performs fast failure recovery. RAN2 #130 - RAN2 agree to not support the LTM recovery to a candidate cell with different Rel-19 ID from the target cell. - If the selected cell has same R19 ID as the target cell after the inter-CU MCG LTM failure, UE performs LTM fast recovery to the selected cell with the security key update using the NCC in the LTM cell switch command and the selected cell’s PCI and frequency.

In this contribution, we discuss open issues on inter-CU LTM.

# Identified open issues:

A UE can be configured with candidate cells for MCG LTM. Some of these candidate cells may belong to the same CU as serving cell while others may belong to a different CU. Further, some LTM candidate configurations of the candidate cells may consist of new SCG configuration while others may simply keep or release the existing SCG configuration (i.e., SCG configuration on the UE at the time of an LTM cell switch to the candidate cell).

## Fast failure recovery

In the current TS 38.331, a CHO candidate cell can be associated with a candidate SCG. A UE is allowed to perform a CHO based fast recovery e.g., UE executes CHO instead of RRC reestablishment. However, if CHO candidate cell, selected during the CHO based fast recovery, is configured with condExecutionCondPSCell, the UE does not execute CHO based fast recovery. In other words, the UE performs a CHO based fast recovery on a selected CHO candidate cell, if the selected CHO candidate cell does not comprise an execution condition that needs to be fulfilled for an associated PSCell in order to trigger the execution of CHO with candidate SCG. For CHO, the candidate cells for which condExecutionCondPSCell is configured is not considered for CHO based failure recovery, as indicated in the blue highlight below.

#### 5.3.7.3 Actions following cell selection while T311 is running Upon selecting a suitable NR cell, the UE shall: … 1> if attemptCondReconfig is configured; and 1> if the selected cell is not configured with CondEventT1, or the selected cell is configured with CondEventT1 and leaving condition has not been fulfilled; and 1> if the selected cell is one of the candidate cells for which the reconfigurationWithSync is included in the masterCellGroup in the MCG VarConditionalReconfig and the condExecutionCondPSCell is not configured for the corresponding condReconfigId in the MCG VarConditionalReconfig: 2> if the UE supports RLF-Report for conditional handover, set the choCellId in the VarRLF-Report to the global cell identity, if available, otherwise to the physical cell identity and carrier frequency of the selected cell; 2> apply the stored condRRCReconfig associated to the selected cell and perform actions as specified in 5.3.5.3; 1> if the cell selection is triggered by detecting radio link failure of the MCG or re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6; and 1> if attemptLTM-Switch is configured; and 1> if attemptLTM-Switch is configured; and 1> if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG: 2> if the selected cell does not have the field ltm-NoSecurityChangeID configured and if the UE does not have any value stored of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or 2> if the cell selection is triggered by detecting radio link failure of the MCG and the selected cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID; or 2> if the cell selection is triggered by detecting re-configuration with sync failure of the MCG for an LTM cell switch procedure triggered upon the indication by lower layers as specified in clause 5.3.5.18.6 and the seleted cell has a ltm-NoSecurityChangeID configured with a value which is equal to the value of ltm-NoSecurityChangeID configured within the LTM candidate configuration for which the re-configuration with sync failure is detected: 3> perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

In LTM, an LTM candidate cell for MCG can be configured with SCG addition. According to the section 5.3.7.3 of TS 38.331 (see the yellow highlight above), for LTM fast recovery, a UE is allowed to use a configuration for MCG LTM with SCG. RAN2 needs to discuss whether a configuration for MCG LTM with SCG is considered for an LTM fast recovery.

Proposal 1: RAN2 to discuss whether LTM candidate configuration for MCG LTM with SCG is considered for LTM fast recovery.

## Eary TA acquisition of SCG within MCG LTM

For a candidate PSCell of SCG LTM, an early TA acquisition is supported. A UE may skip a random access procedure toward the candidate PSCell using a TA acquired by the early TA acquisition procedure. It reduces a switching time of a SCG LTM cell switch procedure for the candidate PSCell. However, when a UE applied a SCG configuration within MCG LTM configuration. the UE need to perform a random access procedure toward a candidate PSCell of SCG within MCG LTM.

Observation 1: early TA acquisition is supported for candidate PSCell of SCG LTM while not supported for a candidate PSCell of SCG within MCG LTM.

For example, the candidate PSCell of SCG within MCG LTM may be of SCG LTM. The UE may acquire a TA value of the candidate PSCell based on UE based TA measurements. The UE may use the TA value of the candidate PSCell to access to the candidate PSCell. It is beneficial to use the TA of the candidate PSCell for access to the candidate PSCell.

Proposal 2: RAN2 discuss whether to support early TA acquisition for a candidate PSCell of SCG within MCG LTM.

## CG resource for LTM

A network may postpone allocating a RACH resource for LTM until transmitting an LTM cell switch command indicating the RACH resource. However, the network may allocate a configured grant (CG) resource when transmitting to a UE an LTM configuration. The network may need to keep allocating the CG resource of a candidate cell until the UE execute an LTM cell switch procedure for the candidate cell. The UE may not execute the LTM cell switch procedure for the candidate cell. The CG resource is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

Observation 2: a CG resource of a candidate cell for LTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

The network may need to release or update the CG resource (e.g., to avoid the waste of the CG resource). For releasing or updating the CG resource, the network needs to transmit an RRC reconfiguration message comprising an LTM configuration. It causes signal overheads and is not desired for subsequent LTM without the RRC reconfiguration.

## Proposal 3: RAN2 discuss dynamic management of CG resource of candidate cell for LTM.

# Conclusions

The contribution discussed aspects related to remaining open issue for inter-CU LTM. The following observations and proposals are made:

## Proposal 1: RAN2 to discuss whether LTM candidate configuration for MCG LTM with SCG is considered for LTM fast recovery.

## Observation 1: early TA acquisition is supported for candidate PSCell of SCG LTM while not supported for a candidate PSCell of SCG within MCG LTM.

## Proposal 2: RAN2 discuss whether to support early TA acquisition for a candidate PSCell of SCG within MCG LTM.

## Observation 2: a CG resource of a candidate cell for LTM is wasted until the UE execute the LTM cell switch procedure for the candidate cell.

## Proposal 3: RAN2 discuss dynamic management of CG resource of candidate cell for LTM.