3GPP TSG-RAN2 Meeting #131	R2-2505795

Bangalore, India, 25 - 29 August 2025

Agenda Item:	8.13.2

Source:	Ericsson

Title:	Discussion on relay discovery and relay (re)selection

Document for:	Discussion, Decision

# 1	Introduction

For the WI on NR sidelink multiple-hop relay, the below objectives on relay discovery and relay (re)selection have been defined in the WID [1].

The objective of this work item is to specify solutions that are needed to support multi-hop Layer-2 UE-to-Network relay for a single indirect path via SL relay UEs based on Rel-17/18 SL relay functionalities [RAN2, RAN3]

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

.

We discuss the remaining open issues summarized in Post129bis][405][Relay] Rel-19 relay main CR to 38.331 (Huawei) and related to discovery and/or relay (re)selection in this paper.

# 2	Discussion

At RAN2#130, RAN2 has made the below agreement

Notification is always sent for AS failure cases at the intermediate relay UE (e.g., upstream RLF that prevents the intermediate relay from having a path to the network), using the legacy mechanism.  FFS new cause values.

For the above FFS, we think it is sufficient for remote UE to know the exact reason of detected failure event in the upstream links/hops so that remote UE may trigger relay reselection. The legacy cause value e.g., PC5-RLF etc is perhaps already enough. In this case, the intermediate relay UE can just use the same cause value as received from patent relay UEs. Meanwhile the rapporteur has suggested new cause cannot hint the actual failure cause.

In addition, although the intermediate relay UE may take recovery action upon reception of a failure notification message from upstream, we see no reason to introduce new cause value to reflect the intermediate relay UE’s own state. Even with the legacy cause value, the remote UE can still guess intermediate UE on the path has already recovered for itself. Upon knowledge of the actual failure cause on the path, it is up to the remote UE’s implementation to determine whether the current path is suitable to it and whether to trigger relay reselection.

In the sidelink notification message towards child UEs, the Intermediate Relay UE sets the indication type to the failure cause received from its parent relay UE, i.e., no new cause needed to reflect its own relay reselection action.

There is another remaining issue from the RAN2 agreement made at RAN2#129bis

For discovery model B when there is no established PC5 link between the last relay UE and the intermediate relay UE, the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the intermediate Relay UE.  FFS if this case can occur or if the intermediate relay UE always establishes itself as a remote UE first.

For this FFS, our view is that there is a valid case whether the intermediate relay UE may have no established PC5 link towards the last relay UE when forwarding discovery solicitation message. When an intermediate relay UE forwards a received discovery solicitation message in case of discovery model B, the intermediate relay UE is not required to have a PC5 link or establishes itself as a remote UE. This would give the best flexibility, otherwise, we add unnecessary restriction to the intermediate relay UE when forwarding discovery message.

RAN2 confirms that an intermediate relay UE is not required to establish itself as a remote UE first when forwarding discovery solicitation message.

In previous meetings, companies have proposed to discuss if the relay (re)selection by the remote UE can be further enhanced, considering additional criteria such as the number of hops and sidelink RSRP contribute to the quality of a path. In our view, such additional criteria can be either categorized as upper layer criteria i.e., not visible to the AS layer or can be well handled by the remote UE implementation. It is unnecessary to further discuss such optimization. In addition, another issue was also raised by companies i.e., a reselection should prioritize a relay with the next hop already established via an existing PC5-RRC connection. Similar as in the above, such criteria can be also categorized as optimization. No further

Not pursue further optimization to the relay (re)selection considering additional criteria such as number of hops, sidelink RSRP contribute to the quality of a path or whether there is existing PC5-connection to the next hop.

# 3	Conclusion

In the previous sections we made the following observations:

No table of figures entries found.

Based on the discussion in the previous sections we propose the following:

Proposal 1	In the sidelink notification message towards child UEs, the Intermediate Relay UE sets the indication type to the failure cause received from its parent relay UE, i.e., no new cause needed to reflect its own relay reselection action.

Proposal 2	RAN2 confirms that an intermediate relay UE is not required to establish itself as a remote UE first when forwarding discovery solicitation message.

Proposal 3	Not pursue further optimization to the relay (re)selection considering additional criteria such as number of hops, sidelink RSRP contribute to the quality of a path or whether there is existing PC5-connection to the next hop.

# 4	References

[1] RP-241609, “New WID on NR sidelink multi-hop relay”, 3GPP TSG RAN Meeting #104, Shanghai, China, 17 – 20 June 2024.