3GPP TSG-RAN2 Meeting #131	R2-2505820

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:	8.11.2

Source:	Qualcomm Incorporated

Title:	Views on random access for SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. One of the objectives is to specify SBFD operation to support random access in SBFD symbols for UE in RRC-connected UE while studying the benefit of extending the random-access operation in SBFD symbols to UE(s) in RRC idle and inactive state.

In RAN2 #127, the following agreements were made regarding random access for SBFD operation.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In RAN2 #127bis, the following agreements were made regarding random access for SBFD operation.

RAN2 understand that if additional RO is selected by SBFD-aware UE, early identification via Msg1 is possible from NW point of view for this UE without specification impact. From R2 point of view, there is no need to introduce SBFD as a new feature combination in the current PRACH preamble partitioning framework. Upon initiation of CBRA RACH procedure for a SBFD-aware UE, UE selects one type of ROs between legacy-ROs and additional-ROs based on certain specified/configured conditions/prioritizations, if no additional indication (FFS if there needs to be any) is from network. For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first. The following two RACH configuration options are considered for SBFD based random access: - Option 1: Use one single RACH configuration based on the existing parameters of the single RACH configuration. Can extend the existing parameters if needed. - Option 2: Use two separate RACH configurations, including one legacy RACH configuration and one additional RACH configuration For RACH configuration Option 2, RAN2 needs to specify RRC signalling for the new SBFD based RACH configuration with a new set of parameters. The RACH configuration for SBFD is transmitted via SIB1. FFS dedicated RRC signalling detail. FFS whether NW can provide both configurations.

RAN2 #128 made agreements related to random access for SBFD operation.

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

RAN2 #129 made agreements related to random access for SBFD operation.

When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

RAN #129bis has the following agreements related to random access of SBFD operation.

On RO type signaling for CFRA - 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. - 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. When both NW indication on RO type and RSRP threshold are absent, it is up to UE implementation to select the RO type. Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.

NW indicate via explicit signaling whether the SBFD RO is selected when SSB RSRP are 'below' or 'above' the configured threshold. Working assumption: For SBFD-aware UE, the selection of RO type is suggested to be performed before the selection of the set of Random Access resources. RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold (we assume it is the same threshold with the fallback from additional RO to legacy RO). If fallback from legacy RO to additional RO occurs, no further fallback to legacy RO is supported.

RAN #130 made the following agreements related to random access of SBFD operations.

Agreements on MAC open issues For L3 HO and BFR cases, CSI-RS based CFRA using SBFD RO is supported from RAN2 perspective. Send LS to RAN1/4 to inform this conclusion. Msg1 repetition number fallback can be supported within SBFD RO. Once the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch. FFS the Msg1 repetition number after RO type switch in this case. For RACH fallback from one RO type to another, the UE shall only be allowed to switch to an RO type that is configured with the same feature combinations. The UE is allowed to switch to an RO type that is configured with the same Msg1 repetition number. FFS on higher Msg1 repetition number, if the same is not available. Other agreements SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA. For PRACH configuration option 1, when a feature combination is configured using preamble partitioning of the RACH-ConfigCommon, using the shared ssb-SharedRO-MaskIndex configuration for Additional-ROs and Legacy-ROs. For PRACH configuration option 1, same featureCombinationPreamblesList configuration is used for both Additional-ROs and Legacy-ROs.

In this contribution, we would like to discuss the configurations of random-access resource in SBFD symbols for UE in RRC_CONNECTED and RRC_IDLE/INACTIVE.

# Discussion

[Open Issue MAC-2-1] Do companies think any additional MAC specification changes (compared to the latest version of the MAC running CR) are needed for capturing the condition of the agreement: SBFD-aware UE uses the CBRA resource with same RO type as indicated in CFRA resource when fallback from CFRA to CBRA is performed, when the RACH resources for the same RO type is provided for CBRA?

In the last RAN2 meeting, the reason to have this condition when RAN2 made this agreement was that some companies commented that if RACH resource for the RO type indicated by CFRA is not provided for CBRA how UE can select the CBRA with same RO type (as indicated in CFRA) when CFRA fallback to CBRA is performed. As a compromise, the Chair suggested adding this condition to make the agreement complete. It does not mean something must be added in the MAC spec.

In addition, in RACH resource selection procedure, the RO type should be determined in the RACH procedure initiation phase, and RACH occasions are selected during the selection of the set of RACH resources phase. Since RO type has been determined in the selection of the set of RACH resource phase, there is no additional chance to update or re-select the RO type later. Thus, no additional MAC specification changes are needed for now unless RAN2 made further agreements.

Proposal 1: Regarding the Open issue MAC 2-1, no additional MAC spec change is needed.

[Open Issue MAC-3-1] Do companies think any additional specification changes (in MAC and/or RRC) are essential to address the issue on how to derive the SBFD RO locations for CFRA (ZTE’s concern)? Please provide your input with the rationale.

In our understanding, it is a reasonable assumption that either sbfd-RACHSingleConfig or sbfd-RACHDualConfig should have been configured to UE by network, when network would like to indicate the SBFD RO in CFRA. The RAN1 spec [2] has specified the following.

If a random access procedure is initiated by a PDCCH order and the UE is provided either sbfd-RACHSingleConfig or sbfd-RACHDualConfig, the PDCCH order indicates [5, TS 38.212] whether the UE uses a PRACH occasion associated with either the first PRACH occasions or the second PRACH occasions, for the PRACH transmission that is initiated by the PDCCH order.

Thus, we have the following proposal.

Proposal 2: If network indicates the SBFD RO resource in CFRA, UE should be already configured either RACH configuration option 1 or option 2.

[Open Issue MAC-5] Do companies support introducing a power offset to compensate the power ramping difference between the two RO types (in SBFD RACH configuration Option 2) for RO type switching? Please provide your input with the rationale.

When a SBFD-aware UE switches from additional-ROs to legacy-ROs or from legacy-ROs to additional-ROs in PRACH transmission re-attempt in one RACH procedure, there may be a UE power gap due to the different power ramping steps configured in different RACH configurations.

RAN1 #120bis has the following conclusions.

Conclusion For RACH configuration Option 2, when a SBFD-aware UE switches from additional-ROs to legacy-ROs in PRACH transmission re-attempt in one RACH procedure, there is no RAN1 consensus to support a power offset to compensate the power ramping difference

From RAN2 perspective, UE would pick up the appropriate power control parameters based on the type of RO of the PRACH transmission. For example, if the RO type is initially selected as the additional-RO and switch to legacy-ROs after certain times of PRACH transmission, the UE would to use the preambleReceivedTargetPoweradditional-RO and PreamblePowerRampingStepadditinal-RO .

Then, when UE fall back to the legacy-RO after a number of PRACH attempts, UE would fall back to the power control parameters of the legacy-ROs and apply a power offset to compensate for the difference of the power step size and number of PRACH attempts.

Poffset=(N-1)×powerRampingStepadditional-RO-powerRampingSteplegacy-RO

In the other way around, when UE fall back from legacy-RO to the additional-RO, the power offset could be

Poffset=(N-1)×powerRampingSteplegacy-RO-powerRampingStepadditional-RO

The UE behavior is similar to the existing power offset used when 2-step fallback to 4-step RACH

2> if RA_TYPE is switched from 2-stepRA to 4-stepRA during this Random Access procedure: 3> set POWER_OFFSET_2STEP_RA to (PREAMBLE_POWER_RAMPING_COUNTER – 1) × (MSGA_PREAMBLE_POWER_RAMPING_STEP – PREAMBLE_POWER_RAMPING_STEP).

### 5.1.3 Random Access Preamble transmission The MAC entity shall, for each Random Access Preamble: <omit> 1> set PREAMBLE_RECEIVED_TARGET_POWER to preambleReceivedTargetPower + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER – 1) × PREAMBLE_POWER_RAMPING_STEP + POWER_OFFSET_2STEP_RA;

Proposal 3: When UE fallback to the other type of RO from the initial selected type of RO, a power offset given by the difference between the two quantities of preamble power step sizes is added.

RAN2 made the following agreements in the last meeting that when the conditions for both RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch first. There is FFS about the Msg1 repetition number after RO type switch.

Since the condition of Msg1 repetition number fallback are met, after UE performs RO type switch, UE should switch to the RACH resource set with the higher Msg1 repetition number.

Proposal 4: When the conditions of RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch first and then switch to the RACH resource set with the higher Msg1 repetition number.

There is an open issue that when non-SBFD RO 2-step RACH fall back to 4-step RACH, which RO type should the 4-step RACH be, since both of the RO types (SBFD RO and non-SBFD RO) may be configured in 4-step CBRA.

In our view, when UE fallback from 2-step to 4-step RACH, UE will not change the RO type. Since the 2-step RACH does not support SBFD resource, UE should fallback to non-SBFD 4-step RACH first. After several times attempting on the non-SBFD 4-step RACH and if conditions are met, UE is allowed to switch from non-SBFD 4-step RACH to SBFD 4-step RACH.

Proposal 5: UE will not change RO type when fallback from 2-step to 4-step RACH (i.e., non-SBFD resource).

Regarding whether to support SBFD with NR-DC, RAN1 has discussed the feasibility of SBFD with NR-DC and they further indicate that the specification impact to support this feature is not restricted to RAN1. The RAN1 conclusion is that it is possible to support SBFD with NR-DC with necessary UE capability support However, RAN1 LS further pointed out that there might be some specification impacts on RAN4, i.e., RAN4 spec TS38.101-1 may be impacted and need to be updated, and it seems that TS38.101-1 is not in the current Rel-19 SBFD WID. Thus, whether the RAN4 spec impact is critical or not and the potential WID update caused by RAN4 should be left to RAN4 for further discussion and decide.

From RAN2 perspective, there may be some spec clarification work on how to configure SBFD on the NR-DC case, and some impacts on the Xn coordination signaling. Thus, RAN2 can support simultaneous configuration of SBFD and NR-DC. It is up to RAN4 to decide whether the simultaneous configuration of SBFD and NR-DC is eventually supported in Rel-19.

Proposal 6: From RAN2 perspective, RAN2 can support simultaneous configuration of SBFD and NR-DC. Send LS to RAN4 to inform RAN2’s understanding and require RAN4 to evaluate the impacts and make decision.

RAN2 #129bis meeting made the following agreements that ‘Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for SI request. FFS for LTM.’. For RACH on SBFD resource for LTM, the following proposal was made by the rapporteur in the post meeting email discussion.

[Proposal for RRC-2] RAN2 to decide whether ‘Early UL synchronization with an LTM candidate cell’ and/or ‘RACH-based LTM’ can be supported in SBFD symbols and the RRC signallings if to support. (based on companies’ meeting contributions, 11/16 companies prefer not to support.)

In our view, there are two issues on whether supporting RACH on SBFD resource for LTM. One is whether RRC signaling is enhanced to support early UL synchronization with an LTM candidate cell on SBFD resource, while the other one is MAC CE enhancement to support CFRA triggered by the MAC CE for RACH-based LTM cell switch on SBFD resource.

For the latter one, it is similar to the PDCCH order triggered CFRA with an RO type indication to indicate UE performing on additional-RO. The source cell can indicate the type of RO in the LTM cell switch command MAC CE. Thus, we propose that an RO type indication is added in LTM cell switch command MAC CE to indicate whether RACH triggered by MAC CE can be performed on additional-RO.

For early UL synchronization with an LTM candidate cell, candidate cell may send EarlyUL-SyncConfig to source cell. Whether the candidate cell indicates type of RO in EarlyUL-SyncConfig to source cell or not is related to whether the EarlyUL-SyncConfig can be extendable to include the SBFD time and frequency information to source cell/UE and how to indicate the type of RO from the candidate cell may involve other WGs further study. Even if the source cell triggers UE to perform early UL synchronization on the additional-RO, the current enhancement on the PDCCH order to indicate the RO type to UE is only for the source cell but not for the candidate cell which may also need RAN1 further check.

Proposal 7: An RO type indication is added in LTM cell switch command MAC CE to indicate whether RACH triggered by MAC CE can be performed on additional-RO.

It is important for gNB to know the UE SBFD-aware capability in early stage of the random-access procedure to leverage the SBFD operation. The SBFD-aware indication is useful to inform the UE SBFD capability at the earliest time during RACH, e.g., network may schedule the resource for Msg3 in the SBFD resource in Msg2 if network can be aware the SBFD capable UE performing the RACH.

If network has configured the RACH occasions on SBFD symbols and UE also supports it, the SBFD-aware UE sends the Msg1 in the SBFD RACH occasions as the SBFD-aware indication to network. So, by using the SBFD RACH occasions can be already an implicit way as the SBFD-aware indication.

If the SBFD-aware UE selects the legacy RACH occasion, the additional SBFD-aware indication is needed. If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

Proposal 8: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: Regarding the Open issue MAC 2-1, no additional MAC spec change is needed.

Proposal 2: If network indicates the SBFD RO resource in CFRA, UE should be already configured either RACH configuration option 1 or option 2.

Proposal 3: When UE fallback to the other type of RO from the initial selected type of RO, a power offset given by the difference between the two quantities of preamble power step sizes is added.

Proposal 4: When the conditions of RO type fallback and Msg1 repetition number fallback are met, UE should perform RO type switch first and then switch to the RACH resource set with the higher Msg1 repetition number.

Proposal 5: UE will not change RO type when fallback from 2-step to 4-step RACH (i.e., non-SBFD resource).

Proposal 6: From RAN2 perspective, RAN2 can support simultaneous configuration of SBFD and NR-DC. Send LS to RAN4 to inform RAN2’s understanding and require RAN4 to evaluate the impacts and make decision.

Proposal 7: An RO type indication is added in LTM cell switch command MAC CE to indicate whether RACH triggered by MAC CE can be performed on additional-RO.

Proposal 8: If Msg1 based SBFD-aware indication is not supported, Msg3 based SBFD-aware indication should be supported from RAN2 perspective.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

- TS 38.213 V19.0.0, Physical layer procedures for control, 3GPP