3GPP TSG-RAN2 Meeting #131	R2-2505821

Bangalore, India, Aug 25th – 29th, 2025

Agenda item:	8.11.3

Source:	Qualcomm Incorporated

Title:	Other aspects of SBFD

WID/SID:	NR_duplex_evo-Core – Release 19

Document for:	Discussion and Decision

# Introduction

RAN plenary #102 approved the work item on NR duplex operation, Sub-Band Full Duplex (SBFD) and the latest WID is in [1]. The objective of the work item is to specify SBFD operation at the gNB side within a TDD carrier. The objectives include to specify the semi-static indication of time and frequency domain locations of the SBFD subbands to the UE and specify UE transmission, reception and measurement procedures in SBFD and non-SBFD symbols. The other objectives of the work item are to study the enhancements for CLI handling, and RAN2 will focus on the signaling design for UL resource muting for PUSCH, and L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework.

In RAN2 #127, the following agreements were made about other aspects of SBFD.

Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. RAN2 assume RACH configuration for SBFD via SIB and/or dedicated RRC signalling is supported. Detailed signalling FFS. RAN2 to strive for a common SBFD CBRA framework independent of RRC state. FFS whether/how early indication is used during a SBFD RA procedure. RAN2 focus on 4-step RACH for SBFD RA, FFS on 2-step if needed. Cell-specific SBFD time/frequency configuration is provided by SIB1 (or via dedicated signalling to covey cell specific configuration). FFS on UE specific dedicated RRC configuration if needed, pending on RAN1 progress.

In RAN2 #127bis, the following agreements were made about other aspects of SBFD.

For UL resource muting for PUSCH, the configuration of time and frequency location for UL resource muting should be introduced based on R1 agreement. For L1 based UE-to-UE CLI measurement mechanism, the configuration of periodic, semi-persistent or aperiodic UE-to-UE CLI measurement resource (set) should be introduced based on R1 agreement. For L1 based UE-to-UE CLI reporting mechanism, the configuration of report quantities should be introduced based on R1 agreement.

RAN2 #128 made the following agreements about the other aspects of SBFD.

Prioritization of SBFD cells / frequencies during cell reselection is not considered. RAN2 wait for input from the other WGs regarding whether for inter-cell CSI-RS measurements, UE needs to be provided with information of the SBFD configuration of neighbouring cells.

RAN2 #129 has agreements about the other aspects of SBFD.

A new SP CLI measurement resource set activation/deactivation MAC CE is introduced to activate/deactivate the SP CLI measurement resource.

RAN2 #129bis made the following agreements about the other aspects of SBFD.

Send an LS to RAN1 (CC RAN3 and RAN4) on whether SBFD and DC can be configured simultaneously, and whether there is any issue for such configuration. Working assumption: The configured SP CLI measurement resource sets are initially deactivated upon (re-) configuration by upper layers and after reconfiguration with sync. SP CLI measurement resource set activation/deactivation MAC CE includes following fields: A/D, Serving Cell ID, BWP ID, CLI measurement resource set ID (for CLI-RSSI or SRS-RSRP measurement), TCI State IDs.

RAN2 #130 made the following agreements about the other aspects of SBFD.

Agreements on other aspects RAN2 confirm that the legacy Aperiodic CSI Trigger State Subselection MAC CE is also used to indicate the selection status of the Aperiodic Trigger States configured for aperiodic CLI measurement report. No spec change is needed. No enhancement is needed for CSI-RS based RLM/BFD/CBD measurements. Keep the legacy RLM, BFD and BFR procedure, i.e., no need to trigger RLF, BFD and BFR for non SBFD symbols and SBFD symbols separately.

In this contribution, we would like to discuss the configuration of SBFD time and frequency locations, the configuration signaling of the L1 based inter-UE CLI measurement and reporting, and RAN2 impacts on transmission, reception in SBFD symbols.

# Discussion

According to RAN1 updated high-layer parameters list [2], the sbfd-RACH-SingleConfig should be cell-specific and explicit indicated in the BWP-UplinkCommon.

| sbfd-RACHSingleConfig | New |  | Indicates whether RACH configuration Option 1 for SBFD random access operation is enabled or not from network side | {enabled} | If the field is absent, RACH configuration option 1 is not enabled. | [Per Cell] In BWP-UplinkCommon | Cell-specific |
| --- | --- | --- | --- | --- | --- | --- | --- |

If the sbfd-RACH-SingleConfig is used to indicate the RACH configuration per RACH-ConfigCommon, we fail to observe the motivation and support of such flexibility from RAN1 and RAN2 should check with RAN1 on whether support such intention. According to the RAN1 parameter list, the RACH configuration option (either RACH configuration option 1 or RACH configuration option 2) is configured in cell level.

Proposal 1: The sbfd-RACH-SingleConfig-r19 should be placed per BWP indication (under the BWP-UplinkCommon).

RAN1 and RAN2 has made the following agreements.

RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

To support only one RACH configuration option configured in a cell, the CHOICE structure should be used here to make sure only one of them can be included at one time.

Further, to align the RACH configuration option in the AdditionalRACH-Config-r17 and the indication under the BWP-UplinkCommon, the conditional code should be used for the sbfd-RACH-DualConfig-r19 under the AdditionalRACH-Config-r17.

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...

[[

sbfd-RACH-DualConfig-r19            SBFD-RACH-DualConfig-r19                                            OPTIONAL  -- Cond NoSingleConfig

]]

}

| NoSingleConfig | This field is optionally present, Need R, if sbfd-RACH-Config in BWP-UplinkCommon is set to sbfd-RACH-DualConfig. It is absent otherwise. |
| --- | --- |

Thus, we have following proposal.

Proposal 2: The CHOICE structure should be used to make sure only one of RACH configuration can be included at one time, and the conditional code is used for the sbfd-RACH-DualConfig-r19 under the AdditionalRACH-Config-r17 to align with the indication under the BWP-UplinkCommon.

RAN1 #120 made the following agreement regarding the RRC parameter carrier in CSI-ReportConfig and bwp-Id in the associated CSI-ResourceConfig.

Agreement For a CSI report with CSI-ReportConfig with higher layer parameter reportQuantity set to ‘cli-RSSI’ or ‘cli-SRS-RSRP’ - carrier in CSI-ReportConfig is used to indicate in which serving cell the CLI-RSSI measurement resources or SRS-RSRP measurement resources in CSI-ResourceConfig are to be found. If the field is absent, the resources are on the same serving cell as this report configuration. - bwp-Id in the associated CSI-ResourceConfig is used to indicate the DL BWP where the CLI-RSSI measurement resources or SRS-RSRP measurement resources are located in. Note: No RAN1 specification impact

According to RAN1 agreements, the description of carrier in CSI-ReportConfig and bwp-Id in the associated CSI-ResourceConfig should be associated with the corresponding CLI measurement resource configuration. The description of these two parameters in the current RRC spec is related to CSI resource.

Proposal 3: The RRC parameters of carrier in CSI-ReportConfig and bwp-Id in the associated CSI-ResourceConfig are reused for CLI measurement resource configurations. The description of these two RRC parameters is updated accordingly to associate with the CLI resource.

# Conclusion

We have the following observations, and we’d recommend RAN2 to discuss and adopt the following proposals:

Proposal 1: The sbfd-RACH-SingleConfig-r19 should be placed per BWP indication (under the BWP-UplinkCommon).

Proposal 2: The CHOICE structure should be used to make sure only one of RACH configuration can be included at one time, and the conditional code is used for the sbfd-RACH-DualConfig-r19 under the AdditionalRACH-Config-r17 to align with the indication under the BWP-UplinkCommon.

Proposal 3: The RRC parameters of carrier in CSI-ReportConfig and bwp-Id in the associated CSI-ResourceConfig are reused for CLI measurement resource configurations. The description of these two RRC parameters is updated accordingly to associate with the CLI resource.

# References

- RP‑241614, Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

- R1-2503243, Consolidated Rel-19 higher layers parameters list Post RAN1#121, Moderator (Ericsson).