3GPP TSG-RAN WG2 #130	R2-2505824

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.9.4

Source:	Ericsson

Title:	Enhancements to support PWS in NB-IoT NTN

Document for:	Discussion, Decision

# 1	Introduction

The Rel-19 WID on IoT-NTN [1] was updated in RAN#105 with the following additional objective:

- Support broadcast of PWS messages for NB-IoT re-using the LTE mechanisms [RAN2]

Note: The solutions will be developed for NTN but can be applicable to TN (if possible). Specific NB-IoT TN optimizations will not be considered as part of this WI.

Note: Support for indication of intended service area for ETWS will be considered based on NR NTN discussions/outcome.

In this contribution we focus on some of the open issues listed by the IoT NTN RRC rapporteur.

# 2	Discussion

## 2.1	Enhancements to eDRX

Clarify in the spec (and at least in 36.300) that for NB-IoT the ETWS, CMAS, PWS requirement may not be met in some scenarios, e.g. when the UE is in eDRX (can come back to further clarify the specific cases).

During RAN2#127bis, it was agreed to include a disclaimer on PWS performance in eDRX as follows:

While such a disclaimer is reasonable for specific scenarios, it is important to emphasize that PWS is a safety-critical mechanism and should not be treated as a best-effort service. This could lead to dangerous situations where users expect to receive warning messages, but the UE is not actively monitoring these notifications or changes in broadcast signaling during emergencies. In the initial commercial deployments of NTN, there is a clear preference for using NB-IoT to provide messaging and emergency services via satellite [2]. These UEs might prioritize better service delivery over energy-saving configurations.

- First commercial NB-IoT NTN UEs may not be battery constrained and may prioritize better service delivery.

An example of improved service delivery is the timely reception of PWS emergency notifications. As discussed earlier, this is challenging or even impossible when an NB-IoT UE employs certain power-saving mechanisms like eDRX. Therefore, it may be advantageous for such NB-IoT UEs to inform the network that they are not battery-constrained and are interested in specific services, such as PWS. This mechanism already exists for regular LTE UEs using UE Assistance Information (powerPrefIndication parameter), but it does not exist in NB-IoT. Based on the UE's preferences, the network can adjust the assigned power-saving configurations to align with the UE's specific interests, such as receiving timely PWS notifications.

A NB-IoT UE interested in receiving PWS may indicate its interest to the network to adjust the assigned power saving configuration.

## 2.2	Reception of PWS segments when switching serving cell

Segmentation can be used in delivering ETWS secondary and CMAS notifications. Upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment, or if the received value of either messageIdentifier or serialNumber or both are different from the current stored values, the UE discards any previously buffered warningMessageSegment. In NTN NGSO scenarios, the serving satellite changes frequently due to its movement. As a result, the UE may not receive all warningMessageSegments before such a switch occurs. With the current discard mechanism, the NB-IoT UE may therefore fail to receive ETWS secondary or CMAS notifications.

- Frequent cell changes inherent to NTN lead the UE to discard warningMessageSegment and potentially result in failure to receive emergency notifications.

A related scenario arises when a NB-IoT UE transitions from RRC_CONNECTED to RRC_IDLE to acquire an emergency notification. If the remaining service time of the NGSO satellite is insufficient, the UE may face significant delays in receiving the notification or may not receive it at all.

- A UE released to RRC_IDLE may not be able to acquire an emergency notification within the remaining service window of the serving satellite.

To address these challenges, the UE should be permitted to assemble warningMessageSegments received from different cells or satellites in the specific case where they remain connected to the same gNB. For example, when multiple cells are served by the same satellite or when satellites covering the same area are connected to the same gNB. Under these conditions, the network can coordinate the delivery of warningMessageSegment with consistent messageIdentifier, serialNumber, and segment size. In such scenarios, the UE should be explicitly informed whether to discard (as per legacy behavior) or retain previously buffered warningMessageSegments.

A UE is allowed to assemble warningMessageSegments received from different cells or satellites but served by the same gNB.

The network can indicate whether the UE should discard or continue using the previously buffered warningMessageSegments upon cell or satellite switch.

A simple way to realize this in practice is to rely on the warningMessageSegment size. Upon cell reselection, the UE should first attempt to process the PWS related SIB without discarding stored PWS segments. Provided the messageIdentifier and serialNumber in the decoded PWS SIB are identical, the UE checks if the size of the warningMessageSegment is equal to the last stored segment from the previous cell. In this case, the UE continues PWS reception without discarding stored segments. Otherwise, stored segments should be discarded. A text proposal is detailed in section 3

Upon network indication, the UE can continue PWS reception without discarding stored segments if the messageIdentifier, the serialNumber and the size of the warningMessageSegment in the new cell’s PWS SIB are identical to those associated with the stored PWS segments.

## 2.3	Accumulation of SI message across SI windows for PWS

In RAN2#131, RAN2 discussed the applicability of SI accumulation to the acquisition of NTN related SIBs. Provided the contents of these SIBs can change without triggering the SI update, the UE does not know whether it can accumulate SI transmissions across SI windows. Despite having a detrimental effect on UEs in deeper coverage, RAN2 reached the following understanding last meeting, in line with a RAN1 prior understanding:

- RAN2 understands that the UE is not required to perform SI accumulation across SI windows when acquiring SIB31(-NB) and SIB33(-NB) (no need for spec change).

Another example of SIBs that may change their contents without triggering the SI update procedure are PWS related SIBs. Depending on its size, an emergency message may be delivered to the UE in separate segments (e.g., ETWS secondary notification). Each segment will be transmitted sequentially and it may be sent in consecutive SI windows.

- PWS related SIBs do not trigger the SI update procedure.

A UE in enhanced coverage may require several SI windows to decode the contents of a SIB. The amount of SI windows largely depends on the radio conditions, SIB size and SI scheduling parameters. According to previous simulations [3], it may take up to 4 SI windows for a UE to decode a 200-bit SIB in challenging radio conditions. Therefore, the SI accumulation feature may become highly relevant for those UEs to be able to receive critical information such an emergency notification.

- A UE in enhanced coverage may require accumulating SI transmissions across several SI windows to decode a SIB.

Provided the critical nature of PWS notifications, RAN2 should have a clear understanding whether SI accumulation can be used for the reception of PWS related SIBs. Noting that it may take several SI windows for UEs in deeper coverage to obtain a SIB, we consider beneficial to enable the use of the SI accumulation feature for PWS notifications.

A NB-IoT UE can use SI message accumulation across SI windows to obtain a PWS notification.

As stated earlier, SI accumulation can only work if the contents of a SIB do not change, but the UE is unaware of this since PWS messages do not trigger the SI update procedure. If the network wants to help UEs in deeper coverage by transmitting the same emergency segment across two or more consecutive SI windows, the UE needs to know in which SI transmissions it can do the SI accumulation. One simple way forward is to provide the scheduling information of the segments, for example, the total number of segments and the number of SI windows where a specific segment is transmitted. This assistance information can potentially streamline the reception of large emergency messages and avoid decoding errors when the UE tries to combine segments with different contents.

Provide the scheduling information of PWS segments to facilitate SI accumulation.

# 3	Text Proposal TS 36.331

- 5.2.2.4	System information acquisition by the UE

- …

1>	if the UE is ETWS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment or upon mobility between TN and NTN:

3>	if is PWS-segmentAccumulation is not enabled:

4>	discard any previously buffered warningMessageSegment;

4>	clear, if any, the current values of messageIdentifier and serialNumber for SystemInformationBlockType11(-NB);

- …

1>	if the UE is CMAS capable:

2>	upon entering a cell during RRC_IDLE, following successful handover or upon connection re-establishment or upon mobility between TN and NTN:

3>	if is PWS-segmentAccumulation is not enabled:

4>	discard any previously buffered warningMessageSegment;

4>	discard any previously buffered warningAreaCoordinatesSegment (if any);

4>	clear, if any, stored values of messageIdentifier and serialNumber for SystemInformationBlockType12(-NB) associated with the discarded warningMessageSegment and warningAreaCoordinatesSegment (if any);

- …

#### 5.2.2.18	Actions upon reception of SystemInformationBlockType11

Upon receiving SystemInformationBlockType11(-NB), the UE shall:

1>	if there is no current value for messageIdentifier and serialNumber for SystemInformationBlockType11(-NB); or

1>	if either the received value of messageIdentifier or of serialNumber or of both are different from the current values of messageIdentifier and serialNumber for SystemInformationBlockType11(-NB), or if the size of the received warningMessageSegment is different from the last buffered warningMessageSegment:

2>	use the received values of messageIdentifier and serialNumber for SystemInformationBlockType11(-NB) as the current values of messageIdentifier and serialNumber for SystemInformationBlockType11(-NB);

2>	discard any previously buffered warningMessageSegment;

2>	if all segments of a warning message have been received:

3>	assemble the warning message from the received warningMessageSegment;

3>	forward the received warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

3>	stop reception of SystemInformationBlockType11(-NB);

3>	discard the current values of messageIdentifier and serialNumber for SystemInformationBlockType11(-NB);

2>	else:

3>	store the received warningMessageSegment;

3>	continue reception of SystemInformationBlockType11(-NB);

1>	else if all segments of a warning message have been received:

2>	assemble the warning message from the received warningMessageSegment;

2>	forward the received complete warning message, messageIdentifier, serialNumber and dataCodingScheme to upper layers;

2>	stop reception of SystemInformationBlockType11(-NB);

2>	discard the current values of messageIdentifier and serialNumber for SystemInformationBlockType11(-NB);

1>	else:

2>	store the received warningMessageSegment;

2>	continue reception of SystemInformationBlockType11(-NB);

#### 5.2.2.19	Actions upon reception of SystemInformationBlockType12(-NB)

Upon receiving SystemInformationBlockType12(-NB), the UE shall:

1>	if the SystemInformationBlockType12(-NB) contains a complete warning message and the complete geographical area coordinates (if any):

2>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and the geographical area coordinates (if any) to upper layers;

2>	continue reception of SystemInformationBlockType12(-NB);

1>	else:

2>	if the received values of messageIdentifier and serialNumber are the same (each value is the same) as a pair for which a warning message and the geographical area coordinates (if any) are currently being assembled:

3>	if the size of the buffered warningMessageSegment is different from the size of the last received warningMessageSegment:

4>	discard any previous buffered warningMessageSegment (if any);

4>	discard any previous buffered warningAreaCoordinatesSegment (if any);

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	if all segments of a warning message and geographical area coordinates (if any) have been received:

4>	assemble the warning message from the received warningMessageSegment;

4>	assemble the geographical area coordinates from the received warningAreaCoordinatesSegment (if any);

4>	forward the received warning message, messageIdentifier, serialNumber, dataCodingScheme and geographical area coordinates (if any) to upper layers;

4>	stop assembling a warning message and warning area coordinates (if any) for this messageIdentifier and serialNumber and delete all stored information held for it;

3>	continue reception of SystemInformationBlockType12(-NB);

2>	else if the received values of messageIdentifier and/or serialNumber are not the same as any of the pairs for which a warning message is currently being assembled:

3>	start assembling a warning message for this messageIdentifier and serialNumber pair;

3>	start assembling the geographical area coordinates (if any) for this messageIdentifier and serialNumber pair;

3>	store the received warningMessageSegment;

3>	store the received warningAreaCoordinatesSegment (if any);

3>	continue reception of SystemInformationBlockType12(-NB);

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	First commercial NB-IoT NTN UEs may not be battery constrained and may prioritize better service delivery.

Observation 2	Frequent cell changes inherent to NTN lead the UE to discard warningMessageSegment and potentially result in failure to receive emergency notifications.

Observation 3	A UE released to RRC_IDLE may not be able to acquire an emergency notification within the remaining service window of the serving satellite.

Observation 4	PWS related SIBs do not trigger the SI update procedure.

Observation 5	A UE in enhanced coverage may require accumulating SI transmissions across several SI windows to decode a SIB.

Based on the discussion in the previous sections we propose the following:

Proposal 1	A NB-IoT UE interested in receiving PWS may indicate its interest to the network to adjust the assigned power saving configuration.

Proposal 2	A UE is allowed to assemble warningMessageSegments received from different cells or satellites but served by the same gNB.

Proposal 3	The network can indicate whether the UE should discard or continue using the previously buffered warningMessageSegments upon cell or satellite switch.

Proposal 4	Upon network indication, the UE can continue PWS reception without discarding stored segments if the messageIdentifier, the serialNumber and the size of the warningMessageSegment in the new cell’s PWS SIB are identical to those associated with the stored PWS segments.

Proposal 5	A NB-IoT UE can use SI message accumulation across SI windows to obtain a PWS notification.

Proposal 6	Provide the scheduling information of PWS segments to facilitate SI accumulation.

# 4	References

RP-242397, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, RAN#105, Sep. 2024.

https://samsungmagazine.eu/en/2024/04/19/samsung-oficialne-predstavil-svuj-prvni-5g-modem-s-obousmernym-satelitnim-pripojenim/

R4-1609680, Ericsson, Simulation result of MIB-NB/SIB1-NB/SIB2-NB acquisition time, RAN4#81, USA, November, 2016.