3GPP TSG-RAN WG2 #131	R2-2505829

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.2.4

Source:	Ericsson

Title:	A-IoT Data Transmission and Other General Aspects

Document for:	Discussion/Decision

# 1	Introduction

This paper discusses some of remaining aspects of data transmission functionality and other general aspects required for A-IoT devices, most of which are listed in the remaining A-IoT MAC open issue email discussion [POST130][027][AIoT] MAC Running CR, and also partially captured in the draft MAC running CR (TS38.391 v0.0.2).

# 2	Discussion

## 2.1	D2R MAC PDU format and content

- Issue 3-5 whether to support D2R message type

Regarding the open issue 3-5 in the OI list, whether to support D2R message type, we think Msg1 in CBRA contains RN16 and thus can be distinguished from other D2R message including first D2R message in CFRA; given that all other D2R messages are larger in size. In addition, given there is no coexistence of both device 1 and active devices in the same reader according to Rel-20 WID, we do not see the need to distinguish the Rel-19 D2R messages from those of A-IoT in future releases. Thus, we think there is no need for the message type field in the MAC header for D2R. If this construction is possible, the message size would be small.

No need for a message type field in D2R message.

RAN2 assumed that segmentation is only needed for D2R messages other than Msg3 and Msg1, e.g., Msg5. However, SA3 and CT1 concluded that Msg3 needs to carry NAS information including not only device identity, but also security parameters for authentication procedure (as detailed in TS24.369, also copied below). While the exact size of security parameters is to be decided by SA3, from TS33.369, mandatory security parameters include RESAIOT and RANDAIOT_d, both of which are supposed to be large, e.g., 128 bits and 64 bits, respectively. SA3 also discusses whether the authentication parameter selected by network and signalled in the paging message RANDAIOT_n needs to be included in the inventory report as well.

4. Upon receiving the paging request message, AIoT device shall derive RESAIOT using KAIoT and RANDAIOT_n for network authenticating AIoT Device.

Editor’s Note: How RESAIOT is derived and whether it is derived from KAIoT or intermediate key is FFS.

From A-IoT NAS specification TS24.369-v0.1.0, security parameters are mandatory information in the inventory report from device to network:

Table 7.1.2.1-1: INVENTORY REPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Message type | Message type 7.2.2 | M | V | 1 |
|  | Security parameters | Security parameters 7.2.3 | M | V | n |
|  | AIoT device identity | AIoT device identity 7.2.x | M | V | n |

We understand from TS33.369 that SA3 assumes device identity is implicitly reported from the security information in the Inventory report. In case the security parameters are with fixed size, RAN2 can assume the D2R Upper layer data transfer message in Msg3 is always known.

Msg3 D2R upper layer data transfer might be large considering mandatory security parameters, whose sizes are to be decided by SA3.

RAN2 assumes the fixed size of security parameters and device identity is implicit from security information and thus D2R Upper layer data transfer message size is known.

## 2.2	R2D MAC PDU format and content

Our views on the content of paging message and R2D Access trigger message and the Random ID Response message are detailed in other contributions, i.e., [7] and [8], respectively. This section focuses on two open aspects of data transmission, i.e., issue 3-3 AS ID release, issue 3-7 how to set value for the “more data indication” in case of no NAS response available, and issue 3-8 how to handle R2D TBS.

- Issue 3-3 AS ID release

In the email discussion, companies are invited to share their view on whether a release message is needed and if so, which option is preferred:

- Opt1. Unicast message (only one AS ID)

- Opt2. Multiplexing with a list of AS ID (NACK message-like)

- Opt3. Broadcast message (without device AS ID)?

Currently, devices release AS ID upon reception of a paging message, either CFRA or CBRA with a new transaction ID. To further enable devices to unnecessarily keep the AS ID long, we think it would be beneficial to have other trigger(s) for AS ID release.

For CBRA case, there are still two cases remaining and need to be further discussed

Case 1 - when a device receives a NACK message from the reader for triggering re-access,

Case 2 – after the current paging, there is relative long time to the next paging message associated with different transaction ID.

For case 1, upon reception of the NACK message addressed to its AS ID, the device waits for the subsequent paging message to perform re-access. Based on the current RAN2 agreement, the device will release its AS ID until receiving a subsequent paging message. Hence, we believe, for scenario with case 1, the reader can utilize existing R2D NACK signalling to release AS ID, or send a new explicit R2D message after NACK message to indicate release of AS ID. The behaviour for re-access applies to specific devices, thus, whether to use NACK message or new R2D message to indicate release, it should be addressed to specific AS IDs.

For case with scenario 2, it may be insufficient solely relying on paging message to release AS ID, hence, it may be suitable to define a new explicit R2D broadcast message to release AS ID. If interval/period between two subsequent paging message is very long or there is no subsequent paging message after a paging message is received, the reader and device(s) would need to keep the AS context for devices for too long time resulting in radio resource wastage. And the reader can send this release message at the end of a paging round.

Based on above discussion, we have following observation and proposals.

To release AS ID, there are two observable cases:

- Re-access case: When NACK is sent to a device to trigger re-access,

- Non-re-access case: When device may receive subsequent new paging message after transmitting in existing paging round.

Utilizing NACK message to release AS ID in case of re-access is fast and saves energy cost at device side.

Upon reception of NACK message addressed to the device, its AS ID is released.

Introduce a new R2D broadcast message (without device AS ID) with a message type for releasing AS ID.

- Issue 3-7 more data indication

Given that the “more data indication” field is always there and value 1 means more data to come while value 0 means no more data. We think that setting value to 1 will imply that the device needs to buffer the NAS PDU in its temporary memory until it has the next opportunity for (re)transmission. In addition, since the reader does not know in how long the delayed D2R NAS container will be available at the device side, it cannot determine when to allocate the resources for subsequent D2R transmission. Even if the device wants to indicate such information to the reader, the device itself cannot determine when the NAS container would be available. In other words, it is not beneficial for the device to indicate “1”, i.e., more data coming. It is proposed that device discards the late NAS PDU and set the value of the “more data indication” field to 0.

As discussed in the email discussion, we think this TBS is only needed for R2D messages with variable size. Considering the fixed size of Access trigger message, and thus

Set the value of “more data indication” field to 0 in case of no NAS response available.

Device’s MAC sublayer discards any late NAS PDU.

- Issue 3-8 how to handle R2D TBS

As discussed in the email discussion, we think this TBS is only needed for R2D messages with variable size. Considering that Access trigger message is with a fixed size, TBS field is needed in other R2D messages including Paging message, Random ID Response message, R2D Upper Layer Data Transfer message and the NACK Feedback message. Regarding the size of the padding fields, given the R2D TBS can be up to 1000 bits, we think 8 bits are needed to cover the large R2D message size, e.g., Random ID Response message with multiple entries.

TBS field is needed in R2D messages except for the Access Trigger message. Support 8 bits for R2D TBS field size.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Msg3 D2R upper layer data transfer might be large considering mandatory security parameters, whose sizes are to be decided by SA3.

Observation 2	To release AS ID, there are two observable cases:

(i)	Re-access case: When NACK is sent to a device to trigger re-access,

(ii)	Non-re-access case: When device may receive subsequent new paging message after transmitting in existing paging round.

Observation 3	Utilizing NACK message to release AS ID in case of re-access is fast and saves energy cost at device side.

Based on the discussion in the previous sections we propose the following:

Proposal 1	No need for a message type field in D2R message.

Proposal 2	RAN2 assumes the fixed size of security parameters and device identity is implicit from security information and thus D2R Upper layer data transfer message size is known.

Proposal 3	Upon reception of NACK message addressed to the device, its AS ID is released.

Proposal 4	Introduce a new R2D broadcast message (without device AS ID) with a message type for releasing AS ID.

Proposal 5	Set the value of “more data indication” field to 0 in case of no NAS response available.

Proposal 6	Device’s MAC sublayer discards any late NAS PDU.

Proposal 7	TBS field is needed in R2D messages except for the Access Trigger message. Support 8 bits for R2D TBS field size.

# 4      References

RP-250796, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #107, Incheon, South Korea, March, 2025.

[POST130][027][AIoT] MAC Running CR (Huawei).

TS38.391, V0.0.2, “Ambient IoT MAC protocol specification”.

TS24.369, V0.2.0, “Ambient IoT NAS protocol for 5GS specification”.

TS23.369, V19.0.0, “Architecture support for Ambient power-enabled IoT”.

TS33.369, V0.2.0, “Security aspects of Ambient IoT devices”.

R2-25XXXX, “Discussion on DL messages for Ambient IoT UEs”, Ericsson, RAN2#131, Bengaluru, India, August 25th – 29th, 2025.

R2-25XXXX, “UL multiple access for Ambient IoT”, Ericsson, RAN2#131, Bengaluru, India, August 25th – 29th, 2025.