3GPP TSG-RAN WG2 #131					      		 R2-2505844

Bengaluru, India – August 25 - 29, 2025

Agenda item:	8.13.2

Source: 	Kyocera

Title: 	Relay reselection and discovery under multihop relay

Document for:	Discussion

# Introduction

In this contribution, we consider the some of the key FFS issues to address for relay reselection and discovery under U2N multihop relay.

# Discussion

## 2.1	Relay reselection

The following FFS issues from RAN2#130 are related to the notification sent by the intermediate relay UE as part of relay reselection.

Agreements:

Notification is always sent for AS failure cases at the intermediate relay UE (e.g., upstream RLF that prevents the intermediate relay from having a path to the network), using the legacy mechanism.  FFS new cause values.

In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed. FFS which parameters are related.  This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified.

FFS if notification behaviour can be optimized for intermediate relay UEs in RRC_CONNECTED.

2.1.1	RRC IDLE/INACTIVE scenarios

During the discussion in RAN2#130, the following aspects related to IDLE/INACTIVE are already agreed:

In principle, RAN2 consider that no AS-layer notification is sent by an intermediate relay UE in RRC_IDLE/RRC_INACTIVE when the AS-visible parameters that could affect relay reselection are not changed. FFS which parameters are related. This issue can be revisited if it is found that there are parameters not visible to the AS layer that would affect reselection and force the remote UE to be notified.

One issue that should be discussed is if the remote UE transmitted the SRB0 message (RRC Setup Request) to its PC5 connected first relay UE that is still in IDLE/INACTIVE.  If the first relay UE detects a PC5 RLF or it receives a notification from its parent relay UE due to AS connection failure, and AS-layer notification is not sent to the remote UE, the remote UE may need to wait until T300 expiry before relay reselection will be performed.  In our view, at least the following two options should be considered:

- Option 1: The SRB0 message can be stored in the first relay UE and sent to the serving cell after successful relay reselection and RRC Connection is established.

- Option 2: The first relay UE sends AS-layer notification to the remote UE and the remote UE may perform relay reselection and send the SRB0 message towards the first relay UE on the reselected path.  It is assumed the SRB0 message received by the first relay UE on the previous path will be discarded.

In our view, either option can work; however, Option 2 may be better aligned with the legacy mechanism.

- Proposal 1	In case the first relay UE in IDLE/INACTIVE receives RRC Setup Request from its child UE, it should be further discussed whether the first relay UE needs to send AS-layer notification to its child UE upon AS failure detection/notification in the upstream path.

2.1.2	RRC connected scenarios

During RAN2#130 discussions, the following agreement for RRC CONNECTED case was reached:

Notification is always sent for AS failure cases at the intermediate relay UE (e.g., upstream RLF that prevents the intermediate relay from having a path to the network), using the legacy mechanism.  FFS new cause values. FFS if notification behaviour can be optimized for intermediate relay UEs in RRC_CONNECTED.

With regards to the second highlighted FFS above on the notification behaviour, it should be further considered for the RRC Connected scenario, that the relay reselection initiation for the parent relay UE is not much different from the case for the IDLE/INACTIVE scenario. For the RRC Connected remote UE, the main issue is whether it is always necessary for the remote UE to send the RRC Re-establishment Request message to the gNB regardless of which entity, the first relay UE or the child UE selected the new path. If the first relay UE initiates relay reselection towards the same cell with the same number of hops (or fewer), there is no need for the remote UE to send the RRC re-establishment request.  The first relay UE may need to inform the serving cell that it is still PC5 connected to a remote UE, but if the remote UE’s RRC re-establishment is not needed towards the same cell, it won’t be necessary for the first relay UE to send the notification to the remote UE, since the existing SL-RLC configuration and SRAP configuration (i.e., Local ID) can be re-used for the remote UE and PDCP may be continued without re-establishment. It should be further considered if this is applicable not just for the same cell case, but also if the target cell belongs to the same gNB. Additionally, the first relay UE can verify from the last relay UE whether the RSC provided in the discovery message remain unchanged. In Figure 1, the signalling flowchart illustrates the condition for the first relay UE’s RRC re-establishment procedure, whereby it is assumed the notification to the remote UE can be skipped.

- Proposal 2	The first relay UE should not need to notify the remote UE  if the first relay UE completes the RRC re-establishment procedure towards the same cell with the same number of hops (or fewer).

It may also be further discussed, if it’s necessary for the intermediate relay UE to inform the remote UE to suspend UL data transmission until its connection re-establishment is completed.

- Proposal 3	It may be further discussed whether it is necessary for the first relay UE to notify the remote UE for the purpose of suspending UL data transmission prior to the intermediate relay UE’s completion of RRC re-establishment.

With regards to the first highlighted FFS above on new cause values, we should consider the case when the first relay UE performs its own relay reselection prior to sending AS-layer notification to the remote UE.  If the first relay UE’s relay reselection is successful and it notifies the remote UE of this outcome, the remote UE would not need to release the existing PC5 link to the first relay UE and further relay reselection by the remote UE may be unnecessary.  In particular, if the PC5 connection between the first relay UE and the remote UE is not released, the first relay UE would perform relay reselection followed by RRC reestablishment.

- Proposal 4	If the PC5 connection between first relay UE and the remote UE is not released when the first relay UE receives notification of AS failure, the first relay UE would perform relay reselection followed by RRC reestablishment.

Assuming Proposal 4 is agreeable, a new cause value “path recovery” in the AS-layer notification from the first relay UE would be helpful to the remote UE’s decision in performing relay reselection. If the remote UE decides to use the recovered path, the remote UE would be required to send its own RRC reestablishment to the cell associated with the recovered path.

- Proposal 5	Upon AS layer failure, the RRC CONN first relay UE may include the cause value “path recovery” in the AS layer notification send to the remote UE.

## 2.2	RRC container in Discovery message

During RAN2#130, the following working assumption was reached:

Working assumption:

The Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container.  As a baseline, remote UE relay (re)selection behaviour based on this information is left to implementation, and it can be discussed in spec implementation/maintenance if some guidance is needed.  No SA2 spec impact is expected.

Companies are asked to confirm with SA2 colleagues if there is any concern with a candidate first relay UE sending the RRC container with its own information.

It was considered by some companies that an alternative to the working assumption above is to allow the gNB to ensure that most if not all relay UEs remain in RRC CONNECTED.  However, keeping all relay UEs in RRC CONNECTED seems overly restrictive to both gNBs and UEs esp. considering that many relay UEs may not be serving any remote UEs. Relay UEs are initially designed to be allowed to transmit discovery messages under all RRC states.

- Proposal 6	Confirm the working assumption that Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container.

If Proposal 6 is confirmed, when a remote UE receives a discovery broadcast from a candidate relay UE with “RRC CONNECTED” indication it may be assumed all relay UEs on the path towards the gNB can be considered to be “RRC CONN” since Approach 1 is assumed.

- Observation 1	Once a remote UE receives a discovery broadcast from a candidate relay UE with “RRC CONNECTED” indication it may be assumed all relay UEs on the path towards the gNB can be considered to be “RRC CONN” since Approach 1 is assumed.

One issue that should be further considered is whether the remote UE should be informed when the first relay UE is no longer RRC CONNECTED prior to the remote UE’s RRC connection establishment. Although the discovery message is usually transmitted periodically, it should be considered whether the first relay UE is allowed to inform the remote UE of the change in RRC state prior to the next discovery transmission occasion.  Currently, in [1], for update of additional parameters it is stated:

Detection of new or updated additional parameters in step 5 triggers the 5G ProSe UE-to-Network Relay to announce the additional parameters by sending a Relay Discovery Additional Information Message immediately and to repeat it periodically with a configurable frequency as in step 4 until there are no UEs requesting to announce the additional parameters, i.e. until the Additional_Parameters_Announcement_Request_Refresh Timer expires in the 5G ProSe UE-to-Network Relay.

Given the above description, it could be interpreted that changes in RRC state could also allow relay UEs to transmit discovery message immediately, while it may be left to upper layer implementation.

- Proposal 7	In case the relay UE’s RRC state changes to IDLE/INACTIVE, the relay UE should be allowed to transmit an updated discovery message immediately, while it may be left to upper layer implementation.

Another issue that should be discussed is whether the RRC connection indication in discovery message applies to both Model A and Model B discoveries.  We assume the working assumption above is intended for the relay UE to include “RRC CONNECTION” indication in the discovery message based on Model A discovery.  With Model B discovery, the remote UE should be allowed to include “RRC CONNECTION” indication in the Relay Discovery Solicitation message, and the first relay UE should also include the same indication in its Relay Discovery Solicitation message to its parent UE.  In Model B discovery, it’s assumed the last relay UE would perform the relay selection according to section 6.3.2.5.3 of [1].  However, in order for the last relay UE to choose the path that is already RRC CONNECTED, it would also need to know whether its child UE is already RRC CONNECTED. In particular, the last relay UE only needs to know whether the first relay UE is RRC CONNECTED since all upstream relay UEs are assumed to be also RRC CONNECTED once the first relay UE is RRC CONNECTED.

- Proposal 8	RRC connection indication in the discovery message should be applicable to both Model A and Model B discoveries.

# Conclusion

In this contribution, we addressed the issues that may arise with the intermediate relay UE’s relay reselection and discovery procedure.  RAN2 is kindly asked to take into account the observation and proposals below:

- Proposal 1	In case the first relay UE in IDLE/INACTIVE receives RRC Setup Request from its child UE, it should be further discussed whether the first relay UE needs to send AS-layer notification to its child UE upon AS failure detection/notification in the upstream path.

- Proposal 2	The first relay UE should not need to notify the remote UE  if the first relay UE completes the RRC re-establishment procedure towards the same cell with the same number of hops (or fewer).

- Proposal 3	It may be further discussed whether it is necessary for the first relay UE to notify the remote UE for the purpose of suspending UL data transmission prior to the intermediate relay UE’s completion of RRC re-establishment.

- Proposal 4	If the PC5 connection between first relay UE and the remote UE is not released when the first relay UE receives notification of AS failure, the first relay UE would perform relay reselection followed by RRC reestablishment.

- Proposal 5	Upon AS layer failure, the RRC CONN firstrelay UE may include the cause value “path recovery” in the AS layer notification send to the remote UE.

- Proposal 6	Confirm the working assumption that Relay UE includes an indication of whether it is RRC_CONNECTED in the discovery message RRC container.

- Observation 1	Once a remote UE receives a discovery broadcast from a candidate relay UE with “RRC CONNECTED” indication it may be assumed all relay UEs on the path towards the gNB can be considered to be “RRC CONN” since Approach 1 is assumed.

- Proposal 7	In case the relay UE’s RRC state changes to IDLE/INACTIVE, the relay UE should be allowed to transmit an updated discovery message immediately, while it may be left to upper layer implementation.

- Proposal 8	RRC connection indication in the discovery message should be applicable to both Model A and Model B discoveries.

# References

- TS 23.304, “Proximity based Services (ProSe) in the 5G System (5GS)”.