3GPP TSG RAN WG2 Meeting #131                          	   R2-2505846

Bengaluru, India, August 25th - 29th, 2025

Agenda item:	8.5.3

Source:	Qualcomm Incorporated

Title:	Discussion on remaining issues of on-demand SIB1

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives of on-demand SSB SCell are outlined with the following [1].

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss the remaining design issues of OD-SIB1.

# Discussion

Supplementary Uplink (SUL) was introduced to improve the UL coverage issue at cell edge with high NR frequency deployment, as described below in TS 38.300 [2].

Observation 1. Originally, SUL was introduced for enhancing UL coverage with a low frequence carrier.

RAN1 had discussed NES cell with SUL, but could not make an agreement on supporting such scenario.

Observation 2. RAN1 could not conclude an agreement to support NES cell with SUL.

At RAN2 #126, the following scenario was agreed for OD-SIB1 NES cells [3].

Agreements on on-demand SIB1:

Further clarification

- Study on-demand SIB1 provisioning for NES Cell(s) in versions of Scenario 1a with multiple Cells A and/or NES Cells:

- More than one Cell A may provide configuration for the same NES cell.

- The same Cell A may assist more than one NES Cells.

- RRC release message assisted intra-cell WUS can be discussed as option of signaling details in stage 3.

Figure 1. Scenario 1a for OD-SIB1

As shown in Figure 1, a UE may be at both Cell A and NES cell coverage (e.g., receiving UL WUS configuration from Cell A and transmitting UL WUS to NES cell), where more than one Cell A may broadcast the UL WUS configuration for a NES cell and more than one NES cell’s UL WUS configurations may be broadcasted by a Cell A. With such combinations of Cell A’s and NES cells, it’s not clear for the case that a NES cell containing a co-located SUL with lower frequency.

Observation 3. It is not clear about the SUL deployment with NES cell (e.g., whether a SUL is deployed with a low frequency and co-located with the NES cell).

Based on the observations, we don’t see the justifications or benefits for supporting OD-SIB1 for SUL. Therefore, we propose not to support this case, especially at the last meeting for Rel 19 NES design.

Proposal: Do not support SUL for OD-SIB1 NES cell.

# Conclusion

In this contribution, we discussed the remaining design issues of OD-SIB1and concluded with the following observations and proposals.

Observation 1. Originally, SUL was introduced for enhancing UL coverage with a low frequence carrier.

Observation 2. RAN1 could not conclude an agreement to support NES cell with SUL.

Observation 3. It is not clear about the SUL deployment with NES cell (e.g., whether a SUL is deployed with a low frequency and co-located with the NES cell).

Proposal: Do not support SUL for OD-SIB1 NES cell.

# References

- RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- TS 38.300 V18.6.0 (2025-06) “NR and NG-RAN Overall Description; Stage 2 (Release 18)”

- “R2_126_R18 SL_R19 NES_MOB_Session Notes (Kyeongin Jeong)_EoM”, RAN2 #126, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.