3GPP TSG RAN WG2 Meeting #131                          	   R2-2505847

Bengaluru, India, August 25th - 29th, 2025

Agenda item:	8.5.4

Source:	Qualcomm Incorporated

Title:	Discussion on remaining issues for RACH adaptation

WID/SID:	Netw_Energy_NR_enh-Core – Release 19

Document for:	Discussion

# Introduction

The objectives of on-demand SSB SCell are outlined with the following [1].

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum.

In this contribution, we discuss the remaining design issues for RACH adaptation in time.

# Discussion

At RAN2 #130, whether there is further specification impact to support feature and feature combination and how to solve the issue if any was discussed with the follows [2].

For Coverage Enhancement (CE), a UE may determine the number of Message 1 repetitions for initial access based on comparing the RSRP measurement with RSRP thresholds; the network may determine the number of repetitions that a UE should perform for Message 1 transmission for handover. With additional ROs in time, Message 1 repetition may be conducted within a shorter time and thus improve the latency for initial access or handover.

Observation 1: It may be beneficial to support message 1 repetition with the additional ROs of PRACH adaptation in time.

Current RAN1 specs can support both Message 1 repetition and the additional ROs without any additional specification work.

Observation 2: Current PHY specs do not exclude Message 1 repetition using the additional ROs of PRACH adaptation in time.

Based on the observations, as well as the consideration of having least impact on specifications, we propose the following proposal.

Proposal: It’s up to network's configuration for Message 1 repetition using the additional ROs of RACH adaptation in time or not, no more RAN2’s specification work is needed.

# Conclusion

In this contribution, we discussed the remaining design issues remaining design issues for RACH adaptation in time and concluded with the following observations and proposal.

Observation 1: It may be beneficial to support message 1 repetition with the additional ROs of PRACH adaptation in time.

Observation 2: Current PHY specs do not exclude Message 1 repetition using the additional ROs of PRACH adaptation in time.

Proposal: It’s up to network's configuration for Message 1 repetition using the additional ROs of RACH adaptation in time or not, no more RAN2’s specification work is needed.

# References

- RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- R2-2504671 “Report from session on R18 SL, R18/19 MOB and R19 NES”, RAN2 #130, Malta, May 19th – 23rd, 2025