3GPP TSG RAN WG2 Meeting #131                          	   R2-2505850

Bengaluru, India, August 25th - 29th, 2025

Agenda item:	8.12.3

Source:	Qualcomm Incorporated

Title:	Discussion on remaining issues for UE initiated beam report

WID/SID:	NR_MIMO_Ph5-Core – Release 19

Document for:	Discussion

# Introduction

For UE-initiated/event-driven beam management, the following subject was outlined in the WID of NR MIMO Phase 5” [1].

- Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting

In this contribution, we discuss the remaining design issues for UE initiated beam report.

# Discussion

At RAN2 #130, TAT expiration with pTAG was discussed for the impact to UE initiated beam report Mode A and Mode B, as the follows [2].

During RAN2 #130 post email discussion, TAG expiration with sTAG was further discussed and concluded with the follows [3].

At RAN1 #120bis, the following agreement was made regarding the specification of the event evaluation of UE initiated beam reporting procedure [4].

Based on this agreement, either an event is triggered for beam reporting or a beam reporting is in the process is transparent to MAC layer, unless the PHY layer indicate the event or beam report status to the MAC layer.

Observation 1. UE initiated beam report event evaluation is handled in PHY, thus it is transparent to MAC whether any UE initiated beam report event has been triggered.

Observation 2: To support further optimization to type-1 configured grant for UE initiated beam report, PHY needs to indicate to MAC each time that an event has been triggered and beam reporting is in the process.

RAN1 has completed Rel-19 MIMO phase 5 and is at maintenance stage. Any further optimization is out of CR scope.

Observation 3: It's too late to ask RAN1 for any further micro-optimization, since RAN1 is in maintenance state already.

Based on the observations, and consideration of least RAN2 spec impact, we support the following proposals.

Proposal 1. Regarding whether to initiate RACH when UEI report is triggered but TAT is expired,

Option 1: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts.

Proposal 2. For mode-A UEI report, if TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed.

# Conclusion

In this contribution, we discussed the remaining design issues for UE initiated beam report and concluded with the following observations and proposals.

Observation 1. UE initiated beam report event evaluation is handled in PHY, thus it is transparent to MAC whether any UE initiated beam report event has been triggered.

Observation 2. To support further optimization to type-1 configured grant for UE initiated beam report, PHY needs to indicate to MAC each time that an event has been triggered and beam reporting is in the process.

Observation 3. It's too late to ask RAN1 for any further micro-optimization, since RAN1 is in maintenance state already.

Proposal 1. Regarding whether to initiate RACH when UEI report is triggered but TAT is expired,

Option 1: UE does not initiate RACH when UEI report is triggered but there is no PUCCH or type-1 CG due to the associated TAT is expired. No MAC impacts.

Proposal 2. For mode-A UEI report, if TAT is expired after transmitting PUCCH and before PUSCH for a triggered report, no enhancement is needed..

# References

- RP-242354 “Revised WID: Enhancements of network energy savings for NR”, RAN Meeting #105, Melbourne, Australia, Sep 9-12, 2024.

- R2-2504672 “Report from session on Rel-18 MIMO, Rel-19 MIMO, LPWUS, SBFD, NR Others”, RAN2 #130, Malta, May 19th – 23rd, 2025

- “R2-250xxxx MAC open issues for MIMO_v13_Rapp”, [Post130][219][MIMO_Ph5] Running CR for 38.321

- “Chair notes RAN1#120bis eom0”, RAN1 #120bis, Wuhan, China, April 7th – 11th, 2025