3GPP TSG-RAN WG2 #131	R2-2505862

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	8.12.2

Source:	Ericsson

Title:	Asymmetric DL/UL mTRP impact from MIMO ph. 5

Release:	19

Document for:	Discussion

# 1	Introduction

Release 19 MIMO with its justification and objective has been endorsed as one of the features for NR MIMO Phase 5 [1].

The subtopic objectives relevant to this WI are:

- UE initiated/event-triggered beam reporting,

- increased number of ports (up to 128) for CSI measurement and reporting,

- inter-TRP calibration for CJT,

- 3Tx for UL,

- asymmetric DL sTRP/UL mTRP.

This contribution discusses some left over impacts of Asymmetric DL sTRP/UL mTRP, considering some RAN1 and RAN2 related agreements.

# 2	Discussion

2.1 Background Asymmetric DL sTRP UL mTRP

Figure 1 illustrates an asymmetric DL sTRP UL mTRP deployment. It consists of an Anchor TRP (providing both DL and UL service), and a TRP which will be called an UL-only TRP (for ease of reference) to provide UL service, where the downlink reference signals, the downlink control channels, and the downlink data transmissions for a UE are only provided from the Anchor TRP.

Figure 1: UL-only TRP in an asymmetric DL sTRP UL mTRP deployment

The main motivation for asymmetric DL sTRP UL mTRP deployment is to improve UL throughput by UE transmitting UL data to the UL-only TRP. For example, the Anchor TRP in Figure 1 provides coverage with both DL and UL transmissions, and the UL-only TRP is deployed to improve UL performance. Other motivations would be to reduce network energy consumption, by avoid sending downlink data from the UL-only TRP. It is envisioned that a UE in the cell may first perform initial access via the Anchor TRP, and then, if the UE is closer to the UL-only TRP than to the Anchor TRP (i.e. better UL link budget), or if the NW decides to move the UE to an UL-only TRP, e.g., to reduce the UL interference for the Anchor TRP, or to improve resource usage efficiency, the NW may direct the UE to transmit towards the UL-only TRP.

Enhancement on power control is needed to support the asymmetric deployment.

UE performs UL transmission to only the UL-only TRP

UE performs UL transmission to two different UL-only TRPs

UE performs UL transmission to one UL-only TRP and to the Anchor TRP

## 2.2	Control Plane Impacts

During [Post130][218][MIMO_Ph5]  e-mail discussion, the following was left as an open issue:

| Asymmetric DL sTRP/UL mTRP | In the last meeting, we have agreed to introduce a new parameter to enable Rel-19 2TA configuraiton, currently named singleDCI-MultiTRP-2TA. According to RAN1 UE feature, Rel-19 2TA is introduced for both intra-cell 2TA with no mDCI mTRP and inter-cell 2TA with no mDCI mTRP. The new parameter should be applied to both and LS to RAN1 may be needed to inform this. | This could be discussed based on company contributions. |
| --- | --- | --- |

In RAN2#130, a new parameter was agreed and implemented in 38.331 as follows:

singleDCI-MultiTRP-2TA

Enables two TA for single DCI multi-TRP. This field is used in clause x.x in TS 38.214 [19].

While the parameter was mostly agreed for backward compatibility with the configuration of tag2 field, it enables single DCI multi-TRP 2TA and thus RAN1 may also need to refer to this field. Hence, it could be helpful to inform RAN1 explicitly about the agreed field.

Inform RAN1 about introduction of singleDCI-MultiTRP-2TA field.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Proposal 1	Inform RAN1 about introduction of singleDCI-MultiTRP-2TA field.

# 4	References

- RP-242394 NR MIMO Phase 5, Samsung (Moderator), RAN meeting #102 Scotland, December 11-15, 2023