3GPP TSG-RAN WG2 #131	R2-2505867

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.6.4

Source: 	Ericsson

Title:  	L2 Reset handling for Conditional LTM

Document for:	Discussion and Decision

# Introduction

This contribution highlights an issue with the L2 reset procedure that is unique for the case of conditional LTM. It also discusses potential ways forward to solve it.

# Discussion

During a non-conditional LTM cell switch the target DU is aware of which is the source DU the UE is coming from through the Cell Switch Notification message, and therefore the target DU can compare its own ltm-noResetID with the one of the source DU and thus determine if a L2 reset has been performed at the UE side.

For the case of conditional LTM, however, there is no Cell Switch Notification message from the source DU to the target DU since the LTM cell switch procedure is initiated by the UE. In this case, there are no means for the target DU to understand from which source DU the UE is coming from and thus is there is a need for a L2 reset or not.

In CLTM, since there is no Cell Switch Notification message from the source DU to the target DU, the target DU cannot understand from which source DU the UE is coming from and thus is there is a need for a L2 reset or not.

To handle this issue, we see three possible options:

- Good Bye message. The UE tells the source DU to which LTM candidate cell it intends to switch before it executes the the LTM cell switch procedure, thus the source DU can inform the target DU.

- Hello message. The UE tells the target DU (of the LTM candidate cell to which the LTM cell switch is executed) in the first UL transmission what the ltm-noResetID of the source DU or whether a L2 reset has been done or not.

- CU informs the target DU. The CU knows which is the current source DU of the UE and, at every LTM cell switch it can keep all the target DUs which have provided an LTM candidate configuration updated about which is the current source DU of the UE (or to which the UE has switched to).

Option 1) above has already been discussed in RAN2 and there was no support to introduce it, mainly because the robustness aspects will be affected if the message doesn’t get through to the source. For this reason, we don’t think it is a good option.

Option 2) would rely on either a new MAC CE with highest priority in the LCP procedure or alternatively an indication in the UCI to let the target DU know if the L2 reset has been performed or not. However, this solution has an high some specification impact as it may require works also from other WGs.

Option 3) seems like a simple option which require almost no standardization effort (expect a small signaling addition by RAN3) as the CU is already aware of the information about which is the source DU of the UE and to which target DU the UE has switched to. Therefore, we propose:

For CLTM, the CU shall update the target DU(s) at every LTM cell switch that the serving cell of the UE has changed and which is the current serving cell of the UE. RAN2 to send an LS to RAN3.

# Conclusion

In the previous sections we made the following observations:

Observation 1	In CLTM, since there is no Cell Switch Notification message from the source DU to the target DU, the target DU cannot understand from which source DU the UE is coming from and thus is there is a need for a L2 reset or not.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For CLTM, the CU shall update the target DU(s) at every LTM cell switch that the serving cell of the UE has changed and which is the current serving cell of the UE. RAN2 to send an LS to RAN3.