3GPP TSG-RAN WG2 #131	R2-2505869

Bengaluru, India, August 25th – 29th, 2025

Agenda item:	8.6.2

Source: 	Ericsson

Title:  	How to include the NCC value in the Enhanced Cell Switch Command MAC CE

Document for:	Discussion and Decision

# Introduction

This contribution highlights a remaining issue in MAC that could enhance the structure of the MAC CE for LTM cell switch command.

# Discussion

## NCC value in MAC CE for event-based L1 reporting

In RAN2#126 the following agreement was taken:

R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM.

Later on, in RAN2#130 some further agreement outlined the base for the running MAC CR:

The Enhanced LTM CSC MAC CE always contains an NCC value field. The Enhanced LTM CSC MAC CE is used when the R19 set IDs of the target cell and the source cell are different. Otherwise, the legacy LTM CSC MAC CE is used.

This means that the UE can distinguish between if there is an NCC value sent by the eLCID of the MAC CE. Apart from the NCC, there is actually no need to change anything for the new Enhanced MAC CE.

The only difference in the MAC CE structure for the Enhanced R19 LTM CSC MAC CE is the actual NCC.

Now consider the existing R18 MAC CE in Figure 1:

Figure 1; Existing LTM Cell Switch Command MAC CE from Rel-18

In the current running CR for 38.321 the Enhanced LTM Cell Switch Command MAC CE has been proposed, as seen in Figure 2.

Figure 2; Current proposal of the Enhanced LTM Cell Switch Command MAC CE for Rel-19

It can be seen that Oct 1-4 are kept the same, while Oct 7 from the Rel-18 CSC has been put as Oct 5 in the Rel-19 CSC, thus pushing the Oct 5 and 6 from the Rel-18 CSC to be Oct 6 and 7 in the Rel-19 CSC.

The proposed Enhanced CSC MAC CE has restructured the octets based on the R18 CSC MAC CE.

The reason for moving the octet 7 to an earlier place in the new MAC CE is that the NCC value needs to be signaled, and for the case where no contention free resources should be sent a number of parameters don’t need to be communicated. This is described by the C-bit in Oct 1 in the running CR.

Though the reasoning for reshuffling the octets to be able to not send the last two octets in case contention free resources are not available is good, it has the implication that we would have two different structures of a cell switch MAC CE; one for Rel-18 and one for Rel-19. This will complicate the structure of the MAC CE and create unnecessary dependencies across the releases.

Some solutions to address this would be:

Option A) Lift out the NCC fields of 3 bits to a separate octet, and move the other content of octet 5, 6 and 7 ahead one octet, thus making them oct 6, 7 and 8. This would create a MAC CE which is one octet larger, but the implementation aspect is nicer since the only difference between Rel-18 and Rel-19 MAC CE would be that there is one extra octet for the NCC, all other fields would be the same.

Option B) Since RAN2#130 agreed to only use the enhanced LTM CSC MAC CE if there is an NCC value to send then it is possible to let the Rel-19 structure for the MAC CE be a copy of the Rel-18 one, with the exception that the spare bits in Oct 3 and 4 is used for the NCC bits. This would simplify the implementation aspects a lot since the MAC CEs would be the same, and thew eLCID of the MAC CE will indicate if NCC is signaled (Enhanced CSC MAC CE) or not (Regular CSC MAC CE).

| C | Target Conf ID |  |  | Timing Advance Command |  |  | Oct 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Timing Advance Command |  |  |  |  |  |  | Oct 2 |
| N1 | TCI state ID |  |  |  |  |  | Oct 3 |
| N2 |  | UL TCI state ID |  |  |  |  | Oct 4 |
| Random Access Preamble index |  |  |  |  |  | SS/PBCH index | Oct 5 |
| SS/PBCH index |  |  |  | PRACH Mask index |  |  | Oct 6 |
| R | R | R | R | R | S/U | Repetition Number | Oct 7 |

Figure 3; Proposal for new Enhanced LTM Cell Switch Command MAC CE

In Figure 3 above, N1 is used for the most significant bit of NCC, and N2 is used for the two least significant bits.

It is possible to reuse the structure of the existing Rel-18 LTM CSC MAC CE and still signaling the NCC.

Use the Rel-18 LTM CSC MAC structure for the new Rel-19 Enhanced LTM CSC MAC CE and add NCC in the remaining spare bits.

# Conclusion

In the previous sections we made the following observations:

Observation 1	The only difference in the MAC CE structure for the Enhanced R19 LTM CSC MAC CE is the actual NCC.

Observation 2	The proposed Enhanced CSC MAC CE has restructured the octets based on the R18 CSC MAC CE.

Observation 3	It is possible to reuse the structure of the existing Rel-18 LTM CSC MAC CE and still signaling the NCC.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Use the Rel-18 LTM CSC MAC structure for the new Rel-19 Enhanced LTM CSC MAC CE and add NCC in the remaining spare bits.

# References

R2-25xxxxx- Running MAC CR for L1 measurements, vivo, RAN2#131, August 2025.