3GPP TSG-RAN WG2 Meeting #131	R2-2505870

Bengaluru, India, August 25 – August 29 2025

Agenda item:	8.6.2

Source:	Nokia

Title:	On remaining issues for inter-CU LTM

WID/SID:	NR_Mob_Ph4 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In this contribution we discuss the remaining open issues related to Inter-CU LTM proposes resolutions for these issues.

# 2	Discussion

In this document we discuss the following remaining open issues.

- Detection of LTM switch failure due to security key issues.

- Possibility of Fast LTM Recovery after RLF for the first LTM switching.

- Clarifications related to reference configuration usage for Inter-SN LTM.

- Interworking of SCG LTM and CHO for MCG.

## 2.1	Inter CU LTM Recovery and Re-establishment

Issue 1: Detection of security failure of Inter-CU LTM

For Inter-CU LTM switching, the cell switch command includes the NCC value as clear text. SA3 has indicated that in case if the NCC is included as clear text and if the NCC value is modified by fake BS it will result in re-establishment as the LTM switching will fail due to security key mismatch (i.e, integrity protection mismtach for handover complete) in this scenario. This UE behavior is also confirmed from SA3 in their LS.

Observation 1: When Inter-CU cell switch fails due to tampering of NCC value the cell switch fails and UE triggers RRC-Reestablishment.

The gNB will not be aware of the cause of failure and re-establishment in this scenario. Based on the information received from UE after RRC-Reestablishment gNB may classify this failure as problem related to mobility robustness for LTM and SON algorithms attempts to correct mobility parameters. However, the actual reason for the failure is security issues that is not known to gNB.

Observation 2: The RRC-Re-establishment procedure after LTM switch failure does not have any information for gNB to deduce that the failure is due to security issues of LTM cell switch command.

In Rel-18, when the UE based SK-counter selection was introduced for subsequent CPAC for the network to know the possible mismatch in the SK-counter usage and UE used SK-counter value, RRC-Reconfiguration-complete message includes the ‘used-SK-counter’ value. This information helps the network to know the know the mismatch and take recovery actions such as reconfiguring the SK-counter values for further SCPAC.

Observation 3: Inclusion of information related to used security keys in RRC Reconfiguration complete is supported in SCPAC for the network to know the possible security key mismatch UE.

In the similar lines of SCPAC, if the UE includes the used NCC counter value when the re-establishment is triggered due to LTM switch failure, the network which knows the NCC value used in CSC can easily identify the mismatch and may opt for triggering security key change via RRC signalling or disable Inter-CU LTM and configure regular L3 handover for Inter-CU scenario.

Proposal 1 : RRC-Re-establishment complete message triggered after Inter-CU LTM cell switch includes used NCC counter value. The text proposal is attached in Annexure A.

Issue 2: Fast Recovery after RLF prior to first LTM switch

For the fast link recovery for RLF, RAN2-129bis made an agreement that fast recovery is possible if the source and target cell have the same Rel-19 ID. This also means that in case of RLF, recovery is possible only for Intra-CU candidate cells. This restriction is introduced where it is not possible for UE have NCC value for other CU prior to LTM Switch. However, if the RLF occurs prior to the first LTM switch all the candidate nodes are prepared with K-GNB value based on current NCC value.

Observation 4: Prior to first LTM switch all the candidate nodes are prepared with Target security key configurations based on the current NCC value.

In this case, even though the cell switch command carries the NCC value it will be the same value as the one stored at UE. Hence if the UE experiences RLF prior to cell switch in this case, it can make use of the current NCC value to generate target security configuration and recovery can be successful.

If RLF is triggered for this case the UE can make use of its current NCC value for generating keys for target cell even if Rel-19 ID is different from source cell.

Observation 5: In case of RLF prior to first LTM switch the fast recovery is possible at UE using its current NCC value.

Based on the above observations, we propose that the Inter-CU Fast recovery is allowed for RLF for the first LTM cell-switch after the preparation.

Proposal 2: RAN2 to consider Inter CU Fast recovery for RLF prior to first LTM switching.

### 2.2 Intra-CU LTM after Inter-CU LTM Switching

In RAN2-130 RAN2 has agreed to include the NCC value used for security key generation for Inter CU LTM in the cell switch command. And for intra CU cell switch network shall send Rel 18 cell switch command (does not include NCC in cell switch command).

During LTM inter CU LTM preparation, network generates Key-NG-RAN separately for all candidates that belongs to same target CU. It means all prepared candidates belonging to same CU shall have generated and stored different AS keys in the candidate context. Upon inter CU cell switch to one of the candidates of the target CU, UE generates the corresponding AS keys on receiving NCC from network in cell switch command. However, if NW decides to perform intra CU cell switch, NW shall send Rel 18 cell switch command, resulting in UE reusing the security keys of source current serving cell. But the target candidate during preparation has received different Key-NG-RAN and hence has different AS keys. This will result in key mismatches at UE and NW for further intra CU LTM cell switch if carried out with the earlier prepared candidate configurations. Additionally, during inter CU LTM preparation, PDCP-reestablishment may have configured for all the target candidates of the target CU. And UE may not perform the PDCP reestablishment due to same CellNoResetID during intra CU LTM cell switch resulting config mismatch.

Observation 6: Intra CU cell switch after inter CU cell switch will result in mismatch in UE and network configuration for security and PDCP/RLC re-establishment related configuration. Target CU need to modify the Intra-CU candidate node configurations accordingly.

For the NW to reprepare the configurations for modified user plane action for no-security key-change /no PDCP re-establishment, UE need to delay or wait for an indication from NW to start the L1 measurement reports to DU. As an alternate option NW may delay the intra CU cell switch until an indication from CU.

Proposal 3: RAN2 to discuss the options to delay the subsequent Intra-CU LTM switch after Inter-CU LTM switch until Target CU modifies its candidate configurations for Intra-CU processing.

## 2.3	Remaining issues DC-LTM

As per the latest version of RRC running CR, in the case of NR-DC, UE may receive two independent ltm configurations: ltm-Config, or an ltm-Config associated with the MCG and an ltm-ConfigNRDC.

i.e. an ltm-Config associated with MCG and an ltm-Config associated with an SCG (or)

an ltm-Config associated with MCG and ltm-ConfigNRDC associated with NR-DC (only when inter-CU SCG LTM is configured).

Issue 1: In the ‘RRCReconfiguration-IEs field description’, the ltm-ConfigNRDC field is described to contain LTM candidate configurations associated with SCG and optionally to the MCG. What is meant by “and optionally, to the MCG” needs to be clarified and the description must be accordingly updated.

Issue 2: When LTM-ConfigNRDC is configured, it is unclear to us how the reference configuration for the other cell group (i.e. MCG) is constructed and signaled to the UE and how the UE is expected to construct the reference configuration for MCG during inter-CU SCG LTM execution.

It is unclear to us, the motivation to have the following highlighted part “and, optionally, to the MCG”. Similar comment was raised in the past by other company, and the rapporteur for the running CR replied that ‘the ltm-ConfigNRDC has candidates for the SCG but also some information which can be associated to the MCG, such as reference configuration (which has MCG part)’, though we agree that candidate configuration included in ltm-ConfigNRDC is expected to have associated MCG configuration part for each inter-CU SCG LTM candidate; it is not clear to us how the reference configuration for both the cell groups is included when LTM-ConfigNRDC is configured.

RAN2 made the following agreements:

[RAN2#128]: It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM. [RAN2#129] The reference configuration related to inter-SN LTM is included within the new ltm-ConfigSCG field. [RAN2#129] UE does not combine reference configurations from ltm-Config and ltm-ConfigSCG. [RAN2#129] FFS whether only one indication is needed to indicate LTM configuration as complete configuration. [RAN2#129bis] How to indicate whether an LTM candidate configuration is a complete configuration is the same as Rel-18 (no different indication for MCG and SCG part of the candidate configuration).

As per the above agreements, when LTM-ConfigNRDC is configured, we think the reference configuration for both the CGs (MCG and SCG) must be included in ltm-ConfigSCG for the UE to be able to construct the complete configuration that should include the MCG and SCG part for each inter-CU SCG LTM candidate configuration. However, as per the latest running CR, we don’t see any change to the description of ReferenceConfiguration IE, which states: “The IE ReferenceConfiguration is used provide a configuration that is common, within the same cell group, to all configured non-complete candidate configurations.”, if the reference configuration is common within the same cell group, then it is unclear to us how the reference configuration for the other cell group (i.e. MCG) is constructed and signaled to the UE and how the UE is expected to construct the reference configuration for MCG during inter-CU SCG LTM execution. Please note that, for each inter-CU SCG LTM candidate configuration, the MCG part may vary.

Observation 7: As per the previous RAN2 agreements, the reference configuration for both the CGs (MCG and SCG) must be included in ltm-ConfigSCG for the UE to be able to construct the complete configuration that should include the MCG and SCG part for each inter-CU SCG LTM candidate configuration.

Observation 8: As per the latest RRC running CR, the reference configuration is expected to be common within the same cell group and is expected to be within ltm-ConfigNRDC when NR DC is configured.

Proposal 4: RAN2 to discuss and clarify how the reference configuration for the other cell group (i.e. MCG) is constructed and signaled to the UE and how the UE is expected to construct the reference configuration for MCG during inter-CU SCG LTM execution.

Proposal 5: RAN2 to agree to update the field description of ‘ltm-ConfigNRDC field’ based on the conclusion from the previous proposal.

# 3	Conclusion

In this paper, we analyzed the remaining open issues for Inter-CU LTM and made the following observations and proposals.

Inter-CU LTM Fast Recovery And Re-establishment

Observation 1: When Inter-CU cell switch fails due to tampering of NCC value the cell switch fails and UE triggers RRC-Reestablishment.

Observation 2: The RRC-Re-establishment procedure after LTM switch failure does not have any information to know the failure due to security issues.

Observation 3: Inclusion of information related to used security keys in RRC Reconfiguration complete is supported in SCPAC for the network to know the possible key mismatch.

Proposal 1 : RRC-Re-establishment complete message triggered after Inter-CU LTM cell switch includes used NCC counter value. The text proposal is attached in Annexure A.

Observation 4: Prior to first LTM switch all the candidate nodes are prepared with Target security key configurations based on the current NCC value.

Observation 5: In case of RLF prior to first LTM switch the fast recovery is possible at UE using its current NCC value.

Proposal 2: RAN2 to consider Inter CU Fast recovery for RLF prior to first LTM switching.

Intra-CU LTM after Inter-CU LTM Switching

Observation 6: Intra CU cell switch after inter CU cell switch will result in mismatch in UE and network configuration for security and PDCP/RLC re-establishment related configuration

Proposal 3: RAN2 to discuss the options to delay the subsequent Intra-CU LTM switch after Inter-CU LTM switch until Target CU modifies its candidate configurations for Intra-CU processing.

Remaining Issues for DC-LTM

Observation 7: As per the previous RAN2 agreements, the reference configuration for both the CGs (MCG and SCG) must be included in ltm-ConfigSCG for the UE to be able to construct the complete configuration that should include the MCG and SCG part for each inter-CU SCG LTM candidate configuration.

Observation 8: As per the latest RRC running CR, the reference configuration is expected to be common within the same cell group and is expected to be within ltm-ConfigNRDC when NR DC is configured.

Proposal 4: RAN2 to discuss and clarify how the reference configuration for the other cell group (i.e. MCG) is constructed and signaled to the UE and how the UE is expected to construct the reference configuration for MCG during inter-CU SCG LTM execution.

Proposal 5: RAN2 to agree to update the field description of ‘ltm-ConfigNRDC field’ based on the conclusion from the previous proposal. Text proposal provided in Annex.

# 4	Annex

Text Proposal for 38.331 (Proposal 1)

The proposed changes are highlighted in yellow.

RRCReestablishmentComplete ::= SEQUENCE {

rrc-TransactionIdentifier RRC-TransactionIdentifier,

criticalExtensions CHOICE {

rrcReestablishmentComplete RRCReestablishmentComplete-IEs,

criticalExtensionsFuture SEQUENCE {}

}

}

RRCReestablishmentComplete-IEs ::= SEQUENCE {

lateNonCriticalExtension OCTET STRING OPTIONAL,

nonCriticalExtension RRCReestablishmentComplete-v1610-IEs OPTIONAL

}

RRCReestablishmentComplete-v1610-IEs ::= SEQUENCE {

ue-MeasurementsAvailable-r16 UE-MeasurementsAvailable-r16 OPTIONAL,

nonCriticalExtension RRCReestablishmentComplete-v1800-IEs OPTIONAL

}

RRCReestablishmentComplete-v1800-IEs ::= SEQUENCE {

flightPathInfoAvailable-r18 ENUMERATED {true} OPTIONAL,

measConfigReportAppLayerAvailable-r18 ENUMERATED {true} OPTIONAL,

musim-CapRestrictionInd-r18 ENUMERATED {true} OPTIONAL,

nonCriticalExtension SEQUENCE {} OPTIONAL

}

RRCReestablishmentComplete-v1900-IEs ::= SEQUENCE {

nccLTMFailure  NextHopChainingCount

}

| nccLTMFailure | This field includes the NCC value included in the cell switch command if the RRC Re-establishment is triggered after Inter-CU LTM cell switch failure. |
| --- | --- |

Text Proposal for 38.331 (Proposal 5)

The proposed changes are show in track changes.

#### LTM-Candidate

The IE LTM-Candidate concerns a LTM candidate configuration to add or modify.

LTM-Candidate information element

-- ASN1START

-- TAG-LTM-CANDIDATE-START

LTM-Candidate-r18 ::=     SEQUENCE {

ltm-CandidateId-r18                            LTM-CandidateId-r18,

ltm-CandidatePCI-r18                           PhysCellId                                            OPTIONAL,    -- Need M

ltm-SSB-Config-r18                             LTM-SSB-Config-r18                                    OPTIONAL,    -- Need M

ltm-CandidateConfig-r18                        OCTET STRING (CONTAINING RRCReconfiguration)          OPTIONAL,    -- Need M

ltm-ConfigComplete-r18                         ENUMERATED {true}                                     OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfig-r18                     OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-EarlyUL-SyncConfigSUL-r18                  OCTET STRING (CONTAINING EarlyUL-SyncConfig-r18)      OPTIONAL,    -- Need R

ltm-TCI-Info-r18                               LTM-TCI-Info-r18                                      OPTIONAL,    -- Need M

ltm-NoResetID-r18                              INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

ltm-UE-MeasuredTA-ID-r18                       INTEGER (1..maxNrofLTM-Configs-plus1-r18)             OPTIONAL,    -- Need M

...,

[[

ltm-NoSecurityChangeID-r19                     LTM-NoSecurityChangeId-r19                            OPTIONAL,    -- Need M

ltm-ExecutionCondition-r19                     SetupRelease {LTM-ExecutionConditionList-r19}         OPTIONAL,    -- Need M

ltm-NZP-CSI-RS-ResourceToAddModList-r19        SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-Resource

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceToReleaseList-r19       SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-Resources)) OF NZP-CSI-RS-ResourceId

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToAddModList-r19     SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSet

OPTIONAL,    -- Need N

ltm-NZP-CSI-RS-ResourceSetToReleaseList-r19    SEQUENCE (SIZE (1..maxNrofNZP-CSI-RS-ResourceSets)) OF NZP-CSI-RS-ResourceSetId

OPTIONAL,     -- Need N

ltm-CSI-ReportConfigr19                        LTM-CSI-ReportConfig-r18                              OPTIONAL,     -- Need N

ltm-CSI-IM-ResourceToAddModList-r19   SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-Resource

OPTIONAL, -- Need N

ltm-CSI-IM-ResourceToReleaseList-r19  SEQUENCE (SIZE (1..maxNrofCSI-IM-Resources)) OF CSI-IM-ResourceId

OPTIONAL  -- Need N

ltm-CSI-IM-ResourceSetToAddModList-r19   SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSet

OPTIONAL, -- Need N

ltm-CSI-IM-ResourceSetToReleaseList-r19  SEQUENCE (SIZE (1..maxNrofCSI-IM-ResourceSets)) OF CSI-IM-ResourceSetId

OPTIONAL  -- Need N

]]

}

LTM-SSB-Config-r18 ::= SEQUENCE {

ssb-Frequency-r18                              ARFCN-ValueNR,

subcarrierSpacing-r18                          SubcarrierSpacing,

ssb-Periodicity-r18                            ENUMERATED {ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1} OPTIONAL,   -- Need R

ssb-PositionsInBurst-r18                       CHOICE {

shortBitmap                                    BIT STRING (SIZE (4)),

mediumBitmap                                   BIT STRING (SIZE (8)),

longBitmap                                     BIT STRING (SIZE (64))

}                                                                                                    OPTIONAL,   -- Need R

ss-PBCH-BlockPower-r18                         INTEGER (-60..50)                                     OPTIONAL,   -- Need R

...

}

LTM-NoSecurityChangeId-r19 ::= INTEGER (1..maxNrofLTM-Configs-plus1-r18)

-- TAG-LTM-CANDIDATE-STOP

-- ASN1STOP

LTM-Candidate field descriptions

cltmExecutionConditions This field can only be included within an LTM-Config IE associated with the MCG.

ltm-CandidateConfig This field includes an RRCReconfiguration message used to configure an LTM candidate configuration. If this filed is included in ltm-Config within ltm-Config-NRDC, the RRC configuration will include configuration for both the cell groups.

ltm-CandidatePCI This field identifies the PCI of the SpCell of the LTM candidate configuration contained in ltm-CandidateConfig.

ltm-EarlyUL-SyncConfig, ltm-EarlyUL-SyncConfigSUL A configuration used to perform the early UL synchronization procedure over an UL or SUL carrier.

ltm-NoResetID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID.

ltm-NoSecurityChangeID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellNoSecurityChangeID within VarLTM-ServingCellNoSecurityChangeID.

ltm-UE-MeasuredTA-ID If the network configures this field for one LTM candidate configuration, the network configures also for all LTM candidate configurations within ltm-CandidateToAddModList in LTM-Config and ensures that the UE has stored a value for ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID. This field is absent if tag2 is present for this LTM candidate configuration.

LTM-Config information element

-- ASN1START

-- TAG-LTM-CONFIG-START

LTM-Config-r18 ::=   SEQUENCE {

ltm-ReferenceConfiguration-r18        SetupRelease {ReferenceConfiguration-r18}                             OPTIONAL,   -- Cond NR-DC

ltm-CandidateToReleaseList-r18        SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-CandidateId-r18    OPTIONAL,   -- Need N

ltm-CandidateToAddModList-r18         SEQUENCE (SIZE (1..maxNrofLTM-Configs-r18)) OF LTM-Candidate-r18      OPTIONAL,   -- Need N

ltm-ServingCellNoResetID-r18          INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToAddModList-r18  SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfig-r18

OPTIONAL,   -- Need N

ltm-CSI-ResourceConfigToReleaseList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-ResourceConfigurations-r18)) OF LTM-CSI-ResourceConfigId-r18

OPTIONAL,   -- Need N

attemptLTM-Switch-r18                 ENUMERATED {true}                                                    OPTIONAL, -- Cond LTM-MCG

ltm-ServingCellUE-MeasuredTA-ID-r18   INTEGER (1..maxNrofLTM-Configs-plus1-r18)                             OPTIONAL,   -- Need N

...,

[[

ltm-ServingCellNoSecurityChangeID-r19       LTM-NoSecurityChangeId-r19                                          OPTIONAL,   -- Need N

ltm-ServingCellExecutionCondition-r19       SetupRelease {LTM-ExecutionConditionList-r19}                       OPTIONAL    -- Need M

]]

}

-- TAG-LTM-CONFIG-STOP

-- ASN1STOP

LTM-Config field descriptions

ltm-ServingCellExecutionCondition This field can can only be included in an ltm-Config associated with the MCG.

| Conditional Presence |  | Explanation |  |
| --- | --- | --- | --- |
| LTM-MCG |  | This field is optional present for the MCG, Need R, if the UE is configured with at least an LTM candidate configuration associated to the MCG. Otherwise, the field absent. |  |
| NR-DC |  | . If this field is included within ltm-ConfigNRDC, the RRC configuration includes configuration for MCG and SCG. |  |

- Reference Configuration

The IE ReferenceConfiguration is used to provide a configuration that is common, within the same cell group, to all configured non-complete candidate configurations.

Note: When NR DC is configured, the IE ReferenceConfiguration is used to provide a configuration that includes reference configuration for both the cell groups.

ReferenceConfiguration information element

-- ASN1START

-- TAG-REFERENCECONFIGURATION-START

ReferenceConfiguration-r18 ::= OCTET STRING (CONTAINING RRCReconfiguration)

-- TAG-REFERENCECONFIGURATION-STOP

-- ASN1STOP