3GPP TSG-RAN WG2 Meeting #131	R2-2505874

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.3.2.1

Source:	InterDigital Inc.

Title:	Functionality management for UE sided model for RRM measurement prediction

Document for:	Discussion

# 1. Introduction

In RAN2 #130, various agreements were made regarding various LCM aspects for the AI/ML mobility case, such as inference configuration and reporting, data collection for NW-side model and performance monitoring of UE-side model [1].

In this contribution, we discuss some of the open issues regarding various LCM and functionality management aspects for a UE-side model for RRM measurement prediction.

# 2. Discussion

### 2.1 Inference output reporting

Regarding the inference output and reporting for the case of RRM measurement prediction for temporal domain case B and frequency domain prediction, the legacy RRM measurement reporting configuration can be reused, as the aim is to reduce the measurements to be performed by the UE. For example, the UE may report the most recent sample it has available (measured or predicted). However, for temporal domain case A, enhancements are needed to configure the UE regarding which predicted results the UE can report. For example, the UE may be configured to report the predicted samples within the whole prediction window (PW), the last predicted sample, some statistical information of the samples within the PW (e.g., the average of the samples), or a combination of these.

Observation 1: The UE may have multiple predicted samples available to report for a given cell/beam.

Based on the discussion above, we propose the following regarding the inference output reporting:

Proposal 1: For temporal domain case B and frequency domain RRM measurement predictions, the legacy RRM measurement reporting configuration can be reused.

Proposal 2: For temporal domain case A RRM measurement prediction, the following enhancements are needed to the legacy RRM measurement reporting configuration:

- To allow the reporting of multiple RRM measurement samples for a given cell/beam.

- To control the content of the reporting (detailed vs summarized information about the predicted samples).

### 2.2 Applicability determination and reporting

Regarding the applicability determination for the AIML mobility use case, the following was agreed in RAN2 #130 [1]:

- Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter if the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not be mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

In the BM case, the associated ID corresponds to the serving cell where the beams are being predicted. In the case of AIML mobility, on the other hand, the UE may be configured to perform simultaneous predictions for multiple cells. For example, a UE may perform temporal prediction for the serving cell and multiple neighbor cells. Likewise, the UE may use RRM measurements from various cells as the input of its AIML model (e.g., in the cluster-based approach) to generate RRM predictions for one or more cells (or for various beams of different cells). In such cases, the UE may need to be provided with associated IDs for multiple cells, and the UE’s model may have been trained only for a subset of these associated IDs.

Observation 2: There may be cases in RRM measurement prediction where there is a partial match between the associated IDs of the input/output cells used during the training of the AIML model and the current associated IDs of the concerned cells.

Like in the BM use case, the UE can determine the applicability of the RRM measurement prediction functionality by considering model availability, NW-side additional conditions (i.e., the provided associated IDs), UE-side additional conditions (internally known by UE) and the provided inference configuration. However, now the provided associated IDs may not all be fully compatible with the training of the UE’s model. This may in turn influence the applicability determination for the configured AIML functionality.

Proposal 3: An RRM measurement prediction configuration may be fully applicable, fully non-applicable, or partially applicable.

### 2.2.3 Performing inference

Regarding when the UE applies the inference configuration and starts performing the inference, the following was agreed in RAN2 #129bis [2]:

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference.

The topic was further discussed in RAN2 #130, but no additional agreements were reached [1].

In the case of BM, periodic CSI reporting is activated upon configuration, while semi-persistent reporting needs to be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI. However, for RRM measurements, the activation is always upon the reception of the configuration.

One approach for the case of the RRM measurement prediction is to apply legacy behavior. That is, the UE will start performing the predictions according to the received inference configuration if it has determined that it has an applicable RRM measurement prediction model as discussed above. A disadvantage of such an approach is that the UE may perform unnecessary inferences/predictions even when it is not needed (e.g., when the UE is near the serving cell center). Even in legacy RRM framework, there are optional configurations such as s-measure that prevent the UE from performing unnecessary measurements. Thus, it should be discussed whether such mechanisms are needed for controlling inferences as well.

Observation 3: Performing RRM measurement predictions all the time is highly inefficient (e.g., when the UE is under excellent serving cell coverage).

As such, we propose the following:

Proposal 4: The UE can be configured to start performing RRM measurement predictions in one of the following ways:

- Automatically, upon the reception of the inference configuration and after determining that it has an applicable model.

- Upon the fulfillment of further conditions. The details of the conditions are FFS (e.g., configuration like the s-measure for starting inference).

### 2.2.4 Performance monitoring

Regarding performance monitoring for UE-side AIML models for the AIML mobility use case, the following were agreed in RAN2 #130 [1]:

1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

For temporal/intra-frequency predictions, the UE already has the measurement configuration to measure/acquire the ground truth to be used for performance monitoring purposes. Even in the case of temporal case B where there is skipping of some samples, the UE can be configured to not skip the measurements of the predicted instances. However, for inter-frequency predictions, the UE may need to be configured with measurement gaps to acquire the ground truth measurements.

Observation 4: For temporal/intra-frequency predictions, the UE already has the measurement configurations required for acquiring the ground truth for the purpose of performance monitoring.

Observation 5: A UE may need to be configured with measurement gaps to be able to conduct performance monitoring for inter-frequency prediction cases.

If the UE is configured to be able to decide (based on performance monitoring) to revert to legacy RRM measurements (e.g., RSRP difference is above a configured threshold), it can notify the network so that it can be provided with the measurement gaps for performing the inter-frequency measurement or keep using the gaps configured for performance monitoring purposes. Alternatively, the UE be already configured with such measurement gaps, so it may only need to activate them. The latter is beneficial for cases such as those hinted by agreement #3 (in the table above), where the UE may undertake a configured (by the network) management decision (e.g., fallback) without engaging in time-consuming signaling exchange with the network.

Based on the above, we propose the following:

Proposal 5: For inter-frequency RRM measurement prediction, the UE can be configured with measurement gaps to be able to undertake performance monitoring and/or fallback to legacy inter-frequency measurements when the prediction performance falls below a required level. FFS if the gap configurations for performance monitoring and fallback are different.

# 3. Conclusion

In this contribution, we discussed various LCM and functionality management aspects for the RRM measurement prediction functionality. The following observations and proposals were made:

Observation 1: The UE may have multiple predicted samples available to report for a given cell/beam.

Observation 2: There may be cases in AIML mobility where there is a partial match between the associated IDs of the input/output cells used during the training of the AIML model and the current associated IDs of the concerned cells.

Observation 3: Performing RRM measurement predictions all the time is highly inefficient (e.g., when the UE is under excellent serving cell coverage).

Observation 4: For temporal/intra-frequency predictions, the UE already has the measurement configurations required for acquiring the ground truth for the purpose of performance monitoring.

Observation 5: A UE may need to be configured with measurement gaps to be able to conduct performance monitoring for inter-frequency prediction cases.

Proposal 1: For temporal domain case B and frequency domain RRM measurement predictions, the legacy RRM measurement reporting configuration can be reused.

Proposal 2: For temporal domain case A RRM measurement prediction, the following enhancements are needed to the legacy RRM measurement reporting configuration:

- To allow the reporting of multiple RRM measurement samples for a given cell/beam.

- To control the content of the reporting (detailed vs summarized information about the predicted samples).

Proposal 3: An RRM measurement prediction configuration may be fully applicable, fully non-applicable, or partially applicable.

Proposal 4: The UE can be configured to start performing RRM measurement predictions in one of the following ways:

- Automatically, upon the reception of the inference configuration and after determining that it has an applicable model.

- Upon the fulfillment of further conditions. The details of the conditions are FFS (e.g., configuration like the s-measure for starting inference).

Proposal 5: For inter-frequency RRM measurement prediction, the UE can be configured with measurement gaps to be able to undertake performance monitoring and/or fallback to legacy inter-frequency measurements when the prediction performance falls below a required level. FFS if the gap configurations for performance monitoring and fallback are different.

# 4. References

[1]	RAN2 #130 Chairman Notes, Malta, May 2025.

[2] 	RAN2 #129bis Chairman Notes, China, April 2025.