3GPP TSG-RAN WG2 Meeting #131	R2-2505875

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.3.2.2

Source:	InterDigital Inc.

Title:	Functionality management for UE sided model for RRM measurement event prediction

Document for:	Discussion

# 1. Introduction

In RAN2 #130, various agreements were made regarding LCM aspects for the AI/ML mobility case, such as inference configuration and reporting, data collection for NW-side model and performance monitoring of UE-side model [1].

In this contribution, we discuss some of the open issues regarding various LCM and functionality management aspects for a UE-side model for RRM measurement event prediction.

# 2. Discussion

### 2.1 Inference and reporting configuration

Regarding the inference configuration for the AIML mobility use case for a UE-side AIML model, the following were agreed in RAN2 #130 [1]:

- The following potential “inference parameters” can be considered for inference/measurement configuration, which provides the necessary information for UE to examine the applicability, it may contain - PW length (for temporal case A) - Skipping pattern (e.g. SSB config that indicates SSBs transmitted) (optional). MRRT (for temporal case B) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted beam pattern (optional). MRRS (for spatial domain prediction) not discussed in study item, but if RAN4 says otherwise can be considered in WI - Measured and predicted frequency (for inter-frequency domain prediction)

It can be easily seen that the inference parameters agreed upon are mostly focusing on the RRM measurement prediction case (in temporal domain). For RRM measurement event prediction, for both the indirect and the direct cases, the UE needs to be configured with the corresponding events (e.g., A1 to A6) and relevant event parameters (e.g., TTT, hysteresis, etc.).

When a UE can support indirect RRM measurement event prediction, it is assumed that the UE has an AIML model that can generate RRM measurement predictions within a given PW. As such, PW length may be also specified in the inference configuration for the case of indirect RRM measurement event prediction.

On the other hand, a UE capable of direct RRM measurement event prediction does not necessarily support RRM measurement prediction. As such, the length of the PW within which the UE may predict how likely an event is to occur needs to be specified. Note that the term “occurrence window” is used for that PW in the AIML mobility SI [3]. In addition to the PW, the UE needs to be configured with a probability threshold to consider when evaluating whether an event prediction is correct or not (which influences the reporting of such event).

Based on the above, we propose the following regarding the required inference configuration for the RRM measurement event prediction case:

Proposal 1: For indirect RRM measurement event prediction, the following are included in the inference configuration:

- The length of PW for the associated AIML model

- Event-related information (event type and event-related parameters)

Proposal 2: For direct RRM measurement event prediction, the following are included in the inference configuration:

- The length of PW (“occurrence window”)

- Event-related information (event type and event-related parameters)

- Probability threshold (to control predicted event reporting)

Regarding the reporting of the inference output, in the indirect RRM measurement event prediction, the UE may send RRM measurement prediction results when it reports the predicted event as in legacy event reporting. Improvements in the reporting configuration are needed, similar to the reporting for the RRM measurement prediction case, to allow the UE to report multiple predicted samples for a given cell/beam, and to specify the content of the report (e.g., detailed vs summarized information). In addition, the UE should report the time that the future event will take place.

In comparison, the reporting for the direct RRM measurement event prediction case may be limited, as the UE may not have RRM measurement predictions to report. Instead, the UE needs to report the time-related information of the occurrence window, the probability of occurrence of an event within it and the currently available RRM measurement (as in legacy).

Observation 1: Unlike indirect RRM measurement event prediction, a UE that performs direct RRM measurement event predictions may not have predicted measurements to report.

Based on the discussion above, we propose the following:

Proposal 3: For indirect RRM measurement event prediction, the event prediction report includes:

- Multiple RRM measurement samples (current measurement and predicted measurements in a detailed and/or summarized form)

- Time-related information about the predicted event (e.g., the time of occurrence)

Proposal 4: For direct RRM measurement event prediction, the event prediction report includes:

- Time-related information about the predicted event (e.g., occurrence window)

- Probability of occurrence for the event

### 2.2 Applicability determination and reporting

Regarding the applicability determination for the AIML mobility use case, the following was agreed in RAN2 #130 [1]:

- Use the beam management agreements as the baseline, including the following aspects (details can be updated after further progress is made in AI/ML PHY BM use case) • Consider Option A and Option B like scheme (if/when AI/ML PHY makes further progress) • Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability. • Upon receiving one or more full inference configuration(s) via RRCReconfiguration message, UE shall maintain all the full inference configuration(s) no matter if the full inference configuration is applicable or inapplicable until the network releases it explicitly. • Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting when the applicability changes. • Together with inapplicability reporting, UE further indicates a simple cause value of inapplicability (FFS pending AI/ML PHY progress). • No prohibit timer is introduced. - Associated ID should be optionally configurable for training and inference (i.e. it may not be mandatorily required for all training/inference configurations). FFS for WI phase further details (what absence means, which use case, whether it is per cell or multiple cells/frequency, terminology).

In the BM case, the associated ID corresponds to the serving cell where the beams are being predicted. In the case of AIML mobility, on the other hand, the UE may be configured to perform simultaneous predictions for multiple cells. For example, a UE may perform temporal prediction for the serving cell and multiple neighbor cells to be used for indirect RRM measurement event prediction. Likewise, a UE may use RRM measurements from (at least) two cells at the input of its AIML model to evaluate whether an event (e.g., A3) will take place (by directly or indirectly predicting whether that event will occur). In such cases, the UE may need to be provided with associated IDs for multiple cells, and the UE’s model may have been trained only for a subset of these associated IDs.

Observation 2: There may be cases in RRM measurement event prediction where there is a partial match between the associated IDs of the input/output cells used during the training of the AIML model and the current associated IDs of the concerned cells.

Like in the BM use case, the UE can determine the applicability of the RRM measurement event prediction functionality by considering model availability, NW-side additional conditions (i.e., the provided associated IDs), UE-side additional conditions (internally known by UE) and the provided inference configuration. However, now the provided associated IDs may not all be fully compatible with the training of the UE’s model. This may in turn influence the applicability determination for the configured AIML functionality.

Proposal 5: For both direct and indirect RRM measurement event prediction, the AIML configuration may be fully applicable, fully non-applicable, or partially applicable.

### 2.2.3 Performing inference

Regarding when the UE applies the inference configuration and starts performing the inference, the following was agreed in RAN2 #129bis [2]:

The UE applies inference configuration for the applicable RRM measurement prediction and measurement event prediction upon receiving configuration via RRC. As baseline, no dynamic signaling (e.g. L1/2) is required. FFS when the UE can perform inference.

The topic was further discussed in RAN2 #130, but no additional agreements were reached [1].

In the case of BM, periodic CSI reporting is activated upon configuration, while semi-persistent reporting needs to be activated by MAC CE/DCI and aperiodic CSI reporting can be activated by DCI. However, for RRM measurements, the activation is always upon the reception of the configuration.

One approach for the case of the RRM measurement event prediction is to apply legacy behavior. That is, the UE will start performing the predictions according to the received inference configuration if it has determined that it has an applicable RRM measurement event prediction model as discussed above. A disadvantage of such an approach is that the UE may perform unnecessary inferences/predictions even when it is not needed (e.g., when the UE is near the serving cell center). Even in legacy RRM framework, there are optional configurations such as s-measure that prevent the UE from performing unnecessary measurements. Thus, it should be discussed whether such mechanisms are needed for controlling inferences as well.

Observation 3: Performing RRM measurement event predictions all the time is highly inefficient (e.g., when the UE is under excellent serving cell coverage).

As such, we propose the following:

Proposal 6: The UE can be configured to start performing RRM measurement event predictions in one of the following ways:

- Automatically, upon the reception of the inference configuration and after determining that it has an applicable model.

- Upon the fulfillment of further conditions. The details of the conditions are FFS (e.g., configuration like the s-measure for starting inference).

### 2.2.4 Performance monitoring

Regarding performance monitoring for UE-side AIML models for the AIML mobility use case, the following were agreed in RAN2 #130 [1]:

1 For UE-sided model, NW-sided monitoring, the performance monitor procedure contains - NW sends monitoring configuration and inference configuration to UE. - UE reports measurements and inference output based on the corresponding configurations to NW. - NW performs monitoring and makes decisions, which can be fully NW-implemented. 2 For UE-sided model, UE-sided monitoring, the performance monitor procedure as baselined contains - NW sends monitoring configuration to UE - UE measures monitoring data and generates monitoring results by comparing inference output and monitoring data. - (NW-decision): UE reports the monitoring result to NW, and NW makes the management decision. - Can further consider UE sided monitoring and (UE-decision) for some use cases: UE makes the management decision based on network configuration and sends the decision to NW. FFS which use cases. - For RRM measurement prediction - RSRP differences can be used as the performance metric for monitoring.

The UE uses predicted RRM measurements in indirect RRM measurement event prediction. As such, agreement #4 in the table above offers a viable way to evaluate the accuracy levels of event prediction for the indirect case, as indirect RRM measurement event prediction is dependent upon the accuracy of individual RRM measurement predictions.

Observation 4: The accuracy of indirect RRM measurement event predictions depends on the accuracy of the predicted RRM measurements that were used in the event prediction.

On the other hand, the UE may not be able to generate RRM measurement predictions for the direct event prediction case. As such, RSRP differences may not be available to evaluate the accuracy of the event prediction. During the evaluation of the simulations performed by the companies in the AI/ML mobility SI, the F1 score has been used as a metric to evaluate the accuracy of direct (and indirect) RRM measurement event predictions [3]. However, one important issue is the amount of such event predictions that should be monitored before an F1 score (or a similar metric) is statistically reliable to be reported. For example, for a new AIML model deployment, the first few event predictions (true/false) may give a partly incomplete picture about the actual accuracy of the model, especially if the event distribution is imbalanced (proportion of events occurring to those that do not). Likewise, a small number of wrong recent event predictions may indicate a problem with the current accuracy of the AI/ML model. The situation is complicated in the case the AIML model is cell specific, as after one correct prediction and HO, the UE may switch to using another model.

Observation 5: For direct RRM measurement event prediction, the UE may not have RRM measurement predictions to use for performance evaluation.

Observation 6: It is difficult to gather enough samples of correct/wrong predictions to calculate the F1 score for direct event predictions.

An alternative option may be to define a small list of events for monitoring purposes only, with lower triggering thresholds than the actual event we want to be predicted. By performing the predictions of these events and checking if these predictions were correct or wrong, the UE can generate more statistically reliable F1 scores to determine if the event prediction is performing well.

Based on what was discussed above, we propose the following:

Proposal 7: RSRP difference can be used as a monitoring metric for evaluating the accuracy level of indirect RRM measurement event prediction.

Proposal 8: For direct RRM measurement event prediction, the UE can be configured with multiple performance monitoring events that are likely to occur before the main event being predicted (e.g., lower triggering thresholds), and the UE calculates the F1 score based on the predictions of these monitoring events.

# 3. Conclusion

In this contribution, we discussed various LCM and functionality management aspects for the RRM measurement event prediction functionality and the following observations and proposals were made:

Observation 1: Unlike indirect RRM measurement event prediction, a UE that performs direct RRM measurement event predictions may not have predicted measurements to report.

Observation 2: There may be cases in RRM measurement event prediction where there is a partial match between the associated IDs of the input/output cells used during the training of the AIML model and the current associated IDs of the concerned cells.

Observation 3: Performing RRM measurement event predictions all the time is highly inefficient (e.g., when the UE is under excellent serving cell coverage).

Observation 4: The accuracy of indirect RRM measurement event predictions depends on the accuracy of the predicted RRM measurements that were used in the event prediction.

Observation 5: For direct RRM measurement event prediction, the UE may not have RRM measurement predictions to use for performance evaluation.

Observation 6: It is difficult to gather enough samples of correct/wrong predictions to calculate the F1 score for direct event predictions.

Proposal 1: For indirect RRM measurement event prediction, the following are included in the inference configuration:

- The length of PW for the associated AIML model

- Event-related information (event type and event-related parameters)

Proposal 2: For direct RRM measurement event prediction, the following are included in the inference configuration:

- The length of PW (“occurrence window”)

- Event-related information (event type and event-related parameters)

- Probability threshold (to control predicted event reporting)

Proposal 3: For indirect RRM measurement event prediction, the event prediction report includes:

- Multiple RRM measurement samples (current measurement and predicted measurements in a detailed and/or summarized form)

- Time-related information about the predicted event (e.g., the time of occurrence)

Proposal 4: For direct RRM measurement event prediction, the event prediction report includes:

- Time-related information about the predicted event (e.g., occurrence window)

- Probability of occurrence for the event

Proposal 5: For both direct and indirect RRM measurement event prediction, the AIML configuration may be fully applicable, fully non-applicable, or partially applicable.

Proposal 6: The UE can be configured to start performing RRM measurement event predictions in one of the following ways:

- Automatically, upon the reception of the inference configuration and after determining that it has an applicable model.

- Upon the fulfillment of further conditions. The details of the conditions are FFS (e.g., configuration like the s-measure for starting inference).

Proposal 7: RSRP difference can be used as a monitoring metric for evaluating the accuracy level of indirect RRM measurement event prediction.

Proposal 8: For direct RRM measurement event prediction, the UE can be configured with multiple performance monitoring events that are likely to occur before the main event being predicted (e.g., lower triggering thresholds), and the UE calculates the F1 score based on the predictions of these monitoring events.

# 4. References

[1]	RAN2 #130 Chairman Notes, Malta, May 2025.

[2] 	RAN2 #129bis Chairman Notes, China, April 2025.

[3]	TR 38.744 v0.0.8 (R2-2503541), Study on AI/ML for mobility in NR, May 2025.