3GPP TSG-RAN WG2 Meeting #131	R2-2505876

Bangalore, India, August 25th – 29th, 2025

Agenda Item:	8.3.2.3

Source:	InterDigital Inc.

Title:	Data collection for UE sided model

Document for:	Discussion

# 1. Introduction

In RAN2-130, it was agreed to use the AI/ML PHY framework as a baseline for the UE sided data collection [1]:

Agreements

- For UE sided data collection – use AI/ML PHY request/configuration framework as baseline. FFS enhancements/or differences

The agreements in RAN2 regarding the UE sided data collection for UE sided models for BM use case are shown in the Annex. An email discussion has also been undertaken after RAN2-130 to discuss some of the open issues for the data collection for the BM use case [2].

In this contribution, we discuss the applicability of these agreements for the AIML mobility use case.

# 2. Discussion

As can be seen in the agreements in Annex 1 and the summary of the email discussion in [2], RAN2 has agreed that the UE can request the data collection configuration and may indicate the preferred configurations (E.g., among possible configurations provided by the network).

One main difference of the AIML mobility case from the BM case is that in the former the data collection configuration is concerning RRM measurement and measurement event configurations, instead of CSI-RS configurations.

Considering this, we propose the following for the case of UE side data collection for RRM measurement and measurement event predictions:

Proposal 1: For the collection of data for the training of a UE-sided model for RRM measurement and measurement event prediction:

- the UE can be provided with a list of candidate data collection configuration, each with a configuration ID.

- The details of the required data collection configuration (e.g., enhancements to the RRM measurement framework, whether the data collection configuration is a full MeasConfig or a measID linking a measObject and a reporting/logging configuration, etc.) and the signaling mechanism (e.g., signaling via otherConfig) can be discussed in the WI phase.

- the UE can indicate its preferred configuration via UAI (e.g., by indicating the configuration ID). FFS if multiple configurations can be indicated by the UE.

- the UE can include start/stop indication in the UAI, indicating to the network whether it has started/stopped performing the measurements according to the indicated measurement configuration

- the UE does not need to perform the measurements according to the chosen/candidate RRM measurement configuration

- No prohibit timer is needed for UE indicating its preferred data collection configuration

The initial triggering of the data collection for a UE sided model may be performed transparently from the RAN (E.g., from the OTT), wherein the UE is informed with the frequencies/cells/beams of interest. Based on that, if the UE doesn’t have the configuration to do the required measurements, it may request the network to be provided with a configuration that will enable it to measure a particular cell and/or frequency. For example, the data to be collected is concerning an inter-frequency cell, and the UE is not currently configured with gaps to do these measurements for RRM purposes, the UE may need to ask the network for such a configuration. For example, the UE may just request a measurement configuration for a particular frequency and the network may implicitly determine the UE requires a gap (e.g., based on UE capability). Alternatively, the UE may explicitly request that it needs a gap to perform measurements of a certain frequency. The details of the exact signaling can be discussed in the WI phase.

Based on the above, we propose the following:

Proposal 2: The UE can request the network for a measurement configuration of a particular cell or frequency.

Proposal 3: The network can configure the UE with measurement gaps for the purpose of collecting measurements for UE side model training. The details of the exact signaling (e.g., whether UE can explicitly request the gaps) can be left for the WI phase.

# 3. Conclusion

In this contribution, we discussed data collection for UE sided models for AIML mobility, and the following proposals were made:

Proposal 1: For the collection of data for the training of a UE-sided model for RRM measurement and measurement event prediction:

- the UE can be provided with a list of candidate data collection configuration, each with a configuration ID.

- The details of the required data collection configuration (e.g., enhancements to the RRM measurement framework, whether the data collection configuration is a full MeasConfig or a measID linking a measObject and a reporting/logging configuration, etc.) and the signaling mechanism (e.g., signaling via otherConfig) can be discussed in the WI phase.

- the UE can indicate its preferred configuration via UAI (e.g., by indicating the configuration ID). FFS if multiple configurations can be indicated by the UE.

- the UE can include start/stop indication in the UAI, indicating to the network whether it has started/stopped performing the measurements according to the indicated measurement configuration

- the UE does not need to perform the measurements according to the chosen/candidate RRM measurement configuration

- No prohibit timer is needed for UE indicating its preferred data collection configuration

Proposal 2: The UE can request the network for a measurement configuration of a particular cell or frequency.

Proposal 3: The network can configure the UE with measurement gaps for the purpose of collecting measurements for UE side model training. The details of the exact signaling (e.g., whether UE can explicitly request the gaps) can be left for the WI phase.

# 4. References

[1]	RAN2 #130 Chairman Notes, Malta, May 2025.

[2]	R2-2505296, Report of [POST130][037][AI PHY] UE candidate data collection (Xiaomi/Ericsson), RAN2-131, Bangalore, India, August 2025

[3]	RAN2-129 Chairman Notes, Athens, Greece, February 2025

[4]	RAN2-129bis Chairman Notes, Wuhan, China, April 2025

[5]	RAN2-130 Chairman Notes, Malta, May 2025

# 5. Annex (Agreements regarding UE side data collection for BM use case)

RAN2-129 agreements [3]:

Agreements on UE side data collection: - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - a. The network can decide when to start/stop the data collection and send configuration. - b. The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication)

RAN2-129bis agreements [4]:

Agreements on data collection configuration: - The UE can request measurement configuration for data collection of AI/ML based beam management. The request can contain one or more of the following: - An indication on start/stop of data collection - Preferred configuration from a list of candidate configurations provided by NW. Details of signaling are FFS. It is up to network what it configures at the end. - Introduce UAI message for UE request of data collection measurement configuration. And it is up to UE implementation when to send the request.

RAN2-130 agreements [5]:

Agreements on data collection configuration: - The UE doesn’t need to measure the candidate data collection configuration(s). This will be specified in RAN2 specs. - For beam management, candidate data collection configuration includes at least: - CSI-ResourceConfigId of Set A - CSI-ResourceConfigId of Set B - One/two associated IDs (up to whether Set B is equal/subset of Set A or not) according to RAN1 agreements FFS the details of how this is signalled (e.g. CSIReport config or simplified signaling)